ZK-RFID1201 UHF RFID

Reader User’s Manual v1.0

Content

1. COMMUNICATION INTERFACE SPECIFICATION	1

2. PROTOCOL DESCRIPTION	1

3. DATA BLOCK FORMAT	2

3.1 COMMAND DATA BLOCK	2

3.2 RESPONSE DATA BLOCK	2

4. OPERATION COMMAND (CMD) SUMMARY	3

4.1 EPC C1G2（ISO18000-6C）COMMAND	3

4.2 18000-6B COMMAND	4

4.3 READER DEFINED COMMAND	4

5. LIST OF COMMAND EXECUTION RESULT STATUS	6

6. TAG RETURNED ERROR CODE	8

7. TAG FEATURES REQUIRING ATTENTION	8

8. DETAILED DESCRIPTION OF OPERATION COMMANDS	9

8.1 OVERVIEW	9

8.2 EPC C1G2 COMMAND	9

8.2.1 Inventory	9

8.2.2 Read Data	12

8.2.3 Write Data	13

8.2.4 Write EPC	14

8.2.5 Kill Tag	15

8.2.6 Lock	16

8.2.7 BlockErase	18

8.2.8 Set Privacy with Mask Pattern	19

8.2.9 Set Privacy without Mask Pattern	20

8.2.10 Reset Privacy	20

8.2.11 Check Privacy	20

8.2.12 EAS Configure	21

8.2.13 EAS Alarm	22

8.2.14 Block Write	22

8.2.15 Read Monza4QT parameters	23

8.2.16 Set Monza4QT parameters	25

8.2.17 Extended Read Data	26

8.2.18 Extended Write Data	27

8.2.19 Mix Inventory	28

8.3 18000-6B COMMAND	31

8.4 READER-DEFINED COMMAND	31

8.4.1 Get Reader Information	31

8.4.2 Set Region	33

8.4.3 Set Address	34

8.4.4 Set InventoryScanTime	34

8.4.5 Set Baud Rate	34

8.4.6 Set RF Power	35

8.4.7 BuzzerAndLED Control	35

8.4.8 Set Work Mode	36

8.4.9 Get System Parameters	37

8.4.10 Set EAS Sensitivity	38

8.4.11 Set Mask	39

8.4.12 Set Response Style of Auto-running Mode	39

8.4.13 Set Inventory Interval	40

8.4.14 Select Tag Type	40

8.4.15 Set Antenna Multiplexing	41

8.4.16 Set Beep Notification	42

8.4.17 Set Real-time Clock	42

8.4.18 Get Time	43

8.4.19 Get Tag Buffer Info	43

8.4.20 Clear Tag Buffer	44

8.4.21 Set Relay	44

8.4.22 Set GPIO	45

8.4.23 Get GPIO Status	45

8.4.24 Set Trigger Delay Time	46

8.4.25 Set TID-Inventory Parameters	46

8.4.26 Get Reader Serial Number	46

8.4.27 Change AT Mode	47

8.4.28 AT Transparent Command	47

8.4.29Tag custom function	48

8.4.30 Set antenna checked	48

8.4.31 Set Write Power	49

8.4.32 Get Write Power	49

8.4.33 Get or Set Rerty times	49

8.4.35 Set detect multiple tag parameter	50

8.4.36 Get detect multiple tag parameter	51

8.4.37 R2000 module ROM update	51

# 1. COMMUNICATION INTERFACE SPECIFICATION

The reader communicates with host (MCU，MPU，Controller) using serial communication interface RS232/ RS485 or TCPIP and complete corresponding operation according to the host command. The default serial communication parameter is 57600bps 1 start bit, 8 data bits, 1 stop bit without parity check bit. In the process of communication, the least significant bit of one byte is transmitted first and the least significant byte of command data sequence is transmitted first.

# 2. PROTOCOL DESCRIPTION

A communication procedure is sponsored by the host sending commands and data to the reader and the reader returns the result status and data to host after command execution.

Reader executes a command after receiving it. Only after command execution completed, the reader would be able to receive another command. During the implementation of one command, the reader ignores all other command data received.

The following table shows the process of the host computer sending command:

| HOST | DIRECTION | READER |
| --- | --- | --- |
| Command Data Block | → |  |

The interval between two consecutive bytes in the command data block should be less than 15ms. During command data block sending, synchronization will lost if the host receives any data from the reader and the host should stop command sending and restart the communication after 15ms.

The reader completes command execution after receiving host command and returns the results. During the period, it doesn’t process any host data. The feedback of command execution results is as follows:

| READER | DIRECTION | HOST |
| --- | --- | --- |
| Response Data Block | → |  |

The interval between two consecutive bytes in the response data block is less than 15ms.

# 3. DATA BLOCK FORMAT

## 3.1 COMMAND DATA BLOCK

| Len | Adr | Cmd | Data[] | LSB-CRC16 | MSB-CRC16 |
| --- | --- | --- | --- | --- | --- |

Remark：

|  | LENGTH(Byte) | COMMENT |
| --- | --- | --- |
| Len | 1 | Command data block length 1 byte (not including itself). Value range is 4~96. The number of Len equals the length of Data [] plus 4. |
| Adr | 1 | Reader address, 1 byte. Value range is 0~254. Only will the reader conforming to the address response the command data block. Value 255 is broadcasting address. All the readers will response to the command data block with a broadcasting address. The default value is 0. |
| Cmd | 1 | Operation command symbol, 1 byte. |
| Data[] | Variable | Operation command parameters. |
| LSB-CRC16 | 1 | CRC-16 LSB. CRC-16 checksum, 2 bytes with least significant byte first. |
| MSB-CRC16 | 1 | CRC-16 MSB. |

## 3.2 RESPONSE DATA BLOCK

| Len | Adr | reCmd | Status | Data[] | LSB-CRC16 | MSB-CRC16 |
| --- | --- | --- | --- | --- | --- | --- |

COMMENT：

|  | LENGTH(Byte) | COMMENT |
| --- | --- | --- |
| Len | 1 | Response data block length 1 byte (not including itself). The number of Len equals the length of Data [] plus 5. |
| Adr | 1 | Reader address, 1 byte. Value rang is 0~254. |
| reCmd | 1 | Received command symbol, 1 byte. If the command is unrecognized, the reCmd is 0x00. |
| Status | 1 | Result status value, 1byte. Refer to following table for details. |
| Data[] | Variable | Response data. |
| LSB-CRC16 | 1 | CRC16 LSB. CRC-16 checksum, 2 bytes with least significant byte first. |
| MSB-CRC16 | 1 | CRC16 MSB. |

The default value of the reader address is 0x00. The host may change it by using reader-defined command “Write Adr”.

Cyclic Redundancy Check (CRC) computation includes all data from Len. A reference CRC computation program is presented as follow:

C-Example:

#define PRESET_VALUE 0xFFFF

#define POLYNOMIAL  0x8408

unsigned int uiCrc16Cal(unsigned char const  * pucY, unsigned char ucX)

{

unsigned char ucI,ucJ;

unsigned short int  uiCrcValue = PRESET_VALUE;

for(ucI = 0; ucI < ucX; ucI++)

{

uiCrcValue = uiCrcValue ^ *(pucY + ucI);

for(ucJ = 0; ucJ < 8; ucJ++)

{

if(uiCrcValue & 0x0001)

{

uiCrcValue = (uiCrcValue >> 1) ^ POLYNOMIAL;

}

else

{

uiCrcValue = (uiCrcValue >> 1);

}

}

}

return uiCrcValue;

}

# 4. OPERATION COMMAND (CMD) SUMMARY

## 4.1 EPC C1G2（ISO18000-6C）COMMAND

| NUM | COMMAND | CODE | COMMENT |
| --- | --- | --- | --- |
| 1 | Inventory | 0x01 | The function is used to inventory tags in the effective field and get their EPC values. |
| 2 | Read Data | 0x02 | The function is used to read part or all of a Tag’s Password, EPC, TID, or User memory. |
| 3 | Write Data | 0x03 | The function is used to write several words in a Tag’s Reserved, EPC, TID, or User memory. |
| 4 | Write EPC | 0x04 | The function is used to write EPC value in one Tag’s EPC memory. The writing process is carried out in a broadcast way. |
| 5 | Kill Tag | 0x05 | The function is used to deactivate (kill) one tag. When killed, the tag will not answer to any command. |
| 6 | Lock | 0x06 | The function is used to set the tag memories’ access control policy by setting them as readable, writable or on the verse. |
| 7 | BlockErase | 0x07 | The function is used to erase multiple words in a Tag’s Password, EPC, TID, or User memory. |
| 8 | Set Privacy with Mask Pattern | 0x08 | The function is used to set a designated tag into privacy state (Only NXP's UCODE EPC G2X tags available). |
| 9 | Set Privacy without Mask Pattern | 0x09 | The function is used to set one tag in the field into privacy state (Only NXP's UCODE EPC G2X tags available). |
| 10 | Reset Privacy | 0x0a | The function is used to remove one tag from privacy state(Only NXP's UCODE EPC G2X tags available). |
| 11 | Check Privacy | 0x0b | The function is used to check one tag if it is in the privacy state (Only NXP's UCODE EPC G2X tags available). |
| 12 | EAS Configure | 0x0c | The function is used to set or reset the EAS bit of one designated tag (Only NXP's UCODE EPC G2X tags available). |
| 13 | EAS Alarm | 0x0d | The function is used to check EAS bit status of one tag in the field (Only NXP's UCODE EPC G2X tags available). |
| 14 | Block Write | 0x10 | The function is used to write multiple words in a Tag’s Reserved, EPC, TID, or User memory. |
| 15 | Read Monza4Qt | 0x11 | The function is used to get Monza4QT work parameters. |
| 16 | Set Monza4Qt | 0x12 | The function is used to Set Monza4QT work parameters. |
| 17 | Extended Read data | 0x15 | The function is used to read large memory of tag. |
| 18 | Extended Write data | 0x16 | The function is used to write large memory of tag. |
| 19 | Mix Inventory | 0x19 | The function is used to mix inventory. |

## 4.2 18000-6B COMMAND

## 4.3 READER DEFINED COMMAND

| NUM | COMMAND | CODE | CONNECT |
| --- | --- | --- | --- |
| 1 | Get Reader Information | 0x21 | This function is used to get reader-related information such as reader address (Adr), firmware version, supported protocol type, InventoryScanTime, RF power and frequency range. |
| 2 | Set Region | 0x22 | This function is used to set the work region which defines the lower limit and the upper limit of frequency range. |
| 3 | Set Address | 0x24 | This function is used to set the reader’s address. The address value will be stored in reader’s inner nonvolatile memory with default value 0x00. The value range is 0x00~0xFE. The address 0xFF is reserved as the broadcasting address. When user tries to write a 0xFF address, the reader will set the value to 0x00 automatically. |
| 4 | Set InventoryScanTime | 0x25 | This function is used to set reader’s InventoryScanTim. The value range is 3~255 corresponding to 3*100ms~255*100ms with default 10 (10*100ms). |
| 5 | Set Baud Rate | 0x28 | The function is used to change the serial communication baud rate. |
| 6 | Set RF Power | 0x2F | The function is used to set the RF power of reader. |
| 7 | BuzzerAndLED Control | 0x33 | The function is used to control the reader’s inner buzzer and LED. |
| 8 | Set Work Mode | 0x35 | The function is used to set the reader’s work mode. |
| 9 | Get System Parameters | 0x36 | The function is used to get the reader’s work mode parameters. |
| 10 | Set EAS sensitivity | 0x37 | The function is used to set EAS operation sensitivity. |
| 11 | Set Mask | 0x3b | The function is used to set mask condition for G2 tag inventory. |
| 12 | Set Response Style of Auto-running mode | 0x3c | The function is used to set response format parameters in auto-running mode. |
| 13 | Set Inventory Interval | 0x3d | The function is used to set inventory interval time. |
| 14 | Select Tag Type | 0x3e | The function is used to select tags type to be inventoried. |
| 15 | Set Antenna Multiplexing | 0x3f | The function is used to configure antennae’ multiplexing. |
| 16 | Set Beep Notification | 0x40 | The function is used to set beep notification action. |
| 17 | Set Real-time Clock | 0x41 | The function is used to set reader’s inner real-time clock. |
| 18 | Get Time | 0x42 | The function is used to read out time of the reader’s inner real-time clock. |
| 19 | Get Tag Buffer Info | 0x43 | The function is used to read out all tag records in reader's tag buffer. |
| 20 | Clear Tag Buffer | 0x44 | The function is used to clear reader's tag buffer. |
| 21 | Set Relay | 0x45 | The function is used to control the reader’s built-in relay. |
| 22 | Set GPIO | 0x46 | The function is used to set GPIO status. |
| 23 | Get GPIO Status | 0x47 | The function is used to read GPIO status. |
| 24 | Set Notification Pulse Output | 0x48 | The function is used to configure GPIO for pulse notification of tag detection. |
| 25 | Set Trigger Delay Time | 0x49 | The function is used to set the delay time in trigger mode. |
| 26 | Set TID-Inventory Parameters | 0x4a | The function is used to set the parameters used in TID-Inventory. |
| 27 | Get seria Number | 0x4C | The function is used to get reader‘s seria number. |
| 28 | AT CMD Change | 0x4d | The function is used to change AT mode for network module. |
| 29 | Transparent CMD | 0x4e | The function is used to transparent cmd for network module |
| 30 | Set tag custom Function | 0x3a | The function is used to set tag’s defined function.. |
| 31 | Set antenna checked | 0x66 | The function is used to set antenna auto checked |
| 32 | Set write power | 0x79 | The function is used to set power when do write operation |
| 33 | Get write power | 0x7A | The function is used to get power when do write operation |
| 34 | Set/Get retry times | 0x7B | The function is used to set or get retry times when write failed. |
| 35 | Set QS | 0x81 | The function is used to set Q and session for mutiple tag query. |
| 36 | Get QS | 0x82 | The function is used to get Q and session for mutiple tag query. |
| 37 | Flash ROM | 0x84 | The function is used to flash ROM |

# 5. LIST OF COMMAND EXECUTION RESULT STATUS

| RESPONSE DATA BLOCK |  |  |  |  |  | STATUS COMMENT |
| --- | --- | --- | --- | --- | --- | --- |
| Len | Adr | reCmd | Status | Data[] | CRC16 |  |
| Length of Data[] +5 | 0xXX | 0xXX | 0x00 | …….. | LSB+MSB | Command is executed successfully and data part contains result data. |
| Length of Data[] +5 | 0xXX | 0x01 | 0x01 | …….. | LSB+MSB | G2 tag inventory operation is completely finished in defined InventoryScanTime and all tags’ EPC data are returned. |
| Length of Data[] +5 | 0xXX | 0x01 | 0x02 | …….. | LSB+MSB | G2 tag inventory operation is not completely finished when defined InventoryScanTime overflows and some tags’ EPC data are returned. |
| Length of Data[] +5 | 0xXX | 0x01 | 0x03 | …….. | LSB+MSB | The number of the inventoried tags is too much and not all tags’ EPC data can be returned within this single response data block. Other response data blocks are followed. |
| Length of Data[] +5 | 0xXX | 0x01 | 0x04 | …….. | LSB+MSB | G2 tag inventory operation is finished since the number of the inventoried tags reaches the reader process limit. All inventoried tags’ EPC data are returned. |
| 5 | 0xXX | 0xXX | 0x05 | - | LSB+MSB | The sent command is password-required operation and the password given in the command is wrong. |
| 5 | 0xXX | 0x05 | 0x09 | - | LSB+MSB | Kill command is not executed successfully. The possible reasons include password error and poor reader to tag air communication. |
| 5 | 0xXX | 0x05 | 0x0a | - | LSB+MSB | Kill Password is zero. Kill tag operation requires a none-zero password. |
| 5 | 0xXX | 0xXX | 0x0b | - | LSB+MSB | The operation not supported by the tag. |
| 5 | 0xXX | 0xXX | 0x0c | - | LSB+MSB | Password is zero. Privacy or EAS related operations require a none-zero password (Only NXP's UCODE EPC G2X tags available). |
| 5 | 0xXX | 0x0a | 0x0d | - | LSB+MSB | The tag is already in privacy. It does not need to be set into privacy state again (Only NXP's UCODE EPC G2X tags available). |
| 5 | 0xXX | 0x0a | 0x0e | - | LSB+MSB | The tag is not in privacy state and does not need to be set out of privacy state (Only NXP's UCODE EPC G2X tags available) or the tag does not support this kind of operation. |
| 5 | 0xXX | 0x53 | 0x10 | - | LSB+MSB | The data block of a 6B tag is locked and can not be rewritten. |
| 5 | 0xXX | 0x55 | 0x11 | - | LSB+MSB | The data block of a 6B tag can not be locked. |
| 5 | 0xXX | 0x55 | 0x12 | - | LSB+MSB | The data block of a 6B tag is already locked and not needed to be re-locked. |
| 5 | 0xXX | 0xXX | 0x13 | - | LSB+MSB | Reader configuration parameters saving operation failed. |
| 5 | 0xXX | 0xXX | 0x14 | - | LSB+MSB | The RF power can not be adjusted in current situation. |
| Length of Data[] +5 | 0xXX | 0x51 | 0X15 | …….. | LSB+MSB | 6B tag inventory operation is completely finished in defined InventoryScanTime and all tags data are returned. |
| Length of Data[] +5 | 0xXX | 0x51 | 0x16 | …….. | LSB+MSB | 6B tag inventory operation is not completely finished when defined InventoryScanTime overflows and some tags’ data are returned. |
| Length of Data[] +5 | 0xXX | 0x51 | 0x17 | …….. | LSB+MSB | The number of the inventoried tags is too much and not all tags’ data can be returned within this single response data block. Other response data blocks are followed. |
| Length of Data[] +5 | 0xXX | 0x51 | 0x18 | …….. | LSB+MSB | 6B tag inventory operation is finished since the number of the inventoried tags reaches the reader process limit. All inventoried tags’ data are returned. |
| 5 | 0xXX | 0xXX | 0x19 | - | LSB+MSB | EAS related operation failed. The possible reasons include the tag does not support this operation or the password is zero. |
| Length of Data[] +5 | 0xXX | 0xXX | 26 |  | LSB+MSB | Return rate packet when inventory finished |
| 5 | 0xXX | 0xXX | 0xF8 | - | LSB+MSB | Antenna checked error. |
| 5 | 0xXX | 0xXX | 0xF9 | - | LSB+MSB | Command execution error. |
| 5 | 0xXX | 0xXX | 0xFA | - | LSB+MSB | Operation aborted since the poor reader to tag air communication. |
| 5 | 0xXX | 0xXX | 0xFB | - | LSB+MSB | No tag in the field. |
| 6 | 0xXX | 0xXX | 0xFC | Err_code | LSB+MSB | Tag operation error and the error code is returned in Err_code. |
| 5 | 0xXX | 0xXX | 0xFD | - | LSB+MSB | Command length error. |
| 5 | 0xXX | 0x00 | 0xFE | - | LSB+MSB | Command can not be recognized since it is a non-exist command or CRC error. |
| 5 | 0xXX | 0xXX | 0xFF | - | LSB+MSB | Command parameters error. |

# 6. TAG RETURNED ERROR CODE

EPC C1G2（ISO18000-6C）Tag returned error code：

|  | Error-Code | Error-Code Name | Description |
| --- | --- | --- | --- |
| Specified error | 0x00 | Other error | Other errors. |
|  | 0x03 | Memory override | Memory not exist or not-supported PC value. |
|  | 0x04 | Memory locked | Memory is locked and can not be rewritten. |
|  | 0x0b | Insufficient power | No sufficient power to meet writing operation requirement. |
| Non-specified error | 0x0f | Non-specified error | Non-specified error. |

# 7. TAG FEATURES REQUIRING ATTENTION

- EPC C1G2 TAG（G2 TAG）

G2 tag memory includes four sector: reserved memory (password memory), EPC memory, TID memory and User memory.

Reserved memory (password memory) is a 4-word sector. The former 2 word is kill password and the latter 2 word is access password. All passwords are available to read and write. Reserved memory can be read/write protected by configuration..

EPC memory contains EPC information of the tag. The address 0 word is CRC checksum of the EPC memory. The address 1 word is PC value indicating the EPC word-unit length. From the address 2 word on, it is the EPC of the tag. EPC memory is available to read and write.

TID memory contains information such as UID etc. provided by the tag vendor. TID memory is available to read and not to write.

User memory is a space for tag user. User memory is available to read and write. User memory is optional.

In G2 tag operation, the word unit is used frequently. 1 word equals 2 bytes.

Some G2 tag operation need password and if the password is not set, please use all 0s instead.

- 18000-6B TAG (6B TAG)

6B tag has one integrated memory space. The lowest 8 bytes is for tag’s UID which can not be altered. Other bytes are for user usage and available to read, write and lock. If the byte locked, it should not be able to alter or unlock.

# 8. DETAILED DESCRIPTION OF OPERATION COMMANDS

## 8.1 OVERVIEW

The reader supports three kinds of commands: protocol related command, reader command and tag customized command.

If the reader gets an unrecognized command such as non-exist command or CRC error command, it will return following response data block:

| Len | Adr | reCmd | Status | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x00 | 0xFE | LSB | MSB |

If the reader gets a command with incorrect length, it will return the following response data block:

| Len | Adr | reCmd | Status | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0xXX | 0xFD | LSB | MSB |

The reader will not respond to next two kinds of commands:

1. A command with incorrect address. That means the Adr byte in the command is neither equal to the reader’s address nor to the broadcast address 0xFF;

2. An un-integrated command. That means the length indicated by the Len byte is greater than the actual received command length.

## 8.2 EPC C1G2 COMMAND

### 8.2.1 Inventory

The command is used to inventory tags in the field and get their EPC values. The user may need to use this function to get tags’ EPC before any further operation to the tags. This function has also been enhanced to get the tag’s TID directly for firmware version V1.20 and above.

This command is executed within predefined InventoryScanTime limit. The default InventoryScanTime is 1 second (10*100ms). The allowable value range is from 3*100ms to 255*100ms.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x01 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QValue | Session | MaskMem | MaskAdr | MaskLen | MaskData | AdrTID | LenTID |
| 0xXX | 0xXX | 0xXX | 2Bytes | 0xXX | Variable | 0xXX | 0xXX |

| Data[] |  |  |
| --- | --- | --- |
| Target | Ant | Scantime |
| 0xXX | 0xXX | 0xXX |

Parameter explanation:

QValue: One byte, Q value,

Bit7: Statistical data packets flag;

0: not send the flag packets when query finished.

1: send the flag packets when query finished.

Bit6-Bit0: range is 0-15. Q value setting should be field tag quantity is approximately equal to 2 Q.

other value return parameter error.

Session: One byte, Session value,

0x00:S0;

0x01:S1;

0x02:S2;

0x03:S3;

0xFF: Auto config session. (Only EPC query effective).

other value return parameter error.

MaskMem: One byte. It specifies the target memory when applying inventory mask with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address of the mask pattern data. The value ranges from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern data. The value ranges from 0 to 255.

MaskData: Mask pattern data. The byte length of the MaskData is MaskLen/8. If MaskLen is not 8bits integer times, the length of MaskData should be int[MaskLen/8]+1 with 0 patching in the low significant location.

AdrTID: One byte. It specifies the start word address in TID memory when doing the TID-inventory.

LenTID: One byte. It specifies the number of words when doing the TID-inventory. The range is 0~ 15.

Target(Optional parameters):One byte,

0x00：Target value is A;

0x01：Target value is B;

Other values are reserved.

Ant(Optional parameters):One byte,antenna number this time to query.

0x80: antenna 1;

0x81: antenna 2;

0x82: antenna 3;

0x83: antenna 4;

0x84: antenna 5;

0x85: antenna 6;

0x86: antenna 7;

0x87: antenna 8;

0x88: antenna 9;

0x89: antenna 10;

0x8A: antenna 11;

0x8B: antenna 12.

ScanTime(Optional parameters): One byte,reader will set max scan time to ScanTime*100ms.

Remark:

- The MaskMem , MaskAdr, MaskLen and MaskData can be all vacant. That means tags will be inventoried without mask pattern. The AdrTID and LenTID can be vacant for EPC inventory. If AdrTID and LenTID exist, it means the current inventory operation is a TID-inventory and the inventory procedure will get the tags’ TID directly instead of their EPC.

- The Target, Ant, ScanTime is optional parameters,these parameters must be used to together.

Response:

| Len | Adr | reCmd | Status | Data[] |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ant | Num | EPC/TID ID |  |  |
| 0xXX | 0xXX | 0x01 | 0xXX | 0xXX | 0xXX | EPC-1, EPC-2, EPC-3… | LSB | MSB |

Parameter explanation:

Status:

| Value | Comment |
| --- | --- |
| 0x01 | Tag inventory operation is completely finished in defined InventoryScanTime and all tags’ EPC data are returned. |
| 0x02 | Tag inventory operation is not completely finished when defined InventoryScanTime overflows and some tags’ EPC data are returned. |
| 0x03 | The number of the inventoried tags is too much and not all tags’ EPC data can be returned within this single response data block. Other response data blocks are followed. |
| 0x04 | Tag inventory operation is finished since the number of the inventoried tags reaches the reader process limit. All inventoried tags’ EPC data are returned. |
| 0x26 | If Qvalue’s bit8=1.when inventory finished,return this code, it’s rate packet |

Ant: It describes from which antenna the tag EPC is collected. For example, Ant = 0000 0100b means ANT3 and Ant = 0000 1000b means ANT4, etc.

Num: The number of tag detected.

EPC/TID ID: Inventoried tag’s EPC/TID data. EPC-1 is the first tag’s EPC/TID length plus EPC/TID data plus RSSI and so on. The most significant word of EPC/TID is transmitted first and the most significant byte of a word is also transmitted first. EPC/TID length is one byte.

### 8.2.2 Read Data

The command is used to read part or all of a Tag’s Password, EPC, TID, or User memory.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x02 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Mem | WordPtr | Num | Pwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 0xXX | 0xXX | 0xXX | 4Byte | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Mem: One byte. It specifies the target memory of the operation with 0x00 for Password memory, 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

WordPtr: One byte. It specifies the starting word address for the operation with 0x00 for the first word, 0x01 for the second word and so on.

Num: One byte. It specifies the number of words to read. The value range is from 1 to 120. Other value will incur a parameters error.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x02 | 0x00 | Word1，Word2,… | LSB | MSB |

Parameter explanation:

Word1, Word2….: read out data in word units with most significant byte of a word first. Word1 is the content of the start-address-defined word in target memory and so on.

### 8.2.3 Write Data

The command is used to write several words into a Tag’s Reserved, EPC, TID or User memory.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x03 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| WNum | ENum | EPC | Mem | WordPtr | Wdt |
| 0xXX | 0xXX | Variable | 0xXX | 0xXX | Variable |
| Pwd | MaskMem | MaskAdr | MaskLen | MaskData |  |
| 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |  |

Parameter explanation:

WNum: One byte. It specifies the number of words to be written into a tag. The value can not be 0 and should be equal to the word length of Wdt.

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Mem: One byte. It specifies the target memory of the operation with 0x00 for Password memory, 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

WordPtr: One byte. It specifies the starting word address for the operation with 0x00 for the first word, 0x01 for the second word and so on.

Wdt: the data to be written into a tag. The word length of Wdt should be equal to WNum and the data should be arranged as most significant word first and most significant byte in a word first.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x03 | 0x00 | - | LSB | MSB |

### 8.2.4 Write EPC

The command is used to write EPC code in a Tag’s EPC memory. When using this command to write the EPC code, please make sure that there is only one tag in the effective field since this command utilize no mask pattern for selecting one tag to operate and just take one tag in the field to operate instead.

Command:

| Len | Adr | Cmd | Data[] |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ENum | Pwd | WEPC |  |  |
| 0xXX | 0xXX | 0x04 | 0xXX | 4Bytes | Variable | LSB | MSB |

Parameter explanation:

ENum: one byte. It specifies the tag’s EPC length in word unit and the value range is from 0 to 15.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

WEPC: The EPC code to be written. The word length of EPC should be equal to Enum and the data should be arranged as most significant word first and most significant byte in a word first.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x04 | 0x00 | - | LSB | MSB |

### 8.2.5 Kill Tag

The command is used to kill tag. After killed, a tag will not answer any request.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x05 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Killpwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Killpwd: Four bytes kill password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed and all 0s kill password is not allowed..

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x05 | 0x00 | - | LSB | MSB |

### 8.2.6 Lock

The command is used to set protection strategy of Reserved, EPC, TID and User memory.

Reserved memory can be set as unprotected readable/writable, permanent readable/writable, password-protected readable/writable and permanent unreadable/unwritable;

EPC and User memory can be set as unprotected writable, permanent writable, password-protected writable and permanent writable. These two memories are always readable;

TID memory is always readable and unwritable.

If the Reserved memory is set to permanent readable/writable or permanent unreadable/unwritable, it can not be set to other state any more.

If EPC or TID memory is set to permanent writable or permanent unwritable, it can not be changed to other state any more.

If a memory is set to password protected state, any access to this memory should be carried out with a correct password.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x06 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Select | SetProtect | Pwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 0xXX | 0xXX | 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Select: One byte. It specifies the operation target as follows:

0x00: Kill Password section. Kill Password section is the first half of the Reserved memory;

0x01: Access Password section. Access Password section is the second half of the Reserved memory;

0x02: EPC memory;

0x03: TID memory;

0x04: User memory.

Other values are reserved.

SetProtect: one byte. It specifies the protection strategy of the Select defined target memory.

When Select is 0x00 or 0x01, SetProtect has the following meaning::

0x00: unprotected readable/writable;.

0x01: permanent readable/writable;

0x02: password-protected readable/writable;

0x03: permanent unreadable/unwritable.

When Select is 0x02, 0x03 or 0x04, SetProtect has the following meaning:

0x00: unprotected writable;.

0x01: permanent writable;

0x02: password-protected writable;

0x03: permanent unwritable.

Other values are reserved.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x06 | 0x00 | - | LSB | MSB |

### 8.2.7 BlockErase

The command is used to erase multiple words in a Tag’s Reserved, EPC, TID or User memory. BlockErase is an advance but non-mandatory feature of EPC C1G2 tag and please check the tag’s datasheet before using it.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x07 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Mem | WordPtr | Num | Pwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 0xXX | 0xXX | 0xXX | 4Byte | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Mem: One byte. It specifies the target memory of the operation with 0x00 for Password memory, 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

WordPtr: One byte. It specifies the starting word address for the operation with 0x00 for the first word, 0x01 for the second word and so on. If the target memory is the EPC memory, the WordPtr should not be 0.

Num: One byte. It specifies the number of words to erase.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x07 | 0x00 | - | LSB | MSB |

### 8.2.8 Set Privacy with Mask Pattern

The command is used to set designated tag into privacy state. When a tag is in privacy state, it will ignore common command such as read, write and query etc. This is an advanced feature which is available for NXP’s UCODE EPC G2X tag.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x08 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Pwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password and the password should not be all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x08 | 0x00 | - | LSB | MSB |

### 8.2.9 Set Privacy without Mask Pattern

The command is used to set a tag in the field into privacy state. When a tag is in privacy state, it will ignore common command such as read, write and query etc. This is an advanced feature which is available for NXP’s UCODE EPC G2X tag.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Pwd |  |  |
| 0x08 | 0xXX | 0x09 | 4Bytes | LSB | MSB |

Parameter explanation:

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password and the password should not be all 0s.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x09 | 0x00 | - | LSB | MSB |

### 8.2.10 Reset Privacy

The command is used to release a tag from privacy state into normal state. This is an advanced feature which is available for NXP’s UCODE EPC G2X tag.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Pwd |  |  |
| 0x08 | 0xXX | 0x0a | 4Bytes | LSB | MSB |

Parameter explanation:

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password and the password should not be all 0s.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x0a | 0x00 | —— | LSB | MSB |

### 8.2.11 Check Privacy

The command is used to check whether a tag in the field is in privacy state or not.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x0b | - | LSB | MSB |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x06 | 0xXX | 0x0b | 0x00 | ReadPro | LSB | MSB |

Parameter explanation:

| ReadPro | Connect |
| --- | --- |
| 0x00 | Tag is not in privacy state. |
| 0x01 | Tag is in privacy state. |

### 8.2.12 EAS Configure

The function is used to set or reset the EAS bit of a designated tag. This is an advanced feature which is available for NXP’s UCODE EPC G2X tag.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x0c | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Pwd | EAS | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 4Bytes | 0xXX | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password and the password should not be all 0s.

EAS: One byte. Bit0=0 means reset EAS state, Bit0=1 means set EAS state. Bit1~Bit7 are reserved.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x0c | 0x00 | - | LSB | MSB |

### 8.2.13 EAS Alarm

The function is used to check EAS bit’s status of a tag in the field. This is an advanced feature which is available for NXP’s UCODE EPC G2X tag.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x0d | - | LSB | MSB |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x0d | 0x00 | - | LSB | MSB |

If no tag’s EAS bit is set, the response status will be “no tag in the field”.

### 8.2.14 Block Write

The command is used to write multiple words in a Tag’s Reserved, EPC, TID or User memory. BlockWrite is an advance but non-mandatory feature of EPC C1G2 tag and please check the tag’s datasheet before using it.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x10 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| WNum | ENum | EPC | Mem | WordPtr | Wdt |
| 0xXX | 0xXX | Variable | 0xXX | 0xXX | Variable |
| Pwd | MaskMem | MaskAdr | MaskLen | MaskData |  |
| 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |  |

Parameter explanation:

WNum: One byte. It specifies the number of words to be written into a tag. The value can not be 0 and should be equal to the word length of Wdt.

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Mem: One byte. It specifies the target memory of the operation with 0x00 for Password memory, 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

WordPtr: One byte. It specifies the starting word address for the operation with 0x00 for the first word, 0x01 for the second word and so on.

Wdt: the data to be written into a tag. The word length of Wdt should be equal to WNum and the data should be arranged as most significant word first and most significant byte in a word first.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x10 | 0x00 | - | LSB | MSB |

### 8.2.15 Read Monza4QT parameters

The command is used to read Monza4Qt work parameters.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x11 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Pwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | recmd | Status | Data[] |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NC | QTcontrol |  |  |
| 0x07 | 0xXX | 0x11 | 0x00 | 0x00 | 1 byte | LSB | MSB |

Parameter explanation:

QTcontrol: Tag work parameters.

Bit0：mirror page.bit0=0:private;bit0=1:public.

Bit1：enabled distance protect or not.bit1=0:diabled;bit1=1:enabled.

### 8.2.16 Set Monza4QT parameters

The command is used to Set Monza4Qt work parameters.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x12 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | QTcontrol1 | QTcontrol0 | Pwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 0xXX | 0xXX | 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

QTcontrol1: reserved，0x00;

QTcontrol0: Tag work parameters.

Bit0：mirror page.bit0=0:private;bit0=1:public.

Bit1：enabled distance protect or not.bit1=0:diabled;bit1=1:enabled.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x12 | 0x00 | - | LSB | MSB |

Parameter explanation:

QTcontrol: Tag work parameters.

Bit0：mirror page.bit0=0:private;bit0=1:public.

Bit1：enabled distance protect or not.bit1=0:diabled;bit1=1:enabled.

### 8.2.17 Extended Read Data

The command is used to read part or all of a Tag’s Password, EPC, TID, or User memory.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x15 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Mem | WordPtr | Num | Pwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 0xXX | 2Bytes | 0xXX | 4Byte | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Mem: One byte. It specifies the target memory of the operation with 0x00 for Password memory, 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

WordPtr: Two byte. It specifies the starting word address for the operation with 0x0000 for the first word, 0x0001 for the second word and so on.

Num: One byte. It specifies the number of words to read. The value range is from 1 to 120. Other value will incur a parameters error.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x15 | 0x00 | Word1，Word2,… | LSB | MSB |

Parameter explanation:

Word1, Word2….: read out data in word units with most significant byte of a word first. Word1 is the content of the start-address-defined word in target memory and so on.

### 8.2.18 Extended Write Data

The command is used to write several words into a Tag’s Reserved, EPC, TID or User memory.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x016 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| WNum | ENum | EPC | Mem | WordPtr | Wdt |
| 0xXX | 0xXX | Variable | 0xXX | 2Bytes | Variable |
| Pwd | MaskMem | MaskAdr | MaskLen | MaskData |  |
| 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |  |

Parameter explanation:

WNum: One byte. It specifies the number of words to be written into a tag. The value can not be 0 and should be equal to the word length of Wdt.

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Mem: One byte. It specifies the target memory of the operation with 0x00 for Password memory, 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

WordPtr: Two byte. It specifies the starting word address for the operation with 0x0000 for the first word, 0x0001 for the second word and so on.

Wdt: the data to be written into a tag. The word length of Wdt should be equal to WNum and the data should be arranged as most significant word first and most significant byte in a word first.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x16 | 0x00 | - | LSB | MSB |

### 8.2.19 Mix Inventory

The command is used to inventory tags in the field, then get their EPC values. And read specified tag data at the same time.

This command is executed within predefined InventoryScanTime limit. The default InventoryScanTime is 1 second (20*100ms). The allowable value range is from 3*100ms to 255*100ms.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x019 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| QValue | Session | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | 0xXX | 0xXX | 2Bytes | 0xXX | Variable |

| Data[] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ReadMem | ReadAdr | ReadLen | Pwd | Target | Ant | Scantime |
| 0xXX | 2Bytes | 0xXX | 4Bytes | 0xXX | 0xXX | 0xXX |

Parameter explanation:

QValue: One byte, Q value,

Bit7: Statistical data packets flag;

0: not send the flag packets when query finished.

1: send the flag packets when query finished.

Bit6-Bit0: range is 0-15. Q value setting should be field tag quantity is approximately equal to 2 Q.

other value return parameter error.

Session: One byte, Session value,

0x00:S0;

0x01:S1;

0x02:S2;

0x03:S3;

0xFF: Auto config session. (Only EPC query effective).

other value return parameter error.

MaskMem: One byte. It specifies the target memory when applying inventory mask with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address of the mask pattern data. The value ranges from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern data. The value ranges from 0 to 255.

MaskData: Mask pattern data. The byte length of the MaskData is MaskLen/8. If MaskLen is not 8bits integer times, the length of MaskData should be int[MaskLen/8]+1 with 0 patching in the low significant location.

ReadMem: One byte. It specifies the target memory of the operation with 0x00 for Password memory, 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

ReadAdr: Two byte. It specifies the start word of read data.0x0000 is first word, 0x0001 is second word,and so on.

ReadLen: One byte. It specifies the length of word to read.the value ranges form 1 to 120.

Pwd: Four byte. It specifies the access-password of tag.

Target(Optional parameters):One byte,

0x00：Target value is A;

0x01：Target value is B;

Other values are reserved.

Ant(Optional parameters):One byte,antenna number this time to query.

0x80: antenna 1;

0x81: antenna 2;

0x82: antenna 3;

0x83: antenna 4.

ScanTime(Optional parameters): One byte,reader will set max scan time to ScanTime*100ms.

Remark:

- The MaskMem , MaskAdr, MaskLen and MaskData can be all vacant. That means tags will be inventoried without mask pattern.

- The Target, Ant, ScanTime is optional parameters,these parameters must be used to together.

(1).When Status is 0x26

Response:

| Len | Adr | reCmd | Status | Data[] |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ant | ReadRate | TotalCount |  |  |
| 0x0C | 0xXX | 0x19 | 0x26 | 0xXX | 2 Bytes | 4 Bytes | LSB | MSB |

Parameter explanation:

Ant: It describes from which antenna the tag EPC is collected. For example, Ant = 0000 0100b means ANT3 and Ant = 0000 1000b means ANT4, etc.

ReadRate: 2 Bytes, Tag identification rate (the number of successfully read tag/SEC). Don't distinguish whether read many times with same tag

TotalCount: Total count of read tag,Don't distinguish whether read many times with same tag.

(2).When Status is not 0x26

Response:

| Len | Adr | reCmd | Status | Data[] |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ant | Num | Data Packet |  |  |
| 0xXX | 0xXX | 0x19 | 0xXX | 0xXX | 0xXX | Packet-1, Packet -2, Packet -3… | LSB | MSB |

Parameter explanation:

Status:

| Value | Comment |
| --- | --- |
| 0x01 | Tag inventory operation is completely finished in defined InventoryScanTime and all tags’ EPC data are returned. |
| 0x02 | Tag inventory operation is not completely finished when defined InventoryScanTime overflows and some tags’ EPC data are returned. |
| 0x03 | The number of the inventoried tags is too much and not all tags’ EPC data can be returned within this single response data block. Other response data blocks are followed. |
| 0x04 | Tag inventory operation is finished since the number of the inventoried tags reaches the reader process limit. All inventoried tags’ EPC data are returned. |
| 0xF8 | If Qvalue’s bit8=1.when inventory finished,return this code, it’s rate packet |

Ant: It describes from which antenna the tag EPC is collected. For example, Ant = 0000 0100b means ANT3 and Ant = 0000 1000b means ANT4, etc.

Num: The number of tag detected.

Data Packet: Inventoried tag’s data. Packet-1 is the first packet data, Packet-2 is the second packet data,and so on. Each packet as follows:

| PacketParam | Len | Data | RSSI |
| --- | --- | --- | --- |
| 0xXX | 0xXX | Variable | 0xXX |

Packet explanation:

PacketParam: Packet parameters, 1 byte.

Bit7: packet types.

0 - this packet with information for the tag EPC;

1 - this packet with information for the tag Data (the data of tag and the last read EPC is same one).

Bit6 ~ Bit0: packet sequence number. Packet number ranges from 0 to 127. Each mix-inventory, packet number will start from 0, per upload a packet, the packet sequence number will automatically add 1, when the packet number more than 127, will start from 0

Len: the langth of Data.

Data:Tag’s EPC or Data.

RSSI:the signal strength when read the tag.

## 8.3 18000-6B COMMAND

## 8.4 READER-DEFINED COMMAND

### 8.4.1 Get Reader Information

The command is used to get the reader’s information such as firmware version, reader type code, supporting protocol, RF power, work frequency band, InventoryScanTime etc.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x21 | - | LSB | MSB |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x10 | 0xXX | 0x21 | 0x00 | Version, Type, Tr_Type, dmaxfre, dminfre, Power, Scntm, Ant1, BeepEn, Ant2, CheckAnt | LSB | MSB |

Parameter explanation:

| Parameter | Length(Byte) | Connect |
| --- | --- | --- |
| Version | 2 | The first byte specifies the main version and the second byte specifies the subversion. |
| Type | 1 | The reader type code with 0x8C-UHFREADER288A12, |
| Tr_Type | 1 | It specifies the protocol the reader supports. For UHFREADER288, two bits (bit1&bit0) are set to 1 with bit1=1 for ISO18000-6C and bit0=1 for ISO18000-6B. |
| dmaxfre | 1 | Bit7~Bit6 is used to indicate frequency band and Bit5~Bit0 is used to specify the maximum frequency point. As to the frequency band definition, please refer to the following table. Please also refer to 8.4.2. |
| dminfre | 1 | Bit7~Bit6 is used to indicate frequency band and Bit5~Bit0 is used to specify the minimum frequency point. As to the frequency band definition, please refer to the following table. Please also refer to 8.4.2. |
| Power | 1 | It specifies the reader’s RF output power. The value range is 0 to 30 with 30 for 30dbm. |
| Scntm | 1 | It specifies the InventoryScanTime. Please also refer to 8.2.1 |
| Ant1 | 1 | Antenna 1-8 config information |
| BeepEn | 1 | It specifies the beep is enabled or not. |
| Ant2 | 1 | Antenna 9-12 config information |
| CheckAnt | 1 | It specifies the Antanna checked before query,please also refer to 8.4.13 |

Frequency Band Table:

| dmaxfre(Bit7) | dmaxfre(Bit6) | dminfre(Bit7) | dminfre(Bit6) | Region Frequency Band |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | RFU |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 0 | 1 | RFU |
| 0 | 1 | 1 | 0 | Ukraine band |
| 0 | 1 | 1 | 1 | Peru band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| … | … | … | … | … |
| 1 | 1 | 1 | 1 | RFU |

### 8.4.2 Set Region

The command is used to set the reader’s working frequency band and the maximum and minimum frequency point in the band.

Command:

| Len | Adr | Cmd | Data[] |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MaxFre | MinFre |  |  |
| 0x06 | 0xXX | 0x22 | 0xXX | 0xXX | LSB | MSB |

Parameter explanation:

MaxFre: one byte. Bit7~Bit6 is used to indicate frequency band and Bit5~Bit0 is used to specify the maximum frequency point. As to the frequency band definition, please refer to the following table.

MinFre: one byte. Bit7~Bit6 is used to indicate frequency band and Bit5~Bit0 is used to specify the minimum frequency point. As to the frequency band definition, please refer to the following table.

Please note that MaxFre should be greater than MinFre.

Frequency Band Table:

| dmaxfre(Bit7) | dmaxfre(Bit6) | dminfre(Bit7) | dminfre(Bit6) | Region Frequency Band |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | RFU |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 0 | 1 | RFU |
| 0 | 1 | 1 | 0 | Ukraine band |
| 0 | 1 | 1 | 1 | Peru band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| … | … | … | … | … |
| 1 | 1 | 1 | 1 | RFU |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x22 | 0x00 | - | LSB | MSB |

Various Region Frequency Band Calculation:

Chinese band2:			Fs = 920.125 + N * 0.25 (MHz), N∈ [0, 19].

US band:					Fs = 902.75 + N * 0.5 (MHz), N∈ [0, 49].

Korean band:				Fs = 917.1 + N * 0.2 (MHz), N∈ [0, 31].

EU band:				    Fs = 865.1 + N*0.2(MHz) N∈[0, 14].

Ukraine band:			    Fs = 868.0 + N*0.1(MHz) N∈[0, 6].

Peru band：			    Fs = 916.2 + N*0.9(MHz) N∈[0, 11].

Chinese band1：		    Fs = 840.125 + N * 0.25 (MHz) N∈[0, 19].

### 8.4.3 Set Address

This command is used to set the reader’s address.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Address |  |  |
| 0x05 | 0xXX | 0x24 | 0xXX | LSB | MSB |

Parameter explanation:

Address: one byte. It specifies the reader’s address and the value range is 0 to 254. 255 is the broadcasting address.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x24 | 0x00 | - | LSB | MSB |

Notes: The Adr in response is the old address and not the new address.

### 8.4.4 Set InventoryScanTime

The command is used to set the InventoryScanTime.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Scantime |  |  |
| 0x05 | 0xXX | 0x25 | 0xXX | LSB | MSB |

Parameter explanation:

Scantime: one byte. It specifies the InventoryScan Time. Please also refer to 8.2.1.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x25 | 0x00 | - | LSB | MSB |

### 8.4.5 Set Baud Rate

The command is used to change the communication baud rate.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BaudRate |  |  |
| 0x05 | 0xXX | 0x28 | 0xXX | LSB | MSB |

Parameter explanation:

BaudRate: one byte. It specifies the communication baud rate as following table.

| BaudRate | Bps |
| --- | --- |
| 0 | 9600bps |
| 1 | 19200 bps |
| 2 | 38400 bps |
| 5 | 57600 bps (default) |
| 6 | 115200 bps |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x28 | 0x00 | - | LSB | MSB |

Notes: The response is still using the old baud rate. After this, later communication will use new baud rate.

### 8.4.6 Set RF Power

The command is used to adjust the RF output power of the reader.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Pwr |  |  |
| 0x05 | 0xXX | 0x2F | 0xXX | LSB | MSB |

Parameter explanation:

Pwr: one byte. It specifies the RF output power. The value range is from 0 to 30 with 30 for around 30dbm.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x2F | 0x00 | - | LSB | MSB |

### 8.4.7 BuzzerAndLED Control

The command is used to control Buzzer and LED action.

Command:

| Len | Adr | Cmd | Data[] |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ActiveT | SilentT | Times |  |  |
| 0x07 | 0xXX | 0x33 | 0xXX | 0xXX | 0xXX | LSB | MSB |

Parameter explanation:

ActiveT: one byte. It specifies the time duration that the LED and the buzzer are on. The time duration is ActiveT*50ms and the default ActiveT value is 0.

SilentT: one byte. It specifies the time duration that the LED and the buzzer are off. The time duration is SilentT *50ms and the default SilentT value is 0.

Times: one byte. It specifies the repeat times that the LED and buzzer go on and off. The default Times value is 0.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x33 | 0x00 | - | LSB | MSB |

### 8.4.8 Set Work Mode

The command is used to set the work mode of the reader. The reader supports 4 work modes that are answer mode, auto-running mode, low-level-active trigger mode and high-level-active trigger mode. When the reader is in auto-running mode, it can only receive and executive reader-defined commands.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Reader_mode |  |  |
| 0x05 | 0xXX | 0x35 | 0xXX | LSB | MSB |

Parameter explanation:

Reader_mode:

| Bit2 | Bit1 | Bit0 | Work Mode |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Answer Mode |
| 0 | 0 | 1 | Auto-running Mode |
| 0 | 1 | 0 | Low-level-active Trigger Mode |
| 0 | 1 | 1 | High-level-active Trigger Mode |

Bit3~Bit7: reserved.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x35 | 0x00 | - | LSB | MSB |

Output Data Format in auto-running mode and trigger mode:

- Tag inventory related operation:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0xee | 0x00 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |
| --- | --- | --- | --- | --- |
| DiscTime | LastTime | Count | Ant | EPC/TID ID |
| 4Bytes | 4Bytes | 2Bytes | 0xXX | Variable |

Parameter explanation:

DiscTime: 4 bytes. It specifies the time stamp when the tag is firstly detected.

| DiscTime |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Year | Month | Date | Hour | Minute | Second |
| Bit31~Bit26 | Bit25~Bit22 | Bit21~Bit17 | Bit16~Bit12 | Bit11~Bit6 | Bit5~Bit0 |

Year: 6bits. It ranges from 0 to 63 for year 2000 to 2063.

Month: 4bits. It ranges from 1 to 12 for Jan. to Dec.

Date: 5bits. It ranges from 1 to 31 for day 1 to day 31.

Hour: 5bits. It ranges from 0 to 23.

Minute: 6bits. It ranges from 0 to 59.

Second: 6bits. It ranges from 0 to 59.

LastTime: 4 bytes. It specifies the time stamp when the tag is latest detected.

| LastTime |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Year | Month | Date | Hour | Minute | Second |
| Bit31~Bit26 | Bit25~Bit22 | Bit21~Bit17 | Bit16~Bit12 | Bit11~Bit6 | Bit5~Bit0 |

Year: 6bits. It ranges from 0 to 63 for year 2000 to 2063.

Month: 4bits. It ranges from 1 to 12 for Jan. to Dec.

Date: 5bits. It ranges from 1 to 31 for day 1 to day 31.

Hour: 5bits. It ranges from 0 to 23.

Minute: 6bits. It ranges from 0 to 59.

Second: 6bits. It ranges from 0 to 59.

Count: 2 bytes. It specifies the times the tag has been detected during time interval from DiscTime to LastTime.

Ant: two byte. It specifies which antenna or antennae have detected the tag. It uses BCD coding. For example, Ant as 0000 0110 0000 1000b means antenna 4,10 and antenna 11 have detected the tag.

EPC/TID ID: Detected tag’s EPC/TID data. The first byte is EPC/TID length and the following bytes are EPC/TID data with most significant word of EPC/TID transmitted first and the most significant byte of a word also transmitted first.

2. Check Tag’s EAS operation

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0xee | 0xee | - | LSB | MSB |

When EAS alarm is activated, the built-in relay will pick up for about 3s.

Note: When EAS checking is arranged before a tag inventory operation, the reader will not give out above response message even an EAS alarm activated but the relay can still act and pick up for about 3s.

### 8.4.9 Get System Parameters

The command is used to get the current set parameters of the reader for tag operation and work mode configuration.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x36 | - | LSB | MSB |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x32 | 0xXX | 0x36 | 0x00 | Reader_mode, Accuracy, RepCondition, RepPauseTime, ReadPauseTim，TagProtocol, MaskMem, MaskAdr，MaskLen, MaskData, TriggerTime, AdrTID, LenTID | LSB | MSB |

Parameter explanation:

Reader_mode: one byte. It specifies the current work mode. Please also refer to 8.4.8.

Accuracy: one byte. It specifies the EAS checking sensitivity and accuracy. Please also refer to 8.4.10.

RepCondition, RepPauseTime: one byte each. They specify the response style parameters when the reader in auto-running and trigger mode. Please also refer to 8.4.12.

ReadPauseTim: one byte. It specifies the inventory interval when the reader in auto-running and trigger mode. Please also refer to 8.4.13.

TagProtocol: one byte. It specifies the tag type when the reader runs a tag related operation command. Please also refer to 8.4.14.

MaskMem, MaskAdr, MaskLen, MaskData: Mask pattern parameters. Please also refer to 8.4.11.

TriggerTime: one byte. It specifies the trigger delay time. Please also refer to 8.4.25.

AdrTID, LenTID: one byte each. They specify TID-inventory parameters. Please also refer to 8.4.26.

Note: TriggerTime, AdrTID, LenTID are available for readers with firmware version 1.20 or above.

### 8.4.10 Set EAS Sensitivity

The command is used to set the EAS checking sensitivity and accuracy. It also defines the relay action when an EAS alarm is activated.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Accuracy |  |  |
| 0x05 | 0xXX | 0x37 | 0xXX | LSB | MSB |

Parameter explanation:

Accuracy: one bit (bit7). It specifies the relay action when an EAS alarm is activated.

=1  pick-up for about 3s.

=0  no action.

bit6~bit0: EAS alarm accuracy definition. The value range is from 0 to 8. Smaller value means higher sensitivity but lower accuracy. The default value is 8.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x37 | 0x00 | - | LSB | MSB |

### 8.4.11 Set Mask

The command is used to set the mask pattern for the G2 tag related operation when the reader is in auto-running and trigger mode.

Command:

| Len | Adr | Cmd | Data[] |  |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MaskMem | MaskAdr | MaskLen | MaskData |  |  |
| 0xXX | 0xXX | 0x3b | 0xXX | 2Bytes | 0xXX | Variable | LSB | MSB |

Parameter explanation:

MaskMem: One byte. It specifies the target memory when applying inventory mask with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address of the mask pattern data. The value ranges from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern data. The value ranges from 0 to 255.

MaskData: Mask pattern data. The byte length of the MaskData is MaskLen/8. If MaskLen is not 8bits integer times, the length of MaskData should be int[MaskLen/8]+1 with 0 patching in the low significant location.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x3b | 0x00 | - | LSB | MSB |

### 8.4.12 Set Response Style of Auto-running Mode

The command is used to set the response style when the reader is in auto-running and trigger mode.

Command:

| Len | Adr | Cmd | Data[] |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RepCondition | RepPauseTime |  |  |
| 0x06 | 0xXX | 0x3c | 0xXX | 0xXX | LSB | MSB |

Parameter explanation:

RepCondition: one byte. It specifies the response style:

0x00: Command notification. The reader will collect all the tags’ information and store them into its inner buffer. The reader will send back the buffer data when it receives the Get Tag Buffer Info command from the host.

0x01: Timer notification. The reader will give out tag’s information it collects during a pre-defined period RepPauseTime.

0x02: Add-in notification. The reader will give out a message when it gets a new tag’s information.

0x03: Delete notification. The reader will give out a message data of a tag that is going to be deleted from inner buffer when the buffer is full and the reader need to empty room for new-coming tag data.

0x04: Change notification. The reader will give out a message when the reader gets a new tag data or deletes a tag data.

Other values are reserved.

RepPauseTime: one byte. It specifies the time period when the reader runs in a Timer notification style.

Notes: The reader’s inner buffer can hold up to 97 tags’ data and it adopts a FIFO strategy when the 98th and later tag data comes.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x3c | 0x00 | - | LSB | MSB |

### 8.4.13 Set Inventory Interval

The command is used to set the time interval when the reader does consecutive inventory operations in auto-running and trigger mode.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | ReadPauseTim |  |  |
| 0x05 | 0xXX | 0x3d | 0xXX | LSB | MSB |

Parameter explanation:

RepPauseTime: one byte. It specifies the time interval of consecutive inventory operations.

| ReadPauseTim | Time interval |
| --- | --- |
| 0x00 | 10ms |
| 0x01 | 20ms |
| 0x02 | 30ms |
| 0x03 | 50ms |
| 0x04 | 100ms |
| Other value | Reserved |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x3d | 0x00 | - | LSB | MSB |

### 8.4.14 Select Tag Type

The command is used to define the tag type when the reader runs a tag related operation command in auto-running mode and trigger mode.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | TagProtocol |  |  |
| 0x05 | 0xXX | 0x3e | 0xXX | LSB | MSB |

Parameter explanation:

TagProtocol: one byte. It specifies the tag type and operation.

| TagProtocol | Result |
| --- | --- |
| 0x00 | G2 tag EPC inventory |
| 0x01 | G2 tag EPC inventory and EAS checking |
| 0x02 | G2 tag EAS checking |
| 0x10 | G2 tag TID-inventory |
| 0x11 | G2 tag TID-inventory and EAS checking |
| 0x80 | ISO18000-6B tag inventory |
| Other value | Reserved |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x3e | 0x00 | - | LSB | MSB |

### 8.4.15 Set Antenna Multiplexing

The command is used to set the antenna multiplexing scenario.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ant |  |  |
| 0x05 | 0xXX | 0x3f | 0xXX | LSB | MSB |

Parameter explanation:

Ant: one byte. It specifies the antenna multiplexing scenario. The reader has 4 antenna ports and each antenna port can be individually enable or disable. At least one antenna port should be enabled.

Bit0: =0: ANT1 disabled.

=1: ANT1 enabled.

Bit1: =0: ANT2 disabled.

=1: ANT2 enabled.

Bit2: =0: ANT3 disabled.

=1: ANT3 enabled.

Bit3: =0: ANT4 disabled.

=1: ANT4 enabled.

Bit4~Bit7: Reserved.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x3f | 0x00 | - | LSB | MSB |

### 8.4.16 Set Beep Notification

The command is used to enable/disable beep notification.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BeepEn |  |  |
| 0x05 | 0xXX | 0x40 | 0xXX | LSB | MSB |

Parameter explanation:

BeepEn: Bit0. It specifies the status of beep notification.

=0: Beep notification disabled.

=1: Beep notification enabled. The inner buzzer will beep once whenever a tag related operation succeeds.

Bit1~Bit7: Reserved.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x40 | 0x00 | - | LSB | MSB |

### 8.4.17 Set Real-time Clock

The command is used to set the time of the inner real-time clock.

Command:

| Len | Adr | Cmd | Data[] |  |  |  |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Year | Month | Date | Hour | Minute | Second |  |  |
| 0x0a | 0xXX | 0x41 | 0xXX | 0xXX | 0xXX | 0xXX | 0xXX | 0xXX | LSB | MSB |

Parameter explanation:

Year: one byte. It ranges from 0 to 63 for year 2000 to 2063.

Month: one byte. It ranges from 1 to 12 for Jan. to Dec.

Date: one byte. It ranges from 1 to 31 for day 1 to day 31.

Hour: one byte. It ranges from 0 to 23.

Minute: one byte. It ranges from 0 to 59.

Second: one byte. It ranges from 0 to 59.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x41 | 0x00 | - | LSB | MSB |

Notes: The time to be set shall not be earlier than any time stamp of the tags stored in the reader’s inner buffer.

### 8.4.18 Get Time

The command is used to get the current time of the reader’s inner real-time clock.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x42 | - | LSB | MSB |

Response:

| Len | Adr | reCmd | Status | Data[] |  |  |  |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Year | Month | Date | Hour | Minute | Second |  |  |
| 0x0b | 0xXX | 0x42 | 0x00 | 0xXX | 0xXX | 0xXX | 0xXX | 0xXX | 0xXX | LSB | MSB |

Parameter explanation:

Year: one byte. It ranges from 0 to 63 for year 2000 to 2063.

Month: one byte. It ranges from 1 to 12 for Jan. to Dec.

Date: one byte. It ranges from 1 to 31 for day 1 to day 31.

Hour: one byte. It ranges from 0 to 23.

Minute: one byte. It ranges from 0 to 59.

Second: one byte. It ranges from 0 to 59.

### 8.4.19 Get Tag Buffer Info

The command is used to fetch the tag information stored in the reader’s inner buffer.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x43 | - | LSB | MSB |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x43 | 0xXX | - | LSB | MSB |

Parameter explanation:

Status: one byte. If it is 0x00, it means no data is available. If it is 0x03, it means the data is presented in the response block but not all data has been fetched. Other data is available in following response blocks.

Data[]: the structure of Data[] is:

| Data[] |  |  |  |  |
| --- | --- | --- | --- | --- |
| DiscTime | LastTime | Count | Ant | EPC-ID |
| 4Bytes | 4Bytes | 2Bytes | 0xXX | Variable |

DiscTime: 4 bytes. It specifies the time stamp when the tag is firstly detected.

| DiscTime |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Year | Month | Date | Hour | Minute | Second |
| Bit31~Bit26 | Bit25~Bit22 | Bit21~Bit17 | Bit16~Bit12 | Bit11~Bit6 | Bit5~Bit0 |

Year: 6bits. It ranges from 0 to 63 for year 2000 to 2063.

Month: 4bits. It ranges from 1 to 12 for Jan. to Dec.

Date: 5bits. It ranges from 1 to 31 for day 1 to day 31.

Hour: 5bits. It ranges from 0 to 23.

Minute: 6bits. It ranges from 0 to 59.

Second: 6bits. It ranges from 0 to 59.

LastTime: 4 bytes. It specifies the time stamp when the tag is latest detected.

| LastTime |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Year | Month | Date | Hour | Minute | Second |
| Bit31~Bit26 | Bit25~Bit22 | Bit21~Bit17 | Bit16~Bit12 | Bit11~Bit6 | Bit5~Bit0 |

Year: 6bits. It ranges from 0 to 63 for year 2000 to 2063.

Month: 4bits. It ranges from 1 to 12 for Jan. to Dec.

Date: 5bits. It ranges from 1 to 31 for day 1 to day 31.

Hour: 5bits. It ranges from 0 to 23.

Minute: 6bits. It ranges from 0 to 59.

Second: 6bits. It ranges from 0 to 59.

Count: 2 bytes. It specifies the times the tag has been detected during time interval from DiscTime to LastTime.

Ant: one byte. It specifies which antenna or antennae have detected the tag. It uses BCD coding. For example, Ant as 00000101b means antenna 1 and antenna 3 have detected the tag.

EPC/TID ID: Detected tag’s EPC/TID data. The first byte is EPC/TID length and the following bytes are EPC/TID data with most significant word of EPC/TID transmitted first and the most significant byte of a word also transmitted first.

### 8.4.20 Clear Tag Buffer

The command is used to clear the reader’s inner buffer.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x44 | - | LSB | MSB |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x44 | 0x00 | - | LSB | MSB |

### 8.4.21 Set Relay

The command is used to control the built-in relay.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | RelayTime |  |  |
| 0x05 | 0xXX | 0x45 | 0xXX | LSB | MSB |

Parameter explanation:

RelayTime: one byte. It specifies the relay pick-up time. The value is 1~255 for (1~255)*50ms pick-up time.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x45 | 0x00 | - | LSB | MSB |

### 8.4.22 Set GPIO

The command is used to set the GPIO output TTL level. The default is TTL high level.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | OutputPin |  |  |
| 0x05 | 0xXX | 0x46 | 0xXX | LSB | MSB |

Parameter explanation:

OutputPin: one byte. It specifies the 4 output GPIOs’ (OUT1~OUT4) level. Bit4~Bit7 correspond to OUT1~OUT4.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x46 | 0x00 | - | LSB | MSB |

### 8.4.23 Get GPIO Status

The command is used to get the current 4 GPIOs output status.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x47 | - | LSB | MSB |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | OutputPin |  |  |
| 0x06 | 0xXX | 0x47 | 0x00 | 0xXX | LSB | MSB |

Parameter explanation:

OutputPin: one byte. It specifies the 4 output GPIOs’ (OUT1~OUT4) level. Bit4~Bit7 correspond to OUT1~OUT4.

### 8.4.24 Set Trigger Delay Time

The command is used to set the trigger delay time. When the reader in trigger mode, it will keep working for a pre-defined time period even if the trigger source has been removed.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | TriggerTime |  |  |
| 0x05 | 0xXX | 0x49 | 0xXX | LSB | MSB |

Parameter explanation:

TriggerTime: one byte. It specifies the trigger delay time. The value range is 0~255 for (0~255)*1s trigger delay time.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x49 | 0x00 | - | LSB | MSB |

Notes: The command is only available for readers with firmware version 1.20 or above.

### 8.4.25 Set TID-Inventory Parameters

The command is used to set TID-Inventory parameters. Please also refer to 8.2.1.

Command:

| Len | Adr | Cmd | Data[] |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AdrTID | LenTID |  |  |
| 0x06 | 0xXX | 0x4a | 0xXX | 0xXX | LSB | MSB |

Parameter explanation:

AdrTID: One byte. It specifies the start word address in TID memory when doing the TID-inventory.

LenTID: One byte. It specifies the number of words when doing the TID-inventory. The range is 0~ 15.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x4a | 0x00 | - | LSB | MSB |

Notes: The command is only available for readers with firmware version 1.20 or above.

### 8.4.26 Get Reader Serial Number

The command is used to get reader’s serial number.

Command

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x4c | - | LSB | MSB |

Parameter explanation:

None.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | SerialNo |  |  |
| 0x09 | 0xXX | 0x4c | 0x00 | 4bytes | LSB | MSB |

SerialNo:reader’s serial number,4bytes.

Notes: The command is only available for readers with firmware version 1.20 or above.

### 8.4.27 Change AT Mode

The command is used to change network module to AT mode or exit..

Command

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | ATMode |  |  |
| 0x05 | 0xXX | 0x4d | 0xXX | LSB | MSB |

Parameter explanation:

ATMode: One byte.network module mode select.

0x01,AT mode;

0x00,exit AT mode.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x4d | 0x00 | - | LSB | MSB |

Notes: The command is only available for readers with firmware version 1.20 or above.

### 8.4.28 AT Transparent Command

The command is used to send transparent cmd to network module.

Command

| Len | Adr | Cmd | Data[] |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TimeOut | ATCmd |  |  |
| 0x05+N | 0xXX | 0x4e | 0xXX | N bytes | LSB | MSB |

Parameter explanation:

TimeOut: One byte.the cmd operation timeout(1~255)*100ms.

ATCmd: Nbytes.the datas of user send to network module.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ATResp |  |  |
| 0x05+N | 0xXX | 0x4e | 0x00 | N bytes | LSB | MSB |

ATResp: Nbytes.the datas of network module response.

Notes: The command is only available for readers with firmware version 1.20 or above.

### 8.4.29Tag custom function

The command is used to set enable some type tag custom work mechanism, to achieve a specific function.

Command

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | InlayType |  |  |
| 0x05 | 0xXX | 0x3a | 0xXX | LSB | MSB |

Parameter explanation:

InlayType: One byte.range is 0~254

The default value is 0, do not specify a tag type.

Value 1 enable the Monza4QT Peek function of tag (tag status temporary from public to private), it will affect the tag reading data, writing data, the piece of writing, write protect words and write the EPC number etc

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x06 | 0xXX | 0x3a | 0x00 | InlayType | LSB | MSB |

InlayType: tag type.0~255..

Notes: The command is only available for readers with firmware version 1.20 or above.

### 8.4.30 Set antenna checked

The command is used to set the antenna checked before query tag.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | CheckAnt |  |  |
| 0x05 | 0xXX | 0x66 | 0xXX | LSB | MSB |

Parameter explanation:

CheckAnt: one byte.check antenna flag.

0x00:Disabled;

0x01:Enabled;

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x66 | 0x00 | - | LSB | MSB |

### 8.4.31 Set Write Power

The command is used to set power parameter when do write operation

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | WritePower |  |  |
| 0x05 | 0xXX | 0x79 | 0xXX | LSB | MSB |

Parameter explanation:

WritePower: one byte. power parameters.

Bit7: whether enable WritePower

0 -Disable;

1 -Enable.

Bit6 ~ Bit0:Power, range is 0 to 30.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x79 | 0x00 | - | LSB | MSB |

### 8.4.32 Get Write Power

The command is used to Get power parameter when do write operation

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x7A | - | LSB | MSB |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | WritePower |  |  |
| 0x05 | 0xXX | 0x79 | 0x00 | - | LSB | MSB |

Parameter explanation:

WritePower: one byte. power parameters.

Bit7: whether enable WritePower

0 -Disable;

1 -Enable.

Bit6 ~ Bit0:Power, range is 0 to 30.

### 8.4.33 Get or Set Rerty times

The command is used to get or set retry times when write failed.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | RetryTimes |  |  |
| 0x05 | 0xXX | 0x7B | 0xXX | LSB | MSB |

Parameter explanation:

HeartBeatTime: one byte. Retry times parameters.

Bit7: Get/set retry times flag.

0 - Get write retry times;

1 - Set write retry times.

Bit6 ~ Bit0: a heartbeat packet interval.

Bit6~Bit3: Reserved.

If Bit7 = 0, Bit2 ~ Bit0 will be ignored;

If Bit7 = 1, Bit2 ~ Bit0 is retry times when write operation failed.range is 0 to 7.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | RetryTimes |  |  |
| 0x06 | 0xXX | 0x7B | 0x00 | 0xXX | LSB | MSB |

### 8.4.35 Set detect multiple tag parameter

The command is used to set the detecting parameters when the reader is in auto-running or trigger mode

in auto-running or trigger mode,the detecting command would not be seted through these,just depend on the parameters in the command which would be sent, See section 8.2.1

Command

| Len | Adr | Cmd | Data[] |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | QValue | Session |  |  |
| 0x06 | 0xXX | 0x81 | 0xXX | 0xXX | LSB | MSB |

Parameter explanation:

QValue: one bits, It is the initial Q ​​used to detec EPC tag. The Q should be seted to meet the requirements  that 2Q venue should be approximately equal to the number of tags. The range is 0~15, when wrong Q turn up in the command, error message will be sent back.

Session：one bits，It is used to detec EPC tag.

0x00：Session ues S0；

0x01：Session use S1；

0x02：Session use S2；

0x03：Session use S3。

Keep other venue, when wrong Session turn up in the command, error message will be sent back

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x81 | 0x00 | - | LSB | MSB |

\

### 8.4.36 Get detect multiple tag parameter

The command is used to get the detecting parameters when detec mutiple tag in auto-running or trigger mode

Command

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x82 | - | LSB | MSB |

Parameter explanation:

None.

Response:

| Len | Adr | recmd | Status | Data[] |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | QValue | Sessionl |  |  |
| 0x07 | 0xXX | 0x82 | 0x00 | 0xXX | 0xXX | LSB | MSB |

### 8.4.37 R2000 module ROM update

The Command is used to change reader to update ROM of R2000-module mode,when reader Response this command,reader is upgrade mode,and Exit this mode requires the reader to power off and on.

Command

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x84 | - | LSB | MSB |

Parameter explanation:

None.

Response:

| Len | Adr | recmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x84 | 0x00 | - | LSB | MSB |