High Performance UHF RFID Fixed Reader

Model：UHFREADER288A12

Size：227mmx170mmx40mm

GENERAL DESCRIPTION

UHFREADER288A12 is a high performance UHF RFID fixed reader. It is designed upon fully self-intellectual property. Based on proprietory efficient digital signal processing algorithm, it supports fast tag read/write operation with high identification rate. It can be widely applied in many RFID application systems such as logistics, access control, anti-counterfeit and industrial production process control system.

FEATURES

- Self-intellectual property;

- Support ISO18000-6C(EPC C1G2) , ISO18000-6B protocol tag;

- 902~928MHz frequency band(frequency customization optional);

- FHSS or Fix Frequency transmission;

- RF output power up to 30dbm(adjustable);

- 4 TNC antenna port with antenna auto-tuning and failure-detection;

- Support auto-running, answer, trigger and real-time-inventory work mode;

- Tag buffer size up to 800PCS (Max. 496bits EPC length);

- Support EPC and TID inventory;

- Low power dissipation with single +9 DC power supply;

- Support RS232, RS485, TCPIP with POE optional;

- High reliability design;

- Support on-the-site firmware upgrading.

CHARACTERISTICS

- Absolute Maximum Rating

| ITEM | SYMBOL | VALUE | UNIT |
| --- | --- | --- | --- |
| Power Supply | VCC | 16 | V |
| Operating Temp. | TOPR | -10~+60 | ℃ |
| Storage Temp. | TSTR | -20~+75 | ℃ |

- Electrical and Mechanical Specification

Under TA＝25℃，VCC＝+9V unless specified

| ITEM | SYMBOL | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| Power Supply | VCC | 8 | 9 | 12 | V |
| Current Dissipation | IC |  | 800 | 1500 | mA |
| Frequency | FREQ | 902 | 902~928 | 928 | MHz |
| Size | Size |  | 227×170×40 |  | mm |

MECHANICAL DATA(UNIT mm)：

INTERFACE

1．Power (DC JACK)

| No. | Symbol | Comment |
| --- | --- | --- |
| Central | PWR | +9VDC |
| Outer | GND | Ground |

2．GPIO （DB15 Female）

| No. | Symbol | Comment |
| --- | --- | --- |
| 1 | Output1 | General output1 |
| 2 | Output2 | General output2 |
| 3 | Output3 | General output3 |
| 4 | Output4 | General output4 |
| 5 | Input1 | General Intput1 with Internal 47k resistor pulled-down to ground |
| 6 | Input2 | General Intput2 with Internal 47k resistor pulled-down to ground |
| 7 | Input3 | General Intput3 with Internal 47k resistor pulled-down to ground |
| 8 | Input4 | General Intput4 with Internal 47k resistor pulled-down to ground |
| 9 | TGIN | Trigger input with internal 10k resistor pulled-up to +5V |
| 10 | R+ | R+ in RS485 |
| 11 | R- | R- in RS485 |
| 12 | GND | Ground |
| 13 | NC | Normal-Close terminal of internal relay |
| 14 | NO | Normal-Open terminal of internal relay |
| 15 | CM | Common terminal of internal relay |

3.  UART （RS232 DB9 Female）

| No. | Symbol | Comment |
| --- | --- | --- |
| 1 | NC | Reserved |
| 2 | TXD | Data output in RS232 |
| 3 | RXD | Data input in RS232 |
| 4 | NC | Reserved |
| 5 | GND | Ground |
| 6 | NC | Reserved |
| 7 | NC | Reserved |
| 8 | NC | Reserved |
| 9 | NC | Reserved |

4．TCPIP network (RJ45)

5．TNC antenna port ANT1~ANT4

6．LED indicator LED1~LED7

| No. | Symbol | Comment |
| --- | --- | --- |
| 1 | LED1 | Antenna 1-4 active indicator |
| 2 | LED2 | Antenna 5-8 active indicator |
| 3 | LED3 | Antenna 9-12 active indicator |
| 4 | LED4 | Reserved |
| 5 | LED5 | Tag-detected indicator |
| 6 | LED6 | Command-executing indicator |
| 7 | LED7 | Power-on indicator |