# GENERAL DESCRIPTION

# ZK_RFID401 is a high performance UHF RFID fixed reader. It is designed upon fully self-intellectual property. Based on proprietory efficient digital signal processing algorithm, it supports fast tag read/write operation with high identification rate. It can be widely applied in many RFID application systems such as logistics, access control, anti-counterfeit and industrial production process control system.

# FEATURES

- Self-intellectual property;

- Support ISO18000-6C(EPC C1G2) , ISO18000-6B protocol tag;

- 902~928MHz frequency band(frequency customization optional);

- FHSS or Fix Frequency transmission;

- RF output power up to 30dbm(adjustable);

- 4 TNC antenna port;

- Support auto-running, answer and trigger work mode;

- Support EPC and TID inventory;

- Low power dissipation with single +9 DC power supply;

- Support RS232, RS485, TCPIP with other interface optional;

- Built-in LED, Buzzer, GPIO and Relay;

- High reliability design.

CHARACTERISTICS

- Absolute Maximum Rating

| ITEM | SYMBOL | VALUE | UNIT |
| --- | --- | --- | --- |
| Power Supply | VCC | 16 | V |
| Operating Temp. | TOPR | -10~+55 | ℃ |
| Storage Temp. | TSTR | -20~+75 | ℃ |

- Electrical and Mechanical Specification

Under TA＝25℃，VCC＝+9V unless specified

| ITEM | SYMBOL | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| Power Supply | VCC | 8 | 9 | 12 | V |
| Current Dissipation | IC |  | 600 | 1500 | mA |
| Frequency | FREQ | 902 |  | 928 | MHz |
| Size | Size |  | 160x186x50 |  | mm |

# INTERFACE

1．Power (DC JACK)

| No. | Symbol | Comment |
| --- | --- | --- |
| Central | PWR | +9VDC |
| Outer | GND | Ground |

2．GPIO （DB15 Female）

| No. | Symbol | Comment |
| --- | --- | --- |
| 1 | Output1 | General Output1 |
| 2 | Output2 | General Output2 |
| 3 | Output3 | General Output3 |
| 4 | Output4 | General Output4 |
| 5 | Output5 | General Output5 |
| 6 | Output6 | General Output6 |
| 7 | Output7 | General Output7 |
| 8 | Output8 | General Output8 |
| 9 | TGIN | Trigger input with internal pull-up to 5V through a 10k resistor |
| 10 | R+ | R+ of RS485 |
| 11 | R- | R- of RS485 |
| 12 | GND | Signal Ground |
| 13 | NO | Normal-Open terminal of internal relay |
| 14 | NC | Normal-Close terminal of internal relay |
| 15 | CM | Common terminal of internal relay |

3．Serial communication port RS232 （DB9 Male）

| No. | Symbol | Comment |
| --- | --- | --- |
| 1 | nc | Reserved |
| 2 | TXD | General Output2 |
| 3 | RXD | General Output3 |
| 4 | nc | Reserved |
| 5 | GND | Ground |
| 6 | nc | Reserved |
| 7 | nc | Reserved |
| 8 | nc | Reserved |
| 9 | nc | Reserved |

4．TCPIP network (RJ45)

5．TNC antenna port ANT1~ANT4

6．LED indicator LED1~LED7

| No. | Symbol | Comment |
| --- | --- | --- |
| 1 | LED1 | Antenna 1 active indicator |
| 2 | LED2 | Antenna 2 active indicator |
| 3 | LED3 | Antenna 3 active indicator |
| 4 | LED4 | Antenna 4 active indicator |
| 5 | LED5 | Tag-detected indicator |
| 6 | LED6 | Command-executing indicator |
| 7 | LED7 | Power-on indicator |