VI-88R

RFID Integrated Reader

Development Handbook

CONTENT

1	Applying development of unilateral communication	5

1.1	Applying development of unilateral communication	5

1.2	Wiegand interface agreement	5

1.2.1	Wiegand26 format	6

1.2.2	Wiegand34 format	6

1.3	RS485 interface agreement	6

2	Serial port intercommunication agreement	8

2.1	Summarize	8

2.1.1	Command packet format without address	8

2.1.2	Command packet format with address	9

2.1.3	Return packet format without address	9

2.1.4	Return packet format with address	10

2.1.5	Error code	10

2.1.6	For example	11

2.1.7	Checksum	11

2.2	Control command format of serial	12

2.2.1	Set BaudRate	12

2.2.2	Get Reader Version	13

2.2.3	Set Relay	15

2.2.4	Get Relay	16

2.2.5	Set Output Power	17

2.2.6	Set Frequency	18

2.2.7	Read Param	20

2.2.8	Set Param	21

2.2.9	Read Auto Param	22

2.2.10	Set Auto Param	23

2.2.11	Select Antenna	25

2.2.12	Restore factory settings	26

2.2.13	Reboot	27

2.2.14	Start/ stop automode	28

2.2.15	Clear Memory	28

2.2.16	Set Reader Time	28

2.2.17	Get Reader Time	29

2.2.18	Set Report Filter	30

2.2.19	Get Report Filter	31

2.2.20	Set Reader Network Address	32

2.2.21	Get Reader Network Address	32

2.2.22	SetReader MAC	33

2.2.23	Get Reader MAC	33

2.2.24	Report Now	33

2.2.25	Get Tag Info.	34

2.2.26	GetReaderID	35

2.3	Command format of serial read-write tag	35

2.3.1	Read-write ISO18000-6B command format	35

2.3.1.1	List Tag ID	36

2.3.1.2	Get ID List	37

2.3.1.3	List Selected ID	38

2.3.1.4	Read Byte Block	39

2.3.1.5	Write Byte Block	40

2.3.1.6	Set Write Protect	42

2.3.1.7	Read Write Protect	42

2.3.1.8	Slow Write A Byte	44

2.3.2	Read write ISO18000-6C command format	45

2.3.2.1	List Tag ID	45

2.3.2.2	Get ID List	47

2.3.2.3	Read Word Block	48

2.3.2.4	Write Word Block	50

2.3.2.5	Kill Tag	52

2.3.2.6	Set Lock	53

2.3.2.7	Write EPC	56

2.4	Collection of operation command	56

2.4.1	ISO18000-6C command	56

2.4.2	ISO18000-6B command	57

2.4.3	Other command	57

2.5	Electronic tag storage area and notes	58

3	SDK software development	61

3.1	SDK compose	61

3.2	Design introduction	61

3.2.1	Basic constant and figure	61

3.2.1.1	Constant definition	61

3.2.1.2	API function return code	61

3.2.1.3	Data type definition	62

3.2.1.4	Parameter figure in reader	62

3.2.1.5	Function return code	70

3.2.2	Control command function	71

3.2.2.1	Connect reader	71

3.2.2.2	Disconnection	73

3.2.2.3	Set baud rate	73

3.2.2.4	Read version	74

3.2.2.5	Set output power	74

3.2.2.6	Set operation frequency	75

3.2.2.7	Read reader basic operation parameter	76

3.2.2.8	Set reader basic operation parameter	76

3.2.2.9	Read reader auto parameter	77

3.2.2.10	Set reader auto parameter	78

3.2.2.11	Select antenna	78

3.2.2.12	Set relay state in reader	79

3.2.2.13	Reboot reader	80

3.2.2.14	Set time	80

3.2.2.15	Get time	81

3.2.2.16	Get record	81

3.2.2.17	Delete all records	82

3.2.3	Network command	83

3.2.3.1	Set IP address for reader	83

3.2.3.2	Get IP address in reader	84

3.2.3.3	Set reader MAC address	84

3.2.3.4	Get reader MAC address	85

3.2.4	Read write ISO18000-6B function	86

3.2.4.1	Identify tag ID number	86

3.2.4.2	Identify selected tag ID number	86

3.2.4.3	Read data block	88

3.2.4.4	Write data block	88

3.2.4.5	Slow write data block	89

3.2.4.6	Set write protection	90

3.2.4.7	Read write protection	91

3.2.5	Read write ISO18000-6C function	92

3.2.5.1	Identify EPC number of ISO18000-6C tag	92

3.2.5.2	Read a block data	93

3.2.5.3	Write a block data	94

3.2.5.4	Set read write protection state	96

- Applying development of unilateral communication

  - Applying development of unilateral communication

Apply in software development of continuous and trigger working mode, upper computer only need to receive tag ID numbers sent by reader, no need to send reader command.

  - Wiegand interface agreement

Wiegand interface transmites unilaterally, can send read card numbers to controller only, but controller can not send signal to reader. Signal transmitted from wiegand interface as follows:

At present, output waveform from Wiegand interface is as follows:

- 400us in pulse width，2.0ms in pulse interval

- 100us in pulse width，1.6ms in pulse interval

- 50us in pulse width，1.0ms in pulse interval

Wiegand interface has 2 types of Wiegand26 and Wiegand34.

    - Wiegand26 format

Wiegand26 transmits 26 bits data every time, 24 bits of them are valid. We stipulate these 24 bits correspondent for low 24 bits of electronic tag ID, or user defined number. Transmission format as follows:

Even: verified data adds parity bit1 is even number.

Odd: verified data adds parity bit1 is odd number.

    - Wiegand34 format

Wiegand34 transmits 32 bits of valid data every time. We stipulate these 32 bits correspondent for low 32 bits of electronic tag ID, or user defined number. Transmission format as follows:

  - RS485 interface agreement

When adopt RS485 interface to output data, need to set up communication rate of RS485 interface in advance. Output data format of RS485 is:

| Introduction: BODY |  |  |  | CHECK |
| --- | --- | --- | --- | --- |
| STX | DATA |  | ETX | BCC |
| 02 | Antenna serial number(2byte ASCII code) | ID number or User-defined number(N bytes ASCII code) | 03 | check code |

- Symbol of data start STX = 02H ，symbol of data end ETX = 03H；

- DATA is antenna serial number(2Byte)+tag ID number(NBytes)，length is N+2bytes ASCII code. Expression mode of converting HEX to ASCII:

|  | Length(byte) | Description |
| --- | --- | --- |
| STX | 1 | Symbol of data startSTX = 02H |
| Data | N | Antenna serial number(2byte ASCII code) ID number or User-defined number(N bytes ASCII code) |
| ETX | 1 | Symbol of data end ETX = 03H |
| Check | 1 | Check code |

Divides data from high to low, every 4 bits in a team, then put value of 4 binary bits in expression of ASCII code. As value range of 4 binary bits is 0H～FH, converted ASCII code is 30H～39H, 41H～46H. For example: data of 32 serial number is 6A90F103H, it is『36H 41H 39H 30H 46H 31H 30H 33H』after converting to 8 bytes ASCII code. Antenna number 1(ASCII code) is『30H 31H』,  antenna number 2(ASCII code) is『30H 32H』.

- Serial port intercommunication agreement

2 ways for application development:

- Use control code of serial port communication agreement to operate reader directly.

- User matched SDK software with reader to call API function to operate reader.

  - Summarize

In RFID application system, reader is connected with controller (PC) via RS232 port, and receives commands from controller, then returns the result of command execution. Therefore, we name Data Communication Packet from controller to reader, to be Command Packet, and name that from reader to controller, to be Return Packet.

    - Command packet format without address

| BootCode | Length | Command | Command Param | CheckSum |
| --- | --- | --- | --- | --- |

See chart above , command packet is composed of 5 parts:

|  | Length(byte) | Description |
| --- | --- | --- |
| BootCode | 1 | Fixed to be 40H |
| Length | 1 | Effective length. The length is total bytes of lateral 3 parts |
| Command | 1 | Command code |
| Command Param | uncertain | Command parameter, length is changing with command |
| CheckSum | 1 | Checksum, is all bytes from bootcode to command param, discard patch code |

    - Command packet format with address

| BootCode | Length | Command | Address | Command Param | CheckSum |
| --- | --- | --- | --- | --- | --- |

See chart above , command packet is composed of 6 parts:

|  | Length(byte) | Description |
| --- | --- | --- |
| BootCode | 1 | Fixed to be 40H |
| Length | 1 | Effective length. The length is total bytes of lateral 4 parts |
| Command | 1 | Command code |
| Address | 1 | Reader address, 1～254, 0 and 255 is broadcast address |
| Command Param | uncertain | Command parameter, length is changing with command |
| CheckSum | 1 | Checksum, is all bytes from bootcode to command param, discard patch code |

    - Return packet format without address

| BootCode | Length | Command | Return Data | CheckSum |
| --- | --- | --- | --- | --- |

See chart above, return packet is composed of 5 parts:

|  | Length(byte) | Description |
| --- | --- | --- |
| BootCode | 1 | When execute command correctly, bootcode of return packet is F0H. When execute command fail, bootcode of return packet is F4H. |
| Length | 1 | Effective length. The length is total bytes of lateral 3 parts. |
| Command | 1 | Command code, same as received command code, means that return packet is the reaction to the command. |
| Return Data | uncertain | Return the result of command execution. Length is changing with command. |
| CheckSum | 1 | Checksum, is all bytes from bootcode to ReturnData, discard patch code. |

    - Return packet format with address

| BootCode | Length | Command | Address | Return Data | CheckSum |
| --- | --- | --- | --- | --- | --- |

See chart above , return packet is composed of 6 parts:

|  | Length(byte) | Description |
| --- | --- | --- |
| BootCode | 1 | When execute command correctly, bootcode of return packet is F0H. When execute command fail, bootcode of return packet is F4H. |
| Length | 1 | Effective length. The length is total bytes of lateral 4 parts. |
| Command | 1 | Command code, same as received command code, means that return packet is the reaction to the command. |
| Address | 1 | Reader address, 1～254, 0 and 255 is broadcast address |
| Return Data | uncertain | Return the result of command execution. Length is changing with command. |
| CheckSum | 1 | Checksum, is all bytes from bootcode to ReturnData, discard patch code. |

    - Error code

When execute command fail, bootcode of return packet is F4H, and ReturnData is 1 byte of error code. Common error code is:

| 00(00H) | Command success or detect correct |
| --- | --- |
| 01(01H) | Antenna connection fail |
| 02(02H) | Detect no tag |
| 03(03H) | illegal tag |
| 04(04H) | read write power is inadequate |
| 05(05H) | Write protection in this area |
| 06(06H) | Checksum error |
| 07(07H) | Parameter wrong |
| 08(08H) | Nonexistent data area |
| 09(09H) | Wrong password |
| 10(0AH) | kill password can’t be 0 |
| 11(0BH) | When reader in Auto mode, the command is illegal. |
| 12(0CH) | Illegal user with unmatched password |
| 13(0dH) | RF interference from external |
| 14(0EH) | Read protection on tag |
| …… |  |
| 30(1EH) | Invalid command, such as wrong parameter command |
| 31(1FH) | Unknown command |
| 32(20H) | Other error |

    - For example

For example: set baud rate of reader to be 9600bps, command packet is『40H  03H  01H  04H  B8H』

Thereinto :

| 40H | Boot code |
| --- | --- |
| 03H | Effective length is 3 bytes |
| 01H | Command code of『SetBaudRate』 |
| 04H | On behalf of 9600bps |
| B8H | Checksum |

Checksum is the patch code of 40H+03H+01H+04H=48H

When execution correct, return packet is: 『F0H  02H  01H  0DH』

When execution fail, return packet is: 『F4H  03H  01H  1FH  E9H』

    - Checksum

Following C language Check Sum calculation program is for reference:

unsigned char CheckSum(unsigned char *uBuff, unsigned char uBuffLen)

{

unsigned char i,uSum=0;

for(i=0;i<uBuffLen;i++)

{

uSum = uSum + uBuff[i];

}

uSum = (~uSum) + 1;

return uSum;

}

  - Control command format of serial

    - Set BaudRate

Function: to set operation baud rate for RS232 interface.

The original baud rate of RS232 interface is 9600bps, after every new program download for reader. When reader receives the command, it will reset baud rate of serial. No matter power closed or not, the baud rate will keep same to next reset.

Command code: 01H

Command parameter: 1byte BPS, value: 00H~08H，respectively on behalf of:

| 04H | 9600bps |
| --- | --- |
| 05H | 19200bps |
| 06H | 38400bps |
| 07H | 57600bps |
| 08H | 115200bps |

Command packet: 『40H  03H  01H  BPS  CheckSum』

Return data: if command execution correct, return data is null.

『F0H  02H  01H  0DH』

Command format with reader address:

Command code:  01H

Parameter of reader address:address

Command parameter: 1byte BPS, value:00H~08H, respectively on behalf of:

| 04H | 9600bps |
| --- | --- |
| 05H | 19200bps |
| 06H | 38400bps |
| 07H | 57600bps |
| 08H | 115200bps |

Command packet with address: 『40H  04H  01H  address  BPS  CheckSum』

Return data: if command execution correct, return data is null.

Return packet with address:『F0H  03H  01H  address  CheckSum』

    - Get Reader Version

Function: to get version number of hardware and software from reader

Command code: 02H

Command parameter: none

Command packet: 『40H  02H  02H  BCH』

Return data: if command execution correct, return data is 4 bytes of version number:

| Byte0 | Major version of hardware |
| --- | --- |
| Byte1 | Minor version of hardware(hardware version number is to show reader model number) |
| Byte2 | Major version of software |
| Byte3 | Minor version of software |

For example: if model number of reader is Reader1102, software version number is V1.5, then return packet is:

『F0H  06H  02H  0BH  02H  01H  05H  DDH』

Command format with reader address:

Command code: 02H

Parameter of reader address: address

Command parameter: none

Command packet: 『40H  03H  02H  address  CheckSum』

Return data: if command execution correct, return data is 4 bytes of version number:

| Byte0 | Major version of hardware |
| --- | --- |
| Byte1 | Minor version of hardware(hardware version number is to show reader model number) |
| Byte2 | Major version of software |
| Byte3 | Minor version of software |

For example: if model number of reader is Reader1102, software version number is V1.5, then return packet is:

『F0H  07H  02H  address  0BH  02H  01H  05H  CheckSum』

    - Set Relay

Function: to set relay status for reader

Command code: 03H

Command parameter: 1 byte

| Bit0=1 | No1 relay on |
| --- | --- |
| Bit0=0 | No1 relay off |
| Bit1=1 | No2 relay on |
| Bit1=0 | No2 relay off |

And so on.

Command packet:  『40H  03H  03H  K  CheckSum』

Return data: if success, return data is null.

『F0H  02H  03H  0BH』

Command format with reader address:

Command code: 03H

Parameter of reader address: address

Command parameter: 1byte

| Bit0=1 | No1 relay on |
| --- | --- |
| Bit0=0 | No1 relay off |
| Bit1=1 | No2 relay on |
| Bit1=0 | No2 relay off |

And so on.

Command packet:  『40H  04H  03H  address  K  CheckSum』

Return data: if success, return data is null.

『F0H  03H  03H  address  CheckSum』

    - Get Relay

Function: to Get relay status for reader

Command code: 0BH

Command packet:  『40H  03H  0BH  CheckSum』

Return data: if success, return data is 1 byte

『F0H  02H  0BH  K  CheckSum』

| Bit0=1 | No1 relay on |
| --- | --- |
| Bit0=0 | No1 relay off |
| Bit1=1 | No2 relay on |
| Bit1=0 | No2 relay off |

Command format with reader address:

Command code: 0BH

Parameter of reader address: address

Command packet:  『40H  04H  0BH  address CheckSum』

Return data: if success, return data is K（1byte）

『F0H  03H  03H  address K CheckSum』

| Bit0=1 | No1 relay on |
| --- | --- |
| Bit0=0 | No1 relay off |
| Bit1=1 | No2 relay on |
| Bit1=0 | No2 relay off |

    - Set Output Power

Function: to set the output power coefficient of reader. It takes effect immediately when setting new output power coefficient for reader, and will keep same till reset, no matter power supply is off or not.

Command code:  04H

Command parameter: 1 byte of P expresses power value 0~59.

Command packet: 『40H  03H  04H  P  CheckSum』

Return data: if command execution correct, return data is null.

『F0H  02H  04H  0AH』

Command format with reader address:

Command code:  04H

Parameter of reader address: address

Command parameter: 1 byte of P expresses power value 20~33.

Command packet: 『40H  04H  04H  address  P  CheckSum』

Return data: if command execution correct, return data is null.

『F0H  03H  04H  address  CheckSum』

    - Set Frequency

Function: to set the frequency channel number for reader to transmit microwave signal. It takes effect immediately when operation frequency is set ready, and will keep same till reset, no matter power supply is off or not.

Command code: 05H

Command parameter: 2 bytes, byte1 expresses start frequency fmin, value 1~59; byte2 expresses end frequency fmax, value 0~59. If fmax>fmin, reader works by FHSS, range is fmin～fmax. If fmax=fmin, reader works in fixed frequency, frequency is fmax. Note: it’s linked with countries’ frequency.

Command packet: 『40H  04H  05H  fmin   fmax  CheckSum』

Return data: if command execution correct, return data is null.

『F0H  02H  05H  09H』

Command format with reader address:

Command code: 05H

Parameter of reader address:address

Command parameter: 2 bytes, byte1 expresses start frequency fmin, value 1~63; byte2 expresses end frequency fmax, value 1~63. If fmax>fmin, reader works by FHSS, range is fmin～fmax. If fmax=fmin, reader works in fixed frequency, frequency is fmax.

Command packet: 『40H  05H  05H  address  fmin   fmax  CheckSum』

Return data: if command execution correct, return data is null.

『F0H  03H  05H  address  CheckSum』

Chart. Relation between requency channel and transmitting frequency

| Channel number | Frequency inMHz | Channel number | Frequency inMHz | Channel number | Frequency inMHz |
| --- | --- | --- | --- | --- | --- |
| 0 | 865.00 | 20 | 908.50 | 40 | 918.50 |
| 1 | 865.50 | 21 | 909.00 | 41 | 919.00 |
| 2 | 866.00 | 22 | 909.50 | 42 | 919.50 |
| 3 | 866.50 | 23 | 910.00 | 43 | 920.00 |
| 4 | 867.00 | 24 | 910.50 | 44 | 920.50 |
| 5 | 867.50 | 25 | 911.00 | 45 | 921.00 |
| 6 | 868.00 | 26 | 911.50 | 46 | 921.50 |
| 7 | 902.00 | 27 | 912.00 | 47 | 922.00 |
| 8 | 902.50 | 28 | 912.50 | 48 | 922.50 |
| 9 | 903.00 | 29 | 913.00 | 49 | 923.00 |
| 10 | 903.50 | 30 | 913.50 | 50 | 923.50 |
| 11 | 904.00 | 31 | 914.00 | 51 | 924.00 |
| 12 | 904.50 | 32 | 914.50 | 52 | 924.50 |
| 13 | 905.00 | 33 | 915.00 | 53 | 925.00 |
| 14 | 905.50 | 34 | 915.50 | 54 | 925.50 |
| 15 | 906.00 | 35 | 916.00 | 55 | 926.00 |
| 16 | 906.50 | 36 | 916.50 | 56 | 926.50 |
| 17 | 907.00 | 37 | 917.00 | 57 | 927.00 |
| 18 | 907.50 | 38 | 917.50 | 58 | 927.50 |
| 19 | 908.00 | 39 | 918.00 | 59 | 928.00 |

    - Read Param

Function: to read the operation parameter in reader written by last command.

Command code: 06H

Command parameter: none

Command packet: 『40H  02H  06H  B8H』

Return data: if command execution correct, return data is 32 bytes PAM from setting command.

『F0H  22H  06H  PAM  CheckSum』

Command format with reader address:

Command code: 06H

Parameter of reader address: address

Command parameter: none

Command packet: 『40H  03H  06H  address  CheckSum』

Return data: if command execution correct, return data is 32 bytes PAM from setting command.

『F0H  23H  06H  address  PAM  CheckSum』

    - Set Param

Function: to set basic parameter of reader, baud rate of serial, transmit power, RF power output and so on.

Command code: 09H

Command parameter: 32 bytes PAM

Command packet: 『40H  22H  09H  PAM  CheckSum』

Return data: if command execution correct, return data is null. If success, return data of ReadParam is 32 bytes parameters with following sequence.

『F0H  02H  09H  05H』

Command format with reader address:

Command code: 09H

Parameter of reader address: address

Command parameter: 32 bytes PAM

Command packet: 『40H  23H  09H  address  PAM  CheckSum』

Return data: if command execution correct, return data is null. If success, return data of ReadParam is 32 bytes parameters with following sequence.

『F0H  03H  09H  address  CheckSum』

32 bytes of parameter(1 parameter 1 byte):

| Byte number | Definition |
| --- | --- |
| 1 | Baud rate of serial, value: 00H~08H， |
| 2 | Transmit power, value: 20～33dbm. |
| 3 | Start point of transmitting microwave signal frequency, value(default 7): 0~59 |
| 4 | End point of transmitting microwave signal frequency, value(default 59): 0~59 |
| 5 | Null |
| 6 | Word mode of reader: 0-Auto，1-Command |
| 7 | RS485 address of reader: 0 and 255 is broadcast address |
| 8 | Max tags of once reading |
| 9 | Tag type: 01H－ISO18000-6B, 02H－EPCC1, 04H－EPCC1G2 |
| 10 | Tag reading duration time: RF emission duration time, only valid for EM tag. 0－10ms, 1－20ms, 2－30ms, 3－40ms. |
| 11 | Reading times M：reader will execute the command for M times when receiving reading command from host pc. |
| 12 | 1:enable buzzer 0: disable buzzer |
| 13 | Reader IP address1 |
| 14 | Reader IP address2 |
| 15 | Reader IP address3 |
| 16 | Reader IP address4 |
| 17 | Reader port –high |
| 18 | Reader port –low |
| 19 | Reader mask1 |
| 20 | Reader mask2 |
| 21 | Reader mask3 |
| 22 | Reader mask4 |
| 23 | Reader address gateway1 |
| 24 | Reader address gateway2 |
| 25 | Reader address gateway3 |
| 26 | Reader address gateway4 |
| 27 | Reader MAC1 |
| 28 | Reader MAC2 |
| 29 | Reader MAC3 |
| 30 | Reader MAC4 |
| 31 | Reader MAC5 |
| 32 | Reader MAC6 |

    - Read Auto Param

Function: to read the auto work parameter in reader written by last command.

Command code: 14H

Command parameter: none

Command packet: 『40H  02H  14H  6DH』

Return data: if success, return data is 32 bytes PAM from setting command.

『F0H  24H  14H  PAR  Check』

Command format with reader address:

Command code: 14H

Parameter of reader address: address

Command parameter: none

Command packet: 『40H  03H  14H  Address Check』

Return data: if success, return data is 32 bytes PAM from setting command.

『F0H  24H  14H  Address  PAR  Check』

    - Set Auto Param

Function: to set basic parameter for reader in auto mode, baud rate of serial, transmit power, RF power output and so on.

Command code: 13H

Command parameter: 32 bytes PAM

Command packet: 『40H  26H  13H  PAM  Check』

Return data: if command execution correct, return data is null. If success, return data of Read Auto Param is 32 bytes parameters with following sequence.

『F0H  02H  13H  FBH』

Command format with reader address:

Command code: 13H

Parameter of reader address: address

Command parameter: 32 bytes PAM

Command packet: 『40H  27H  13H  Address  PAM   Check』

Return data: if command execution correct, return data is null. If success, return data of ReadParam is 32 bytes parameters with following sequence.

『F0H  03H  13H  Address  Check』

32bytes parameters (1 parameter 1 byte):

| Byte number | Definition |
| --- | --- |
| 1 | Tag reading mode：0-timing，1-trigger |
| 2 | Tag storage time：unit：s. default 1. High. |
| 3 | Tag storage time：unit：s. default 1. Low. |
| 4 | 0-10ms, 1-20ms, 2-30ms, 3-50ms, 4-100ms. Default 2. Atumatically read tag once at intervals. |
| 5 | Tag storage quantity：default 1. The quantity of read tag ID stored in reader memory. High. |
| 6 | Tag storage quantity：default 1. The quantity of read tag ID stored in reader memory. Low. |
| 7 | Data output format：0-terse，1-standard，2-XML format. Default0. |
| 8 | Output interface: 0-RS232, 1-RS485, 2-RJ45. Default 0. 3- Wiegand26, 4- Wiegand34. |
| 9 | Wiegand output pulse width, default40. |
| 10 | Wiegand output pulse interval, default200. |
| 11 | Set start bit of output tag ID, value 0～4. Default 0. (Wiegand) |
| 12 | Set storage address of ID in tag(default0): 0-tag ID number 1-usder defined number(Wiegand) |
| 13 | Notify interval：unit：s. Default 1. Automatically notify host pc once at intervals. Default 120s, 1～255. |
| 14 | Notify condition：default1. 0-notify now，1-timing，2-add，3-remove，4-change |
| 15 | Output port of notify (if no reading for long time, send EPC number with length 0 to output port) 0—no use 1—use time depends on ”notify interval”. |
| 16 | Select antenna. 1-ant1,2-ant2,4-ant3,8-ant4 |
| 17 | Set trigger mode(default0): 0-low level 1-high level |
| 18 | Host PC IP address1 |
| 19 | Host PC IP address2 |
| 20 | Host PC IP address3 |
| 21 | Host PC IP address4 |
| 22 | Host PC port -high |
| 23 | Host PC port -low |
| 24 | Null |
| 25 | Null |
| 26 | Null |
| 27 | Null |
| 28 | Null |
| 29 | Null |
| 30 | 0-no alarm，1-alarm. Detect alarm or not in timing and trigger mode. |
| 31 | null |
| 32 | Control relay or not in Auto state. 0- no control 1- control |

    - Select Antenna

Function: to select from which antenna to transmit-receive signal.

Command code: 0AH

Command parameter: 1 byte of selected antenna No.

| 1 | Select No.1 antenna |
| --- | --- |
| 2 | Select No.2 antenna |
| 4 | Select No.3 antenna |
| 8 | Select No.4 antenna |

Command packet: 『40H  03H  0AH  No.  CheckSum』

Return data: if command execution correct, return data is null.

『F0H  02H  0AH  04H』

Command format with reader address:

Command code: 0AH

Parameter of reader address: address

Command parameter: 1 byte of selected antenna No

| 1 | Select No.1 antenna |
| --- | --- |
| 2 | Select No.2 antenna |
| 4 | Select No.3 antenna |
| 8 | Select No.4 antenna |

Command packet: 『40H  04H  0AH  address  No.  CheckSum』

Return data: if command execution correct, return data is null.

『F0H  03H  0AH  address  CheckSum』

    - Restore factory settings

Function: The parameters of the Reader device back to factory

Command code: 0EH

Command parameter: none

Command packet: 『40H  02H  0DH  Check』

Return data: if success, return data is null.

『F0H  02H  0DH Check』

    - Reboot

Function: reboot reader, which equivalent to power on again after power off.

Command code: 0EH

Command parameter: none

Command packet: 『40H  02H  0EH  B0H』

Return data: if success, return data is null.

『F0H  02H  0EH  00H』

Command format with reader address:

Command code: 0EH

Parameter of reader address: address

Command parameter: none

Command packet: 『40H  03H  0EH  address  CheckSum』

Return data: if success, return data is null.

『F0H  03H  0EH  address  CheckSum』

    - Start/ stop automode

Function: Set the operation mode of the Reader.

Command code: 0FH

Command parameter: 1 byte, automode is the working mode of Reader.

Command packet: 『40H  03H  0FH  Automode  Check』

Automode is 0, mean stop automode

Automode is 1, mean start automode

Return data: if success, return data is null.

『F0H  02H  0FH  00H』

    - Clear Memory

Function: Remove the cache of tag data in Reader.

Command code: 10H

Command parameter: Null

Command packet: 『40H  02H  10H  AEH』

Return data: if success, return data is null.

『F0H  02H  10H  FEH』

    - Set Reader Time

Function: to set time for reader.

Command code: 11H

Command parameter: 6bytes:yy/mm/dd/hour/minute/second

Command packet:  『40H  08H  11H  yy  mm  dd  hh  ff  ss  CheckSum』

Return data: if success, return data is null.

『F0H  02H  11H  FDH』

Command format with reader address:

Command code: 11H

Parameter of reader address:address

Command parameter: none

Command packet: 『40H  09H  11H  address  yy  mm  dd  hh  ff  ss  CheckSum』

Return data: if success, return data is null.

『F0H  03H  11H  address  CheckSum』

    - Get Reader Time

Function: to get time from reader.

Command code: 12H

Command parameter: none

Command packet:  『40H  02H  12H  ACH』

Return data: if success, return data is yy/mm/dd/hour/minute/second.

『F0H  08H  12H   yy  mm  dd  hh  ff  ss  CheckSum』

Command format with reader address:

Command code: 12H

Parameter of reader address: address

Command parameter: none

Command packet:  『40H  03H  12H  address  CheckSum』

Return data: if success, return data is yy/mm/dd/hour/minute/second.

『F0H  09H  12H   address  yy  mm  dd  hh  ff  ss  CheckSum』

    - Set Report Filter

Function: read tag chosen from setted filter can be put into output list.

Command code: 15H

Command parameter1: 2 bytes of mask address ADDR, high digit before,

low digit after. That’s bit address.

Command parameter2: 2 bytes of mask length LEN, high digit before, low digit after. That’s bit length.

Command parameter3: M bytes of data, high digit before, low digit after. If

LEN/8 is integer, then M=LEN/8；if LEN/8 is not integer, then M=LEN/8＋1, lastbyte data in high-order-position, low-order filling zero.

Command packet: 『40H 4+M 15H ADDR LEN M CheckSum』

ReturnData: if success, return data will be null.

『F0H 02H 15H F9H』

Note:

1. When LEN＝0, filter unavailable, without command parameter3.

Command packet:『40H 04H 15H 00 A7H』

2. Filter object:

| ISO18000-6B | 64 bits ID |
| --- | --- |
| EPCC1 | EPC No. |
| ISO18000-6C | EPC No. |
| EM Tag | 64 bits ID |

    - Get Report Filter

Function: to get parameter of filter.

Command code: 16H

Command parameter: No

Command packet:『40H 02H 16H A8H』

ReturnData: if success, return data will be (2 bytes of mask ADDR＋2 bytes of mask LEN＋M bytes of data). If LEN/8 is integer, then M=LEN/8；if LEN/8 is not integer, then M=LEN/8＋1, last byte data in high-order-position, low-order filling zero.

『F0H 4+M 16H ADDR LEN M CheckSum』

    - Set Reader Network Address

Function: to set network address for reader.

Command code: 30H

Command parameter: 14bytes parameter. (IP(4Bytes)+PORT(2Bytes)+MASK(4Bytes)+Gateway(4Bytes)).

Command packet:  『40H  10H  30H  IP  PORT  MASK  Gateway  CheckSum』

Return data: if success, return data is null.

『F0H  02H  30H  DEH』

    - Get Reader Network Address

Function: to get network address from reader.

Command code: 31H

Command parameter: none

Command packet:  『40H  02H  31H  8DH』

Return data: if success, return data is (IP(4Bytes)+PORT(2Bytes)+MASK(4Bytes)+Gateway (4Bytes)).

『F0H  10H  31H  IP  PORT  MASK  Gateway  CheckSum』

    - SetReader MAC

Function: to set network MAC for reader.

Command code: 32H

Command parameter: 6bytes parameter MAC

Command packet:  『40H  08H  32H  MAC  CheckSum』

Return data: if success, return data is null.

『F0H  02H  32H  DBH』

    - Get Reader MAC

Function: to get network MAC from reader.

Command code: 33H

Command parameter: none

Command packet:  『40H  02H  33H  8B』

Return data: if success, return data is 6bytes MAC.

『F0H  08H  33H  MAC CheckSum』

    - Report Now

Function: to send all reserved tag information to PC when reader receive the

command.

Command code: 54H

Command parameter: no

Command packet: 『40H 02H 54H 6AH』

ReturnData: if success, return data will be null.

『F0H 02H 54H BAH』

Following is some data packets. Each data packet with 1 tag information and

each tag information is fixed as 120 bytes. Format of Data packet in appendix A.

    - Get Tag Info.

Function: reader will send the Tag information reader memory stored to PC immedicately after receiving this command.

Command code: 57H

Command parameter1:2 bytes of initial record address ADDR,higher bit first,lower bit last.

Command parameter2；1byte of record size LEN(<=5)

Command packet: 『40H  05H  57H  ADDR  LEN  CheckSum』

ReturnData: if success, return packet definition as following:

『F0H 4+N*(17+L) 57H N N *RECORD CheckSum』

N(<=5) is the record number reader return. Record formate is HEX: 6bytes ofstart time＋6bytes of end time＋2bytes of reading times＋1byte of antenna No.＋1byte of tag type＋1byte of EPC Length＋L byte of EPC. Records range fromsmall to large according to EPC value.

If record parameter appointed is not exist, return packet definition as

following:

『F0H 04H 57H 00H 00H B5H』

    - GetReaderID

Function: to get reader ID No.

Command code: 8CH

Command parameter: Null

Command packet:	『40H 02H 8CH CheckSum』

ReturnData: if success, return data will be 6 bytes of ID.

『F0H 0AH 8CH ID CheckSum』

  - Command format of serial read-write tag

    - Read-write ISO18000-6B command format

For electronic tag, internal storage capacity is 2048bits, which is devided into 256bytes. An address is for each byte, corresponding to 0～255.

Thereinto:

- Address 0～7, 8 words(64bits): tag ID number. Fixed before product leaves factory, can’t be modified.

- Address 8～223: user information storage area, can be distributed at discretion.

- Address 224～255: write protection byte.

      - List Tag ID

Function: to list readable tag ID in antenna radiation field.

Command code: FEH

Command parameter: none

Command packet:  『40H  02H  FEH  C0H』

Return data: if success, byte count of return data = number of all listed tags M(1byte)+ M*data of (tag length sent out L(<=8，1byte)+ 8(ID)).

『F0H  3+M*（L+8）FEH  M  M*（L+8）CheckSum』

Command format with reader address:

Command code: FEH

Parameter of reader address:address

Command parameter: none

Command packet:  『40H  03H  FEH  address  CheckSum』

Return data: if success, byte count of return data = number of all listed tags M(1byte)+M* data of (tag Length sent out L(<=8，1byte)*8(ID)).

『F0H  4+M*（L+8）  FEH  address  M  M*(L+8)  CheckSum』

      - Get ID List

Function: to get listed tag ID by rfs_ListID command from reader memory.

Command code:FDH

Command parameter: 2bytes, the first byte is start number ADDR, the second byte is tag count M (<=8,1byte).

Command packet:  『40H  04H  FDH  ADDR  M   CheckSum』

Return data: if success, byte count of return data=M+M*data of (tag length L*8(ID)).

『F0H  2+M*（L+8）  FDH  M  M*( L+8)  CheckSum』

Command format with reader address:

Command code: FDH

Parameter of reader address: address

Command parameter: 2bytes, the first byte is start number ADDR, the second byte is tag count M(<=8).

Command packet:  『40H  05H  FDH  address  ADDR  M  CheckSum』

Return data: if success, byte count of return data=M+M*data of (tag length L*8(ID)).

『F0H  3+M*(L+8)  FDH  address M  M*( L+8)  CheckSum』

      - List Selected ID

Function: to list readable tag ID in antenna radiation field based on following parameter condition.

Command code: FBH

Command parameter1: 1byte is the condition of selected tag.

| 00 | Equal to |
| --- | --- |
| 01 | Unequal to |
| 02 | Greater than |
| 03 | Less than |

Command paramter2: 1byte is start address of tag data ADDR, value 0～223.

Command parameter3: 1byte is data MASK. Each bit in the byte is corresponding to a comparative byte.

| 0 | The byte not for comparison |
| --- | --- |
| 1 | The byte for comparison |

Command parameter4: 8bytes is data for comparison.

Command packet:  『40H  0DH  FBH  SEL  ADDR  MASK  DATA  CheckSum』

Return data: if success, byte count of return data= number of all listed tags M(1byte)+ M *data of (tag length L (1byte)*8(ID)).

『F0H  3+M*(L+8)  FBH  M  M*（L+8）CheckSum』

Command format with reader address:

Command code:FBH

Parameter of reader address:address

Command parameter1: 1byte is the condition of selected tag.

| 00 | Equal to |
| --- | --- |
| 01 | Unequal to |
| 02 | Greater than |
| 03 | Less than |

Command paramter2: 1byte is start address of tag data ADDR, value 0～223.

Command parameter3: 1byte is data MASK. Each bit in the byte is corresponding to a comparative byte.

| 0 | The byte not for comparison |
| --- | --- |
| 1 | The byte for comparison |

Command parameter4: 8bytes is data for comparison.

Command packet:  『40H  0EH  FBH  address  SEL  ADDR  MASK  DATA  CheckSum』

Return data: if success, byte count of return data= number of all listed tags M(1byte)+ M *data of (tag length L (1byte)*8(ID)).

『F0H  4+M*(L+8)  FBH  address  M  M*(L+8)  CheckSum』

      - Read Byte Block

Function: to read a block of data starting from appointed address of appointed tag. Memory capacity of ISO18000-6B tag is 2048bits(256bytes). Readable byte address for user is 0～223. Length of data block takes byte as unit. Stipulates 32bytes can be read at most each time.

Command code:  F6H

Command parameter: 8bytes ID(subject to the sequence of tag ID number); 1byte start address(aa), value 0~223; 1byte length of block(nn), value 1~32.

Command packet:  『40H  0CH  F6H  id  aa  nn  CheckSum』

Return packet: if success, return data is nn bytes of data.

『F0H  nn+2  F6H  xx  …… xx  CheckSum』

Command format with reader address:

Command code:  F6H

Parameter of reader address:address

Command parameter: 8 bytes ID(subject to the sequence of tag ID number); 1byte start address(aa), value 0~223; 1byte length of block(nn), value 1~32.

Command packet: 『40H  0DH  F6H  address  id  aa  nn  CheckSum』

Return packet: if success, return data is nn bytes of data.

『F0H  nn+3  F6H  address  xx  …… xx  CheckSum』

      - Write Byte Block

Function: to write data into the appointed address unit of appointed tag. Data length to write takes byte as unit, and 4bytes can be written in at most once. Writable byte address for user is 8～223.

Command code: F5H

Command parameter: 8 bytes ID(subject to the sequence of tag ID number); 1byte start address(aa), value 8~223; 1byte length of block(nn), value 1~4; nn bytes written data.

Command packet: 『40H  12+nn  F5H  id  aa  nn  xx  ---  xx  CheckSum』

Return packet: if success, return data is null.

『F0H  02H  F5H  19H』

Command format with reader address:

Command code:  F5H

Parameter of reader address: address

Command parameter: 8 bytes ID(subject to the sequence of tag ID number); 1byte start address(aa), value 8~223; 1byte length of block(nn), value 1~4; nn bytes written data.

Command packet:  『40H  13+nn  F5H  address  id  aa  nn  xx  ---  xx  CheckSum』

Return packet: if success, return data is null.

『F0H  03H  F5H  address  CheckSum』

      - Set Write Protect

Function: to set write protection for the appointed addess unit of appointed tag.

Command code:F4H

Command parameter: 8 bytes ID(subject to the sequence of tag ID number); 1byte start address(aa), value 8~223.

Command packet: 『40H  0BH  F4H  ID  aa  CheckSum』

Return data: if success, boot code of return packet is F0H, data part is null.

『F0H  02H  F4H  1AH』

Command format with reader address:

Command code: F4H

Parameter of reader address: address

Command parameter:	8 bytes ID(subject to the sequence of tag ID number); 1byte start address(aa), value 8~223.

Command packet:  『40H  0CH  F4H  address  ID  aa  CheckSum』

Return data: if success, boot code of return packet is F0H, data part is null.

『F0H  03H  F4H  address  CheckSum』

      - Read Write Protect

Function: to read the appointed address unit of appointed tag if with write protect or not.

Command code: F3H

Command parameter: 8 bytes ID(subject to the sequence of tag ID number); 1byte start address(aa), value 0~223.

Command packet: 『40H  0BH  F3H  ID  aa  CheckSum』

Return data: if success, boot code of return packet is F0H, data part is 1 byte.

| 0 | Without protection, 『F0H 03H F3H 00H 1AH』 |
| --- | --- |
| 1 | With protection, 『F0H 03H F3H 01H 19H』 |

Command format with reader address:

Command code:F3H

Parameter of reader address:address

Command parameter: 8 bytes ID(subject to the sequence of tag ID number) ; 1byte start address(aa), value 0~223.

Command packet:  『40H  0CH  F3H  address  ID  aa  CheckSum』

Return data: if success, boot code of return packet is F0H, data part is 1 byte.

| 0 | Without protection, 『F0H 04H F3H address 00H CheckSum』 |
| --- | --- |
| 1 | With protection, 『F0H 04H F3H address 01H CheckSum』 |

      - Slow Write A Byte

Function: to write data into the appointed address unit of appointed tag. Length of written data takes byte as unit, and 4bytes can be written in at most once. Writable byte address for user is 8～223.

Command code:  F2H

Command parameter: 8 bytes ID(subject to the sequence of tag ID number); 1byte start address(aa), value 8~223; 1byte length of block(nn), value 1~4; nn bytes written data.

Command packet:  『40H  12+nn  F2H  id  aa  nn  xx  ---  xx  CheckSum』

Return packet: if success, return data is null.

『F0H  02H  F2H  1CH』

Note: the command writes data into tag byte by byte, slow in speed. Only used for tags not support previous write command.

Command format with reader address:

Command code: F2H

Parameter with reader address: address

Command parameter: 8 bytes ID(subject to the sequence of tag ID number); 1byte start address(aa), value 8~223; 1byte length of block(nn), value 1~4; nn bytes written data.

Command packet:  『40H  13+nn  F2H  address  id  aa  nn  xx  ---  xx  CheckSum』

Return data: if success, return data is null.

『F0H  03H  F2H  address  CheckSum』

    - Read write ISO18000-6C command format

Storage unit of ISO18000-6C is devided into 4 memory banks:

- EPC memory bank: to store EPC number. It can store 96bits EPC number at most presently. Readable and writable.

- TID memory bank: to store ID number set by tag manufacturer. There are 32bits and 64bits ID presently. Readable but non-writable.

- User memory bank: different from various manufacturer.

- Password memory bank: with 32Bits access password and 32Bits kill password. Readable and writable.

      - List Tag ID

Function: to identify the readable tag ID in antenna radiation field based on MASK condtion.

Command code: EEH

Command parameter1: 1byte mem, select memory bank.

| 0 | Password |
| --- | --- |
| 1 | EPC |
| 2 | TID |
| 3 | User |

Command parameter2: 2bytes, start address of MASK (unit: bit).

Command parameter3: 1byte, the length of MASK (unit: bit).

Command parameter4: m bytes, Mask. If LEN％8=0, then m=LEN/8. If LEN％80, then m=LEN/8+1.

Command packet:  『40H  m+6  EEH  mem  addr  LEN  Mask  CheckSum』

Return data: if success, byte count of return data= number of all listed tags M(1byte)+ data of (tag number sent out L(<=8)*L(digit count of EPC+EPC)).

Note: LEN=0, to identify all readable tag ID in antenna radiation field.

Digit count of EPC:00H-0Word，01H-1Word，02H-2Word，……，FFH-256Word

『F0H  3+L*N  EEH  M  L*N  CheckSum』

Command format with reader address:

Command code: EEH

Parameter of reader address: address

Command parameter1: 1byte mem, select memory bank.

| 0 | Password |
| --- | --- |
| 1 | EPC |
| 2 | TID |
| 3 | User |

Command parameter2: 2bytes addr, start address of MASK (unit: bit).

Command parameter3: 1byte LEN, the length of MASK (unit: bit).

Command parameter4: m bytes, Mask. If LEN％8=0, then m=LEN/8. If LEN％80, then m=LEN/8+1.

Command packet:『40H  m+7  EEH  address  mem  addr  LEN  Mask  CheckSum』

Return data: if success, byte count of return data= number of all listed tags M(1byte)+ data of (tag number sent out L(<=8)*L(digit count of EPC+EPC)).

Note: LEN=0, to identify all readable tag ID in antenna radiation field.

Digit count of EPC:00H-0Word，01H-1Word，02H-2Word，……，FFH-256Word

『F0H  4+L*N  EEH  address  M  L*N  CheckSum』

      - Get ID List

Function: to get listed tag ID by rfs_ListID command from reader memory.

Command code:EDH

Command parameter: 2bytes, the first byte is start number NO, the second byte is tag count L(<=8).

Command packet:  『40H  04H  EDH  no  m  CheckSum』

Return data: if success, byte count of return data=data of (1byte tag count M*L Bytes(digit count of EPC+EPC)).

『F0H  2+L*8  EDH  L*M  CheckSum』

Command format with reader address:

Command code:EDH

Parameter of reader address:address

Command parameter:2bytes, the first byte is start number NO, the second byte is tag count L(<=8).

Command packet:  『40H  05H  EDH  address  no  m  CheckSum』

Return data: if success, byte count of return data=data of (1byte tag count M*L Bytes(digit count of EPC+EPC)).

『F0H  3+L*8  EDH  address  L*M  CheckSum』

      - Read Word Block

Function: to read a block of data starting from appointed address of appointed tag. Length of data block takes word(16bits) as unit.

Command code:  ECH

Command parameter1: 1byte L, digit count of EPC, to show the word count of EPC number.

Command parameter2: L*2bytes EPC number, to show which tag data to read.

Command parameter3: 1byte mem, select memory bank.

| 0 | Password |
| --- | --- |
| 1 | EPC |
| 2 | TID |
| 3 | User |

Command parameter4: 1byte addr, start address(unit:Word).

Command parameter5: 1byte len, length of data(unit:Word).

Command parameter6: 4bytes, AccessPassword.

Command packet:  『40H  10+L*2  ECH  L  EPC  mem  addr  len  AccessPassword  CheckSum』

Return packet: if success, return data is len*2bytes data xx.

『F0H  len*2+2  ECH  xx  …… xx  CheckSum』

Note: AccessPassword is workable when password memory bank is locked.

Command format with reader address:

Command code: ECH

Parameter of reader address:address

Command parameter1: 1byte L, digit count of EPC, to show the word count of EPC number.

Command parameter2: L*2bytes EPC number, to show which tag data to read.

Command parameter3: 1byte mem, select memory bank.

| 0 | Password |
| --- | --- |
| 1 | EPC |
| 2 | TID |
| 3 | User |

Command parameter4: 1byte addr, start address(unit:Word).

Command parameter5: 1byte len, length of data(unit:Word).

Command parameter6: 4bytes, AccessPassword.

Command packet:  『40H  11+L*2  ECH  address  L  EPC  mem  addr  len  AccessPassword  CheckSum』

Return packet: if success, return data is len*2bytes data xx.

『F0H  len*2+3  ECH  address  xx  …… xx  CheckSum』

      - Write Word Block

Function: write data into the appointed address unit in appointed memory bank of appointed tag. Length of written data takes word as unit.

Command code:  EBH

Command parameter1: 1byte L, digit count of EPC, to show the word count of EPC number.

Command parameter2: L*2bytes EPC number, to show which tag data to write.

Command parameter3: 1byte mem, select memory bank.

| 0 | Password |
| --- | --- |
| 1 | EPC |
| 2 | TID |
| 3 | User |

Command parameter4: 1byte addr, start address(unit:Word).

Command parameter5: 1byte len, length of data(unit:Word).

Command parameter6: len*2bytes written data.

Command parameter7: 4bytes, AccessPassword.

Command packet:  『40H  10+L*2+len*2  EBH  L  EPC  mem  addr  len  data  AccessPassword  CheckSum』

Return packet: if success, return data is null.

『F0H  02H  EBH  23H』

Note: AccessPassword is workable only when memory bank is locked. When unlocked, writable without password. When permanently locked, password is useless.

Command format with reader address:

Command code:  EBH

Parameter of reader address:address

Command parameter1: 1byte L, digit count of EPC, to show the word count of EPC number.

Command parameter2: L*2bytes EPC number, to show which tag data to write.

Command parameter3: 1byte mem, select memory bank.

| 0 | Password |
| --- | --- |
| 1 | EPC |
| 2 | TID |
| 3 | User |

Command parameter4: 1byte addr, start address(unit:Word).

Command parameter5: 1byte len, length of data(unit:Word).

Command parameter6: len*2bytes written data.

Command parameter7: 4bytes, AccessPassword.

Command packet:  『40H  11+L*2+len*2  EBH  address  L  EPC  mem  addr  len  data  AccessPassword  CheckSum』

Return packet: if success, return data is null.

『F0H  03H  EBH  address  CheckSum』

      - Kill Tag

Function: Permanentkill tag

Command code: E8H

Command parameter1: 1byte L, digit count of EPC, to show the word count of EPC number.

Command parameter2: L*2bytes EPC number, to show which tag to set read-write protect.

Command parameter3: kill password , 4bytes mem

Command packet:『40H  L*2+7  E8H  L  EPC  KillPassword  Check』

Command packet with reader address:『40H  L*2+8  E8H  Address L  EPC  KillPassword  Check』

| Head | Len | Cmd | Address | Data | Check |
| --- | --- | --- | --- | --- | --- |
| 40H | L*2+7/L*2+8 | E8H |  | L EPC KillPassword | 1Byte |

Specific command package parameters definition to see the table below:

| L | 1byte，digit count of EPC (Unit:Word) |
| --- | --- |
| EPC | L*2 bytes EPC |
| KillPassword | 4bytes |

ReturnData: if success, return data will be null.

『F0H  02H  E8H  Check』

      - Set Lock

Function: to set write protect for the appointed memory bank of appointed tag.

Command code:EAH

Command parameter1: 1byte L, digit count of EPC, to show the word count of EPC number.

Command parameter2: L*2bytes EPC number, to show which tag to set read-write protect.

Command parameter3: 1byte mem, select memory bank.

| 0 | Kill Password |
| --- | --- |
| 1 | Access Password |
| 2 | EPC number |
| 3 | ID number in TID |
| 4 | User |

Command parameter4: 1byte Lock, control word.

| 0 | Writable |
| --- | --- |
| 1 | Writable permanently |
| 2 | Writable in secured state |
| 3 | Never writable |
| 4 | Readable and writable |
| 5 | Readable and writable permanently |
| 6 | Readable and writable in secured state |
| 7 | Never readable and writable |

0～3 only apply to EPC, TID and User memory bank; 4～7 only apply to Kill Password and Access Password.

Command parameter5: 4bytes AccessPassword.

Command packet:  『40H  9+L*2  EAH  L  EPC  mem  Lock  AccessPassword  CheckSum』

Return data: if success, boot code of return packet is F0H, return data is null.

『F0H  02H  EAH  24H』

Command format with reader address:

Command code:EAH

Parameter of reader address:address

Command parameter1: 1byte L, digit count of EPC, to show the word count of EPC number.

Command parameter2: L*2bytes EPC number, to show which tag to set read-write protect.

Command parameter3: 1byte mem, select memory bank.

| 0 | Kill Password |
| --- | --- |
| 1 | Access Password |
| 2 | EPC number |
| 3 | ID number in TID |
| 4 | User |

Command parameter4: 1byte Lock, control word.

| 0 | Writable |
| --- | --- |
| 1 | Writable permanently |
| 2 | Writable in secured state |
| 3 | Never writable |
| 4 | Readable and writable |
| 5 | Readable and writable permanently |
| 6 | Readable and writable in secured state |
| 7 | Never readable and writable |

0～3 only apply to EPC, TID and User memory bank; 4～7 only apply to Kill Password and Access Password.

Command parameter5: 4bytes AccessPassword.

Command packet:  『40H  10+L*2  EAH  address  L  EPC  mem  Lock  AccessPassword  CheckSum』

Return data: if success, boot code of return packet is F0H, return data is null.

『F0H  03H  EAH  address  CheckSum』

      - Write EPC

Function: to write EPC data into tag’s EPC bank. Word as unit of written data

length.

Command code: E7H

Command parameter 1: 1 byte EPC digit L, indicate number of Word of EPC

No.;

Command parameter 2: L*2 bytes EPC number;

Command parameter 3: 4 bytes AccessPassword;

Command packet: 『40H 7+L*2 E7H L EPC AccessPassword

CheckSum』

ReturnData: if success, return data will be null.

『F0H 02H E7H 27H』

Note: AccessPassword is available for MemBank in password locked status only. If MemBank unlocked, write without password; if MemBank locked forever, password is useless.

  - Collection of operation command

    - ISO18000-6C command

| Serial Number | Command | Function |
| --- | --- | --- |
| 1 | EEH | To identify readable tag ID in antenna radiation field based on mask condition |
| 2 | EDH | To get listed tag ID by rfs_ListTagID command from reader memory |
| 3 | ECH | To read a block of data starting from the appointed address in appointed memory bank of appointed tag |
| 4 | EBH | To write data into the appointed address unit in appointed memory bank of appointed tag |
| 5 | EAH | To set write protect for appointed memory bank of appointed tag |
| 6 | E7H | To write EPC number into EPC memory bank of tag |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |

    - ISO18000-6B command

| Serial number | Command | Function |
| --- | --- | --- |
| 1 | FEH | To list readable tag ID in antenna radiation field |
| 2 | FDH | To get listed tag ID by rfs_ListTagID command from reader memory |
| 3 | FBH | To list readable tag ID in antenna radiation field based on following parameter condition |
| 4 | F6H | To read a block of data starting from the appointed address of appointed tag |
| 5 | F5H | To write data into the appointed address unit of appointed tag |
| 6 | F4H | To set write protect for the appointed address unit of appointed tag |
| 7 | F3H | To read if the appointed address unit of appointed tag is set write protect |
| 8 | F2H | To write data into the appointed address unit of appointed tag |

    - Other command

| Serial number | Command | Function |
| --- | --- | --- |
| 1 | 01H | To set operation baud rate for RS232 interface |
| 2 | 02H | To get hardware and software version number of reader |
| 3 | 03H | To set relay state for reader |
| 4 | 04H | To set transmitting power coefficient for reader |
| 5 | 05H | To set frequency channel number of transmitting microwave signal for reader |
| 6 | 06H | To read operation parameter written by last command from reader |
| 7 | 09H | To set basic operation parameter: baud rate, transmitting frequency, RF power output and so on |
| 8 | 0AH | To select which antenna to transmit and receive signal |
| 9 | 0EH | To reboot reader |
| 10 | 10H | Delete all tag records stored in reader |
| 11 | 11H | To set time for reader |
| 12 | 12H | To get time from reader |
| 13 | 13H | To set the auto work parameter in reader |
| 14 | 14H | To read the auto work parameter in reader |
| 15 | 15H | To set parameter of filter. |
| 16 | 16H | To get parameter of filter. |
| 17 | 30H | To set network addess for reader |
| 18 | 31H | To get network address in reader |
| 19 | 32H | To set network MAC for reader |
| 20 | 33H | To get network MAC in reader |
| 21 | 54H | To send all reserved tag information to PC |
| 22 | 57H | Get Tag records stored in reader |

  - Electronic tag storage area and notes

- Memory unit of ISO18000-6C tag is devided into 4 areas:

- EPC memory bank: to store EPC number. It can store 96bits EPC number at most presently. Readable and writable.

- TID memory bank: to store ID number set by tag manufacturer. There are 32bits and 64bits ID presently. Readable but non-writable.

- User memory bank: different from various manufacturer. G2 tag from Impinj has no user memory bank, while NXP with 96bits. Readable and writable.

- Password memory bank: with 32Bits access password and 32Bits kill password. Readable and writable.

ISO18000-6C tag can set different protection mode for different memory bank, 4 kinds of protection modes for each memory bank:

- EPC, TID and User memory bank of ISO18000-6C tag

EPC, TID and User memory bank of ISO18000-6C tag with write protect function but without read protect:

Writable from any state—can be written without access password, and can be set to writable from secured state or permanently writable or never writable later;

Permanently Writable—can be written without access password, but can’t be set to writable from secured state or never writable later;

Writable from secured state—can be written with access password only, and can be set to never writable or writable from any state or permanently writable later;

Never Writable—can’t be written even if with access password.

- Password memory bank of ISO18000-6C tag

Password memory bank of ISO18000-6C tag can be set read protect and write protect. The read and write protect state won’t influence the usage of password.

Readable and Writable from any state—can be read and written without access password, and can be set to readable and writable from secured state or permanently readable and writable or never readable and writable later;

Permanently Readable and Writable—can be read and written without access password, but can’t be set to readable and writable from secured state or never readable and writable later;

Readable and Writable from secured state—can read and modify password with access password only, and can be set to permanently readable and writable or readable and writable from any state or never readable and writable later;

Never Readable and Writable—can’t be read or write even if with access password, can’t read or modify password forever.

Note: to set read and write protect for tag, must know access password of tag in advance.

- Memory unit of ISO-18000-6B is devided into 2 memory banks. Internal storage capacity is 2048bits, which is devided into 256bytes. An address is for each byte, corresponding to 0～255.

Thereinto:

- Address 0～7, 8 words(64bits): tag ID number. Fixed before product leaves factory, can’t be modified.

- Address 8～223: user information storage area, can be distributed at discretion. Can be rewrited, and can be locked. Once locked, can’t be rewrited again, and can’t be unlocked.

- Address 224～255: write protection byte.

- SDK software development

  - SDK compose

SDK is provided in the package of VF-447, mainly including:

- Reader1400dll.dll file —— dynamic link library

- Reader1400.lib file —— static link library

- Reader1400API.h file —— statement file of API function

- SDK catalogue —— including example program for learning API function application

  - Design introduction

    - Basic constant and figure

      - Constant definition

| Description | Introduction |
| --- | --- |
| #define ID_MAX_SIZE_64BIT 8 | //tag ID number is 64bit |
| #define ID_MAX_SIZE_96BIT 13 | //tag ID number is 128bit |
| #define MAX_LABELS 100 | // no more than 100 tags for once read and write operation |

      - API function return code

| #define _OK | 0x00 | // operation success |
| --- | --- | --- |
| //error message for communication |  |  |
| #define _init_rs232_err | 0x81 | //communication interface initialization fail |
| #define _no_scanner | 0x82 | //can not find reader |
| #define _comm_error | 0x83 | //communication data error |
| #define _baudrate_error | 0x84 | //set baud rate error |
| // error message returned from reader |  |  |
| #define _no_antenna | 0x01 | //antenna connection fail |
| #define _no_label | 0x02 | //detect no tag |
| #define _invalid_label | 0x03 | //illegal tag |
| #define _less_power | 0x04 | //read-write power not enough |
| #define _write_prot_error | 0x05 | //the memory bank in write protection |
| #define _check_sum_error | 0x06 | //checksum error |
| #define _parameter_error | 0x07 | //parameter error |
| #define _memory_error | 0x08 | //the memory bank nonexistence |
| #define _password_error | 0x09 | //password error |
| #define _killpassword_error | 0x0a | //kill password of G2 tag all zero |
| #define _nonlicet_command | 0x0b | //illegal command |
| #define _nonlicet_user | 0x0c | //illegal user with unmatched password |
| #define _invalid_command | 0x1e | //invalid command, such as command with wrong parameter |
| #define _other_error | 0x1f | //unknown command |
| //function input error |  |  |
| #define _no_cardID_input | 0x20 | //other error |

      - Data type definition

typedef USHORT apiReturn;   // type of function return value

it will return a value of apiReturn type after all API functions executed. It can judge if the function execution is successful. If fail, what’s the failure reason and etc.

      - Parameter figure in reader

typedef struct tagReaderBasicParam

{

BYTE	BaudRate;			   //(1)baud rate of serial, value：00H~08H.

BYTE	Power;				   //(2)RF power output, value：20~30dBm.(0-63)

BYTE	Min_Frequence;		//(3)start point of transmitting microwave signal frequency, value： 1~63.

BYTE	Max_Frequence;		//(4)end point of transmitting microwave signal frequency, value： 1~63.

BYTE	Reserve5;			    //(5)reserve, changed into modulation depth later.

BYTE	WorkMode;			    //(6)work mode of reader：0-Auto, 1-Command

BYTE	ReaderAddress;		    //(7)RS485 address:0--255

BYTE	NumofCard;			//(8)max tags of once reading.

BYTE	TagType;			    //(9)type of tag：01H－ISO18000-6B，02H－EPCC1，04H－EPCC1G2，08H－EM4442。

BYTE	ReadDuration;		    //(10)tag reading duration time：RF emission duration time is only effective for EM tag; 0－10ms，1－20ms，2－30ms，3－40ms.

BYTE	ReadTimes;			    //(11)read times M：reader will execute the command for M times when receiving reading command from host computer.

BYTE	EnableBuzzer;		   //(12)1:enable buzzer 0:disable buzzer

BYTE	IP1;			       //(13)IP address of reader

BYTE	IP2;			       //(14)

BYTE	IP3;			       //(15)

BYTE	IP4;			       //(16)

BYTE	Port1;				   //(17)high-order of reader port

BYTE	Port2;			      //(18)

BYTE	Mask1;				  //(19)reader mask1

BYTE	Mask2;				  //(20)reader mask2

BYTE	Mask3;				  //(21)reader mask3

BYTE	Mask4;				  //(22)reader mask4

BYTE	Gateway1;			  //(23)reader address gateway

BYTE	Gateway2;			  //(24)

BYTE	Gateway3;			  //(25)

BYTE	Gateway4;			  //(26)

BYTE	MAC1;			      //(27)MAC address of reader

BYTE	MAC2;			      //(28)

BYTE	MAC3;			      //(29)

BYTE	MAC4;			      //(30)

BYTE	MAC5;			      //(31)

BYTE	MAC6;			      //(32)

} ReaderBasicParam;

//Auto parameter of reader

typedef struct tagReaderAutoParam

{

BYTE	AutoMode;			  //(1)tag reading mode：0-timing, 1-trigger.

BYTE	TimeH;				  //(2)tag storage time: unit: second. Default 1.

BYTE	TimeL;				  //(3)

BYTE	Interval;			  //(4)0-10ms，1-20ms，2-30ms，3-50ms，4-100ms. Default 2. Auto reading tag once at intervals.

BYTE	NumH;				  //(5)tag storage quantity：default 1. The quantity of read tag ID stored in reader memory.

BYTE	NumL;				  //(6)

BYTE	OutputManner;	      //(7)data output format：0-terse，1-standard，2-XML. Default 0.

BYTE	OutInterface;		  //(8)output interface：0－RS232，1－RS485，2－RJ45. Default 0.  3- Wiegand26    4- Wiegand34

BYTE	WiegandWidth;		   //(9)value of Weigand pulse width.

BYTE	WiegandInterval;	  //(10)value of Weigand pulse interval.

BYTE	ID_Start;			  //(11)start address of output ID, value 0～4.

BYTE    IDPosition;		  //(12)storage address for tag ID in tag.

BYTE	Report_Interval;	  //(13) report interval：unit is second. Default 1. Automatically notify host pc once at intervals.

BYTE	Report_Condition;	  //(14)condition of report：default 1. 0-notify now，1-timing，2-add，3-remove，4-change

BYTE	Report_Output;		  //(15)report output port

BYTE	Antenna;			  //(16)select antenna.1-ant1,2-ant2,4-ant4,8-ant8

BYTE	TriggerMode;	      //(17)trigger mode(default0): 0-low level 1-high level

BYTE	HostIP1;			  //(18)notified IP address

BYTE	HostIP2;			  //(19)

BYTE	HostIP3;			  //(20)

BYTE	HostIP4;			  //(21)

BYTE	Port1;				  //(22)notified port

BYTE	Port2;				  //(23)

BYTE	Reserve24;			  //(24)notified MAC,mofi by mqs 20121207 reserve

BYTE	ArgentinaSim;		  //(25)//emulation mode(argentina),0—non-emulation，1--emulation

BYTE	CardTime1;			  //(26)//reading time-out 1

BYTE	CardTime2;			  //(27)//reading time-out 2

BYTE	ArgentinaMode;		  //(28)//5 modes for argentina, 0---Only ATA ; 1---Only EPC; 2---Only EPC & TID; 3---ATA + EPC; 4---ATA + EPC & TID.

BYTE	Reserve29;			  //(29)

BYTE	Alarm;				  //(30)0-no alarm，1-alarm. To detect if alarm in timing and trigger modes.

BYTE	Reserve31;		      //(31)time interval for standard output，default value is 120s，1～255.

BYTE	EnableRelay;		  //(32)to control relay or not in Auto mode 1:control  0: no control

} ReaderAutoParam;

//frequency for various countries

static const tagReaderFreq stuctFreqCountry[]=

{

{"00---FCC(American)", 63, 400, 902600},							//(0),{"00---FCC(American)", 50, 500, 902750},

{"01---ETSI EN 300-220(Europe300-220)", 11, 200, 865500},			//(1),{"01---ETSI EN 300-220(Europe300-220)", -1, -1, -1},

{"02---ETSI EN 302-208(Europe302-208)", 4, 600, 865700},			//(2)

{"03---HK920-925(Hong Kong)", 10, 500, 920250},					//(3)

{"04---TaiWan 922-928(Taiwan)", 12, 500, 922250},					//(4)

{"05---Japan 952-954(Japan)", 0, 0, 0},								//(5)

{"06---Japan 952-955(Japan)", 14,200, 952200},						//(6)

{"07---ETSI EN 302-208(Europe)", 4, 600, 865700},					//(7)

{"08---Korea 917-921(Korea)", 6, 600, 917300},						//(8)

{"09---Malaysia 919-923(Malaysia)", 8, 500, 919250},				//(9)

{"10--China 920-925(China)", 16, 250, 920625},						//(10)

{"11--Japan 952-956(Japan)", 4, 1200, 952400},						//(11)

{"12--South Africa 915-919(Poncho)", 17, 200, 915600},				//(12)

{"13--Brazil 902-907/915-928(Brazil)", 35, 500, 902750},			//(13)

{"14--Thailand 920-925(Thailand)", -1, -1, -1},						//(14)

{"15--Singapore 920-925(Singapore)", 10, 500, 920250},				//(15)

{"16--Australia 920-926(Australia)", 12, 500, 920250},				//(16)

{"17--India 865-867(India)", 4, 600, 865100},						//(17)

{"18--Uruguay 916-928(Uruguay)", 23, 500, 916250},					//(18)

{"19--Vietnam 920-925(Vietnam)", 10, 500, 920250},					//(19)

{"20--Israel 915-917", 1, 0, 916250},								//(20)

{"21--Philippines 918-920(Philippines)", 4, 500, 918250},			//(21)

{"22--Canada 902-928(Canada)", 42, 500, 902750},					//(22)

{"23--Indonesia 923-925(Indonesia)", 4, 500, 923250},				//(23)

{"24--New Zealand 921.5-928(New Zealand)", 11, 500, 922250},		//(24)

};

      - Function return code

When command execution fail, function returns error code. Common error code:

| Command | Function |
| --- | --- |
| 00(00H) | Command success or detection correct |
| 01(01H) | Antenna connection fail |
| 02(02H) | Detect no tag |
| 03(03H) | Illegal tag |
| 04(04H) | Read-write power not enough |
| 05(05H) | Read and write protection in the memory bank |
| 06(06H) | Checksum error |
| 07(07H) | Parameter error |
| 08(08H) | Memory bank nonexistence |
| 09(09H) | Password error |
| 10(0AH) | Kill password is all zero |
| 11(0BH) | When reader in auto mode, only receive AutoMode and Reboot commands, other command is illegal |
| 12(0CH) | Illegal user with unmatched password |
| 13(0DH) | External RF interference |
| 14 (0EH) | Tag with read protection |
| …… | …… |
| 30(1EH) | Invalid command, such as wrong command |
| 31(1FH) | Unknown command |
| 32(20H) | Other error |

    - Control command function

      - Connect reader

Using serial port Connection:

apiReturn  ConnectScanner(HANDLE *hScanner, char *szPort, int  nBaudRate);

Function: to establish communication connection with reader, and set baud rate for reader.

Input parameter:

szPort : character pointer directing at communication port, eg. 『COM1』、『COM2』……

nBaudRate: baud rate of serial, effective value is: 9600，19200，38400，57600，115200.

Output parameter: to judge if connection success or failure reason according to apiReturn value returned by function.

hScanner : reader handle

command with reader adress:

apiReturn _stdcall ConnectScanner485 (HANDLE *hScanner, char *szPort, int nBaudRate,int Address);

Function: to establish communication connection with reader, and set baud rate for reader.

Input parameter:

szPort : character pointer directing at communication port, eg.『COM1』、『COM2』……

nBaudRate: baud rate of serial, effective value is: 9600，19200，38400，57600，115200.

Address: reader address.

Output parameter: to judge if connection success or failure reason according to apiReturn value returned by function.

hScanner : reader handle

connection via Ethernet port

apiReturn _stdcall Net_ConnectScanner(SOCKET *hSocket,char *nTargetAddress,UINT nTargetPort,char *nHostAddress,UINT nHostPort);

Function: to establish communication connection with reader, and set baud rate for reader.

Input parameter:

nTargetAddress: target addess, such as『192.168.0.1』……

nTargetPort: target communication port, such as『1969』

nHostAddress: host address, such as『192.168.0.2』……

nHostPort: host communication port, such as『5000』

output parameter:

hSocket : reader communication handle

return: if return value of function is OK, it means connection success, otherwise, connection fail.

Note: each reader shall execute the command to get corresponding reader communication handle hSocket.

      - Disconnection

RS232,RS485 disconnection with reader

apiReturn  DisconnectScanner(HANDLE hScanner)；

apiReturn Net_DisconnectScanner();

Function: to stop the connection with reader to release serial resource.

Input parameter:

hSacnner: reader communication handle

      - Set baud rate

apiReturn _stdcall SetBaudRate(HANDLE hScanner, int nBaudRate,int Address)

apiReturn Net_SetBaudRate(SOCKET hSocket, int nBaudRate);

Function: to set operation baud rate of RS232 port.

Input parameter:

hSacnner/hSocket: reader communication handle

nBaudRate: vaule:9600，19200，38400，57600，115200

Address: RS485 networking address of reader，Address =0 no networking.

return: if function return value is OK, setting success, otherwise failure reason.

      - Read version

apiReturn _stdcall GetReaderVersion(HANDLE hScanner, WORD *wHardVer, WORD  *wSoftVer,int Address)

apiReturn Net_GetReaderVersion(SOCKET hSocket, WORD *wHardVer, WORD  *wSoftVer,BYTE * IPaddress);

Function: to read hardware and software version number of reader.

Input parameter:

hSacnner/hSocket: reader communication handle

Address: RS485 networking address of reader，Address =0 no networking.

Output parameter:

wHardVer: hardware version number of reader.

WSoftVer: software version number of reader.

return: if function return value is OK, read success, otherwise fail.

      - Set output power

apiReturn _stdcall SetOutputPower(HANDLE hScanner, int nPower1,int Address)

apiReturn Net_SetOutputPower(SOCKET hSocket, int nPower,BYTE * IPaddress);

Function: to set RF output power of reader.

Input parameter:

hSacnner/hSocket: reader communication handle

nPower1/nPower: output power value

Address: RS485 networking address of reader，Address =0 no networking.

return: if function return value is OK，it means setting success, otherwise fail.

      - Set operation frequency

apiReturn _stdcall SetFrequency(HANDLE hScanner, int Min_Frequency, int Max_Frequency,int Address)

apiReturn _stdcall Net_SetFrequency(SOCKET hSocket, int Min_Frequency, int Max_Frequency)

Function: to set operation frequency of reader.

Input parameter:

hSacnner/hSocket: reader communication handle

Min_Frequency: start frequency of reader，vaule is 0-59.

Max_Frequency: end frequency of reader，value is 0-59.

When Min_Frequency = Max_Frequency，reader works in fixed frequency.

Address: RS485 networking address of reader，Address =0 no networking.

function: if function return value is OK，it means setting success, otherwise fail.

      - Read reader basic operation parameter

apiReturn _stdcall ReadBasicParam(HANDLE hScanner, ReaderBasicParam * pParam,int Address);

apiReturn Net_ ReadBasicParam (SOCKET hSocket, ReaderBasicParam * pParam);

Function: to read operation parameter written by last command in reader.

Input parameter:

hSacnner/hSocket: reader communication handle

Address: RS485 networking address of reader，Address =0 no networking.

Output parameter:

pParam: return operation parameter of reader, 32 bytes

return: if function return value is OK, it means read success, otherwise failure reason.

      - Set reader basic operation parameter

apiReturn _stdcall WriteBasicParam(HANDLE hScanner, ReaderBasicParam * pParam,int Address);

apiReturn Net_ WriteBasicParam (SOCKET hSocket, ReaderBasicParam * pParam);

Function: to reader operation parameter for reader.

Input parameter:

hSacnner/hSocket: reader communication handle

pParam: operation parameter of reader, 32 bytes

Address: RS485 networking address of reader，Address =0 no networking.

return: if function return value is OK，it means setting success, otherwise failure reason.

      - Read reader auto parameter

apiReturn _stdcall ReadAutoParam (HANDLE hScanner, ReaderAutoParam* pParam,int Address);

apiReturn Net_ ReadAutoParam (SOCKET hSocket, ReaderAutoParam* pParam);

Function: to read operation parameter written by last command in reader.

Input parameter:

hSacnner/hSocket: reader communication handle

Address: RS485 networking address of reader，Address =0 no networking.

Output parameter:

pParam: return operation parameter of reader, 32 bytes

return: if function return value is OK，it mean read success, otherwise failure reason.

      - Set reader auto parameter

apiReturn _stdcall WriteAutoParam (HANDLE hScanner, ReaderAutoParam* pParam,int Address);

apiReturn Net_ WriteAutoParam (SOCKET hSocket, ReaderAutoParam* pParam);

Function: to set auto operation parameter in reader.

Input parameter:

hSacnner/hSocket: reader communication handle

pParam: operation parameter of reader, 32 bytes

Address: RS485 networking address of reader，Address =0 no networking.

return: if function return value is OK，it means set success, otherwise failure reason.

      - Select antenna

apiReturn _stdcall SetAntenna(HANDLE hScanner, int Antenna,int Address) ;

apiReturn Net_SetAntenna(SOCKET hSocket, int Antenna);

Function: to select which antenna to receive and transmit signal.

Input parameter:

hSacnner/hSocket: reader handle

Antenna: antenna number，1-No1 antenna，2-No2 antenna，4-No3 antenna，8-No4 antenna

Address: RS485 networking address of reader，RS485Address=0 no networking.

Output parameter:

return: if function return value is OK, it means set success, otherwise failure reason.

      - Set relay state in reader

apiReturn _stdcall SetRelay(HANDLE hScanner, int Relay,int Address) ;

apiReturn Net_SetRelay(SOCKET hSocket, int Relay);

Function: to set relay state for reader.

Input parameter:

hSacnner/hSocket: reader handle

Relay: 1byte. Bit0=1, No1 relay on; Bit0=0, No1 relay off. Bit1=1, No2 relay on; Bit1=0, No2 relay off. And so on.

Address: RS485 networking address of reader，Address =0 no networking.

return: if function return value is OK, it means set success, otherwise failure reason.

      - Reboot reader

apiReturn Reboot(HANDLE hScanner,int Address);

apiReturn Net_Reboot(SOCKET hSocket);

Function: to reboot reader, power on again.

Input parameter:

hSacnner/hSocket: reader communication handle

Address: RS485 networking address of reader，Address =0 no networking.

return: if function return value is OK, it means set success, otherwise failure reason.

      - Set time

apiReturn _stdcall SetReaderTime(HANDLE hScanner, ReaderDate time ,int Address) ;

apiReturn Net_SetReaderTime(SOCKET hSocket, ReaderDate time)

Function: to set time for reader based on Host pc time.

Input parameter:

hSacnner/hSocket: reader communication port handle

time: host pc time，6bytes

Address: RS485 networking address of reader, Address =0 no networking.

return: if function return value is OK, it means set success, otherwise failure reason.

      - Get time

apiReturn _stdcall GetReaderTime(HANDLE hScanner, ReaderDate *time ,int Address) ;

apiReturn GetReaderTime(SOCKET hSocket, ReaderDate *time)

Function: to read time of reader.

Input parameter:

hSacnner/hSocket: reader communication handle

Address: RS485 networking address of reader，Address =0 no networking.

Output parameter:

time: return time of reader, 6bytes

return: if function return value is OK，it means read success, otherwise failure reason.

      - Get record

apiReturn _stdcall GetRecord(HANDLE hScanner, ReaderDate *stime, ReaderDate *etime, int startaddr, int listlen, int *relistlen, int *taglen, BYTE * data) ;

apiReturn _stdcall Net_GetRecord(SOCKET hSocket, ReaderDate *stime, ReaderDate *etime, int startaddr, int listlen, int *relistlen, int *taglen, BYTE * data);

Function: to read record of identified tags in reader.

Input parameter:

hSacnner/hSocket: reader communication handle

stime: start time

etime: end time

startaddr: start record

listlen: records count to read

output parameter:

relistlen: records count by actual reading

taglen: length of each record by actual reading

data: records by reading

return: if function return value is OK，it means read success, otherwise failure reason.

      - Delete all records

apiReturn _stdcall DeleteAllRecord(HANDLE hScanner) ;

apiReturn _stdcall Net_DeleteAllRecord(SOCKET hSocket);

Function: to delete all records in reader.

Input parameter:

hSacnner/hSocket: reader communication port handle

return: if function return value is OK, it means set success, otherwise failure reason.

    - Network command

      - Set IP address for reader

apiReturn _stdcall SetReaderNetwork(HANDLE hScanner, BYTE IP_Address[4], int Port, BYTE Mask[4], BYTE Gateway[4],int Address) ;

apiReturn _stdcall Net_SetReaderNetwork(SOCKET hSocket, BYTE IP_Address[4], int Port, BYTE Mask[4], BYTE Gateway[4]);

Function: to set network IP address for reader.

Input parameter:

hSacnner/hSocket: reader communication port handle

IP_Address[4]: IP address of reader

Port: network port number of reader

Mask[4]: network IP address mask of reader

Gateway[4]: gateway of reader

Address: RS485 networking address of reader，Address =0 no networking.

Return: if function return value is OK, it means set success, otherwise failure reason.

      - Get IP address in reader

apiReturn _stdcall GetReaderNetwork(HANDLE hScanner, BYTE *IP_Address, int *Port, BYTE *Mask, BYTE *Gateway,int Address) ;

apiReturn _stdcall Net_GetReaderNetwork(SOCKET hSocket, BYTE *IP_Address, int *Port, BYTE *Mask, BYTE *Gateway);

Funtion: to get network IP addess of reader.

Input parameter:

hSacnner/hSocket: reader communication port handle

output parameter:

IP_Address[4]: IP address of reader

Port: network port number of reader

Mask[4]: network IP address mask of reader

Gateway[4]: gateway of reader

Address: RS485 networking address of reader，Address =0 no networking.

return: if function return value is OK, it means set success, otherwise failure reason.

      - Set reader MAC address

apiReturn _stdcall SetReaderMAC(HANDLE hScanner, BYTE MAC[6],int Address) ;

apiReturn _stdcall Net_SetReaderMAC(SOCKET hSocket, BYTE MAC[6]);

Function: to set network MAC address for reader.

Input parameter:

hSacnner/hSocket: reader communication port handle

MAC[6]: network MAC address of reader

Address: RS485 networking address of reader，Address =0 no networking.

return: if function return value is OK, it means set success, otherwise failure reason.

      - Get reader MAC address

apiReturn _stdcall GetReaderMAC(HANDLE hScanner, BYTE *MAC,int Address) ;

apiReturn _stdcall Net_GetReaderMAC(SOCKET hSocket, BYTE *MAC);

Function: to get network MAC address of reader.

Input parameter:

hSacnner/hSocket: reader communication port handle

output parameter:

MAC: network MAC address of reader

Address: RS485 networking address，Address =0 no networking.

return: if function return value is OK, it means set success, otherwise failure reason.

    - Read write ISO18000-6B function

      - Identify tag ID number

apiReturn _stdcall ISO6B_ReadLabelID(HANDLE hScanner, BYTE *IDBuffer, int *nCounter,int Address) ;

apiReturn _stdcall Net_ISO6B_ReadLabelID(SOCKET hSocket, BYTE *IDBuffer, int *nCounter);

Function: to read all ID number of all readable tags in antenna radiation field.

Input parameter:

hSacnner/hSocket: reader communication port handle

output parameter:

nCounter: return tag count by actual reading ID number

IDBuffer: store ID number of tag by reading in cache

Address: RS485 networking address of reader，Address =0 no networking.

return: if function return value is OK, it means identify success, otherwise failure reason.

      - Identify selected tag ID number

apiReturn _stdcall ISO6B_ListSelectedID(HANDLE hScanner, int Cmd, int ptr, BYTE Mask, BYTE *Data, BYTE *IDBuffer, int *nCounter,int Address) ;

apiReturn _stdcall Net_ISO6B_ListSelectedID(SOCKET hSocket, int Cmd, int ptr, BYTE Mask, BYTE *Data, BYTE *IDBuffer, int *nCounter);

Function: to read ID number of selected tags in antenna radiation field.

Input parameter:

hSacnner/hSocket: reader communication port handle

Cmd: condition of selected tag

| 00 | Equal to |
| --- | --- |
| 01 | Unequal to |
| 02 | Greater than |
| 03 | Less than |

ptr: start address of tag data, value 0～223

Mask: data mask；each bit of the byte is corresponding to a comparative bit. 0 means the byte not for comparison；1 means the byte for comparison.

Data: data to compare

Address: RS485 networking address of reader，Address =0 no networking.

Output parameter:

nCounter: return tag count by actual reading ID number

IDBuffer: store ID number of tag by reading in cache

return: if function return value is OK, it means identify success, otherwise failure reason.

      - Read data block

apiReturn ISO6B_ReadByteBlock(HANDLE hScanner, BYTE *IDBuffer, BYTE ptr, BYTE len,BYTE *Data,int Address)

apiReturn _stdcall Net_ISO6B_ReadByteBlock(SOCKET hSocket, BYTE *IDBuffer, BYTE ptr, BYTE len,BYTE *Data);

Function: to read data in a section of continous memory of tag.

Input parameter:

hSacnner/hSocket: reader communication port handle

IDBuffer: ID number of tag to read

ptr: start addess of tag memory to read(0～223 Byte)

len: length of data block, that’s how many bytes for once reading(Byte)

Address: RS485 networking address of reader，Address =0 no networking.

Output parameter:

Data: return data by reading

Note: nLen shall be  32.  (nAddress+nLen) 223。

Return: if function return value is OK, it means read success, otherwise failure reason.

      - Write data block

apiReturn _stdcall ISO6B_WriteByteBlock(HANDLE hScanner, BYTE *IDBuffer, BYTE ptr, BYTE len, BYTE *Data,int Address) ;

apiReturn _stdcall Net_ISO6B_WriteByteBlock(SOCKET hSocket, BYTE *IDBuffer, BYTE ptr, BYTE len, BYTE *Data);

Function: to write data into appointed address unit of tag

Input parameter:

hSacnner/hSocket: reader communication port handle

IDBuffer: ID number of tag to write

ptr: start address of tag memory to write(8～223)

len: length of data block, that how many words for once wirting(4Bytes/word)

Data: data to write

Note: ptr shall be integral multiple of 4. (nAddress+nLen) 223.

Address: RS485 networking address of reader，Address =0 no networking.

return: if function return value is OK，it means write success, otherwise failure reason.

      - Slow write data block

apiReturn _stdcall ISO6B_WriteAByte(HANDLE hScanner, BYTE *IDBuffer, BYTE ptr, BYTE len, BYTE *Data,int Address);

apiReturn _stdcall Net_ISO6B_WriteAByte(SOCKET hSocket, BYTE *IDBuffer, BYTE ptr, BYTE len, BYTE *Data);

Function: to write data into appointed address unit of tag byte by byte.

Input parameter:

hSacnner/hSocket: reader communication port handle

IDBuffer: ID number of tag to write

ptr: start address of tag memory to write(8～223)

len: length of data block，that’s how many words to write once(4Bytes/word)

Data: data to write

Note: (nAddress+nLen) 223。

Address: RS485 networking address of reader，Address =0 no networking.

return: if function return value is OK，it means write success, otherwise failure reason.

Note: the command writes data into tag byte by byte, slow in speed. Only used for tags not support previous write command.

      - Set write protection

apiReturn _stdcall ISO6B_WriteProtect(HANDLE hScanner, BYTE *IDBuffer, BYTE ptr,int Address);

apiReturn _stdcall Net_ISO6B_WriteProtect(SOCKET hSocket, BYTE *IDBuffer, BYTE ptr);

Function: to set write protect for appointed address unit of appointed tag.

Input parameter:

hSacnner/hSocket: reader communication port handle

IDBuffer: ID number of tag to write

ptr: memory address of tag to set write protect(8～223)

Address: RS485 networking address of reader，Address =0 no networking.

return: if function return value is OK, it means set success, otherwise failure reason.

      - Read write protection

apiReturn _stdcall ISO6B_ReadWriteProtect(HANDLE hScanner, BYTE *IDBuffer, BYTE ptr, BYTE *Protected,int Address);

apiReturn _stdcall Net_ISO6B_ReadWriteProtect(SOCKET hSocket, BYTE *IDBuffer, BYTE ptr, BYTE *Protected);

Function: to read if the appointed address unit of appointed tag is set write protect.

Input parameter:

hSacnner/hSocket: reader communication port handle

IDBuffer: ID number of tag to write

ptr: memory address of tag to read write protection state (0～223)

Address: RS485 networking address of reader，Address =0 no networking.

Output parameter:

Protected: protection state，0- no protect，1-protected

return: if function return value is OK, it means read success, otherwise failure reason.

    - Read write ISO18000-6C function

      - Identify EPC number of ISO18000-6C tag

apiReturn _stdcall EPC1G2_ReadLabelID(HANDLE hScanner, BYTE mem, int ptr, BYTE len, BYTE *mask, BYTE *IDBuffer, int *nCounter,int Address);

apiReturn _stdcall Net_EPC1G2_ReadLabelID(SOCKET hSocket, BYTE mem, int ptr, BYTE len, BYTE *mask, BYTE *IDBuffer, int *nCounter);

Function: to read EPC number of all eligible readable tag in antenna radiation field.

Input parameter:

hSacnner/hSocket: reader communication port handle

mem: select memory bank；

| 0 | Passwrod |
| --- | --- |
| 1 | EPC |
| 2 | TID |
| 3 | User |

ptr: start address of mask(unit:Bit)

len: length of mask(unit:Bit)

mask: mask(unit:Byte)，if len/8 is integer，length of mask is len/8；if len/8 is not integer,length of mask is len/8+1. Last byte data of mask in high-order position, zero fill in low-order position.

Address: RS485 networking address of reader，Address =0 no networking.

Output parameter:

IDBuffer: EPC number of tag by read

NCounter: tag count by read

return: if function return value is OK，it means identify success, otherwise failure reason.

Note: LEN=0, to identify all readable tag ID in antenna radiation field.

      - Read a block data

apiReturn _stdcall EPC1G2_ReadWordBlock(HANDLE hScanner, BYTE EPC_WORD, BYTE *IDBuffer, BYTE mem, BYTE ptr, BYTE len, BYTE *Data, BYTE *AccessPassword,int Address);

apiReturn _stdcall Net_EPC1G2_ReadWordBlock(SOCKET hSocket, BYTE EPC_WORD, BYTE *IDBuffer, BYTE mem, BYTE ptr, BYTE len, BYTE *Data, BYTE *AccessPassword);

Function: to read data in a section of continous address of tag.

Input parameter:

hSacnner/hSocket: reader communication port handle

EPC_WORD: length of EPC, L(unit:Word)；such as length of 96BitsEPC L=6(Words)；

IDBuffer: EPC number of selected tag

mem: select memory bank；0-Password，1-EPC，2-TID，3-User.

ptr: start address to read(unit:WORD)

len: length to read(unit:WORD)

AccessPassword: 4bytes AccessPassword

Address: RS485 networking address of reader，Address =0 no networking.

Output parameter:

Data: data to read

return: if function return value is OK，it means read success, otherwise failure reason.

Note: AccessPassword only workable for password memory bank in password lock state.

      - Write a block data

apiReturn _stdcall EPC1G2_WriteWordBlock(HANDLE hScanner, BYTE EPC_WORD, BYTE *IDBuffer, BYTE mem, BYTE ptr, BYTE len, BYTE *Data, BYTE *AccessPassword,int Address);

apiReturn _stdcall Net_EPC1G2_WriteWordBlock(SOCKET hSocket, BYTE EPC_WORD, BYTE *IDBuffer, BYTE mem, BYTE ptr, BYTE len, BYTE *Data, BYTE *AccessPassword);

Function: to write data into appointed address unit of tag.

Input parameter:

hSacnner/hSocket: reader communication port handle

EPC_WORD: length of EPC, L(unit:Word)；such as length of 96BitsEPC L=6(Words)；

IDBuffer: EPC number of selected tag

mem: select memory bank

| 0 | Password |
| --- | --- |
| 1 | EPC |
| 2 | TID |
| 3 | User |

ptr: start address to write(unit:WORD)

len: length to write(unit:WORD)

Data: data to write

AccessPassword: 4bytes AccessPassword

Address: RS485 networking address of reader，Address =0 no networking.

return: if function return value is OK，it means write success, otherwise failure reason.

Note: AccessPassword only workable for memory bank in password lock state. If memory bank unlocked, can be written without password. If memory bank locked permanently, password useless.

      - Set read write protection state

apiReturn _stdcall EPC1G2_SetLock(HANDLE hScanner, BYTE EPC_WORD, BYTE *IDBuffer, BYTE mem, BYTE Lock, BYTE *AccessPassword,int Address);

apiReturn _stdcall Net_EPC1G2_SetLock(SOCKET hSocket, BYTE EPC_WORD, BYTE *IDBuffer, BYTE mem, BYTE Lock, BYTE *AccessPassword);

Function: to set write protect for appointed memory bank of appointed tag.

Input parameter:

hSacnner/hSocket: reader communication port handle

EPC_WORD: length of EPC, L(unit:Word)；such as length of 96BitsEPC L=6(Words)；

IDBuffer: EPC number of selected tag

mem: select memory bank

| 0 | Kill Password |
| --- | --- |
| 1 | Access Password |
| 2 | EPC number |
| 3 | TID ID number |
| 4 | User |

Lock: control word.

| 0 | Writable |
| --- | --- |
| 1 | Permanently writable |
| 2 | Writable from secured state |
| 3 | Never writable |
| 4 | Readable and writable |
| 5 | Permanently readable and writable |
| 6 | Readable and writable from secured state |
| 7 | Never readable and writable |

Note:0～3 only apply to EPC、TID and User memory bank；4～7 only apply to Kill Password and Access Password.

AccessPassword: 4bytes AccessPassword

Address: RS485 networking address of reader，Address =0 no networking.

return: if function return value is OK，it means set success, otherwise failure reason.