VD-68

RFID Desktop Reader

Directory of Product Information

Content

1	Product Summarize	3

1.1	Shape And Interface	3

1.2	Performance Index	5

1.3	Function	5

2	Device Installation	6

2.1	Connection	6

2.2	Connector And Controller	6

2.3	Connection Power Supply	7

2.4	Device Adjustment	8

2.5	Tag Place	8

3	Usual Faults	9

4	After-Sales Service	10

4.1	Support And Service	10

4.2	Service Hotline	12

- Product Summarize

VD-68 Desktop reader is the latest product developed by us, which can read and write tag or card in accordance with ISO18000-6B/ISO180006C.It is widely used for vehicle access management, library, ETC, access management, electronic security, logistic, production automatic management and so on.

  - Shape And Interface

Shape of VD-68 integrated reader is as follow picture, which is with ABS shell,

There is a communication interface on the side of the VD-68 reader. The port surface has the following characteristics:

A. Power interface: +12V DC socket

B. RJ45 communication interface: 1 RJ45 communication port is directly connected to the network through a network cable.

C. RS232 communication interface

D. GP I/O interfaceI/O

I/O interface (12-pin)-the front view of the reader interface panel

| Name | Signal direction | illustrate |
| --- | --- | --- |
| GND | I/O | ground wire |
| WD1-0 | O | Output 1 (TTL Level) |
| WD1-1 | O | Output 2 (TTL Level) |
| WD2-0 | O | Output 3 (TTL Level) |
| WD2-1 | O | Output 4 (TTL Level) |
| GND | I/O | ground wire |
| FIN1 | I | Trigger 1 (TTL Level) |
| FIN2 | I | Trigger 2 (TTL Level) |
| 485+ | I/O | RS485+ |
| 485- | I/O | RS485- |
| D1 | O | relay |
| D1 | O | relay |

The two connectors of the relay can be short-circuited or disconnected, and the user controls the relay through the upper computer.

Trigger 1～2: The user controls the high and low levels through the upper computer.

  - Performance Index

- Read-write tag:electronic tag in accordance with ISO18000-6C

- Operating frequency: 860～960MHz

- Way of Working:by FHSS or fixed frequency launch

- Output power:13-27dbm(could be a little difference from various countries or area)

- Readdistance:read distance>500cm,write distance>100cm( antenna and tag dependent)

- Read rate:32bits/word,6ms in time,average

- Write rate:32bits/word,50ms in time ,avreage

- Power supply:+DC12V/3A direct current

- power:<10w ,in average

- Working temperature: -10℃~+55℃

  - Function

- Read tag:electronic tag in accordance with ISO18000-6C

- Can read EPC tags in different length.(16,32,48,64,80 or 96)

- Can read EPC Data.

- Can read data from user area.

- Can read data from TID area

- Can visit tag and kill password

- Can write different length EPC code(16,32,48,64,80 and 96)

- Can write data from user area

- Can revise tag visit and kill code

- PC,TID and user area can write protection

- Password section can write protection

- Can kill tag

- Device Installation

  - Connection

In system, connection of reader, antenna, tag and controller (or PC machine) shows as follows:

Figure 3-1-1 typical connection about the relevant equipments

  - Connector And Controller

VD-68 reader can connect with controller through any interface from RS232

RS232 interface can connect with pc directly, by matched cable, length of RS232 interface connection line should be less than 10meters, drawing shows as 2-3

Figure 3-2-2 Reader’s RS232 is connected with PC

  - Connection Power Supply

VD-68 reader adopts +12V/3A direct current power supply, we can supply you reader with required alternating and current convertor,the way to connect power is as follows:

- Check if voltage of alternating power supply and operating frequency is in accordance with AC 100-240V/50Hz

- Make DC output pin of power convertor insert into +12V direct current power interface of reader.

- Make 220V alternating power input line of convertor insert or connect with line of alternating power supply.

- If the Indicator light of power supply is ture on ,that means workable

  - Device Adjustment

Key of device adjustment is: adjust height, direction angle, obiquity, to make reader can read electronic tag in the expected range.

- Turn on reader power supply,set reader to be mode of auto working,close parameter setting procedure ,and turn off the connection with PC

- Ture off reader power supply first, and then turn on, reader will automatically turn into state of continuous working.

(3)As material of identified object and required tag is different,so tag should be stuck into object with same material as identified one,traverses object in the expected range of tag is to read.If reader can get tag,then inlay buzzer tweets and green LED blinks.

- Adjust height and angle of antenna; make reading range at the best.

Note: reader only radiates microwave power when reading tag, then adjuster should keep a distance of 30cm from antenna, that is  meet American FCC requirements.

  - Tag Place

In work process of RFID, tag place is a usual job, some notes are as follows:

- Adopts gluewater recommended by factory

- If labeled in the surface of metal, professional glue is recommended.

- Labeled place has to be field tested to make sure read best.

Note: Our Company offers 2 e tags to customers, in ISO18000-6B standard

- Air medium.can be held to use, also can be instead by our block set, insert card into it directly.

- Glass medium,card should be stuck in glass.(non water-solubility is required)

- Usual Faults

Table 3-1 usual faults and solutions for reader

| Fault Phenomena | Possible Reason | Solution |
| --- | --- | --- |
| can not read card | antenna is not well connected | Check antenna connection details |
|  | beyond to reading area of module | make card close to module antenna |
|  | antenna is broken | Change new antenna |
|  | Parameter of RF Power is too low | Renew parameter |
|  | tag is broken | Test in other tag |
|  | reader is broken | Contact technican |
| Can not be connected with reader | no power | Check oulet and adapter |
|  | Com ties up or broken | Check interface through software and test if it cummunicates |
|  | baud rate is not set in accordance | Set it to be in accordance by software |
|  | reader is broken | Contact technican |
| Reading distance is too near | antenna is not well connected | Check details of antenna connection |
|  | Parameter value of RF Power is too low | renew factory parameter |
|  | tag is not in accordance with antenna polarization | revolve tag in 90degree |
|  | reader is broken | Contact technican |
|  | tag is broken | change new tag |

- After-Sales Service

  - Support And Service

Website please be online with internet by computer; enter http://www.vanch.NET  to get specifications, purchasing devices and other information about VD-68

Customer support  If you have doubts about instruction and can not get solutions from our website,please contact our customers’ service centre.If your device has to be repaired,please contact us for service information,delivery instruction and maintenance cost excludes from after service before it is sent out

Service If device needs to be diagnosed or other service,please contact our customers’ service centre.please don’t send it out before you contact us.If your device is under guarantee,please prepare receipt of purchasing.

Please do as follows before you contact us:

- read introduction

- prepare your machine

  - model

  - serial number

  - receipt of purchasing

- Prepare your machine, customer service representative maybe ask you to operate machine.

- Prepare your questions; detailed problem description can help customer service representative solve your problem at the earliest.

Guarantee

Important information this text is a guarantee declaration for hardware products, please read carefully.

One year guarantee for hardware

Shenzhen Vanch Intelligent Technology Co., Ltd offers this hardware product guarantee for no faults in material and manufacture from receipt date of purchasing.

If Shenzhen Vanch Intelligence Technology Co., Ltd receives fault advise as above during Guarantee date.Shenzhen Vanch Intelligent Technology Company. Ltd can decide whether to repair or replace the proved failure product.

If Shenzhen Vanch Intelligent Technology Co., Ltd can not repair or replace the product within reasonable time, customer can ask for refundment.

Scope of warranty

Guarantee above excludes from faults caused as follows:

Misapplication,modification without authorization,open the hardware,operates or stores up this product in an improper environment,damages during transportation,improper maintaince,or caused by not using software ,parts,media,article,expendable from Shenzhen Vanch Intelligent Technology Co.,Ltd,or project not for this product.

This is the only guarantee declaration for products from Shenzhen Vanch Intelligent Technology Co., Ltd, noelse guarantee in writing or oral

Shenzhen Vanch Intelligent Technology will not be responsible for cases as follows: direct, indirect, particular and accidental damage, including profit, whatever it is based on guarantee declaration, agreement, defect, or any other law theory.

Disclaimer clauses:Radio transmission equipment can make interference to the task of any unprotected industry,medical and scientific equipment.There is possible interference to other electronic equipment when this equipment works.If any inquiries in this aspect,please consult from Shenzhen Vanch Intelligent Technology Co.,Ltd for improvement.

  - Service Hotline

Thank you for choosing our product, we will support you with technology about product application, and offer solutions to any difficulties.Contact mode of our company are as follows:

Company: Shenzhen Vanch Intellifent Technology Co., Ltd

Address: 5F, 535, BaGua 6 st., FuTian district, Shenzhen, Guangdong, China

Postal code: 518029

Tel: +86 755 82426775

Fax: +86 755 82403457

Website: www.vanch.NET

Email: info@vanch.net