# General Description

# ZK-RFID202R is a high performance UHF RFID integrated reader. It is designed upon fully self-intellectual property. Based on proprietary efficient digital signal processing algorithm, it supports fast tag read/write operation with high identification rate. It can be widely applied in many RFID application systems such as logistics, access control, anti-counterfeit and industrial production process control system.

# Features

- Self-intellectual property;

- Design based on Impinj R2000 with excellent anti-collision performance;

- Support ISO18000-6B, ISO18000-6C(EPC C1G2) protocol tag;

- 902~928MHz frequency band(frequency customization optional);

- FHSS or Fix Frequency transmission;

- RF output power up to 30dbm(adjustable);

- Built-in high performance 8dbi circular antenna with effect distance up to 9m;

- one more extendable external antenna port;

- Support auto-running, interactive and trigger-activating work mode;

- Wide power supply voltage input (+9V~+24V);

- Support RS232, RS485, Wiegand, TCPIP interface;

- Support 1 external indicator control port and 2 built-in relays;

- IP67 seal grade housing, ideal for applications in harsh outdoor environment;

* Effective distance depends on antenna, tag and environment.

# Characteristics

- Absolute Maximum Rating

| ITEM | SYMBOL | VALUE | UNIT |
| --- | --- | --- | --- |
| Power Supply | VCC | 28 | V |
| Operating Temp. | TOPR | -10~+55 | ℃ |
| Storage Temp. | TSTR | -20~+75 | ℃ |

- Electrical and Mechanical Specification

Under TA＝25℃，VCC＝+9V unless specified

| ITEM | SYMBOL | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| Power Supply | VCC | 8 | 9 | 24 | V |
| Current Dissipation | IC |  | 600 | 1500 | mA |
| Frequency | FREQ | 902 |  | 928 | MHz |

# Interface

1．Cable of OUTPUTA

| Item | Comment |
| --- | --- |
| 1 | +9VDC power interface |
| 2 | RS232 DB9 interface |
| 3 | TCPIP RJ45 interface |

2．Cable of OUTPUTB

| Item | Symbol | Comment |
| --- | --- | --- |
| Black | TGIN | Trigger input (TTL level) |
| Brown | VOUT+ | +9~+24V output (directly connected to power supply) |
| Red | WD0 | Wiegand DATA0 |
| Orange | CM_RLY2 | Relay2 common node |
| Yellow | NC_RLY1 | Relay1 normal-close node |
| Green | NO_RLY1 | Relay1 normal-open node |
| Blue | R+ | RS485 R+ |
| Purple | OUTPUT0 | external indicator control |
| Red Black | GND | GND |
| Yellow Black | WD1 | Wiegand DATA1 |
| Pink | NC_RLY2 | Relay2 normal-close node |
| Grey | NO_RLY2 | Relay2 normal-open node |
| White | CM_RLY1 | Relay1 common node |
| Light Green | R- | RS485 R- |

3．TNC external antenna port