What is EAS?

EAS (Electronic Article Surveillance), also known as Electronic Article Surveillance (Pirates) system is one of the major commodity security measures currently widely used in the retail industry. EAS in the United States came in the mid-1960s, first used in the apparel industry, has now been extended to more than 80 countries and regions worldwide, applications are extended to department stores, supermarkets, book a variety of industries, especially in large supermarkets (storage) of been fully developed.

EAS works: Set the detector at the outlet channel or cashier at the supermarket when the cashier has not been treated by the EAS tag detection area may cause interference, the receiver detects this interference, it will trigger the audio alarm.

EAS market mainstream technology used in two ways: one is the radio frequency technology, one is accustom-magnetic technology. Global markets using radio frequency technology EAS systems accounted for about 52%, the use of accustom-magnetic EAS technology systems account for about 30%.

Here is the setting of our reader:

Set the EAS alarm by usb desktop reader

Connect the Desktop reader

The affect setting of kill password and access password for EAS alarm：

| kill password | access password | result |
| --- | --- | --- |
| 00000000 | 00000000 | set EASAlarm fail |
| 11112222 | 00000000 | set EASAlarm fail |
| 11112222 | 22223333 | set EASAlarm successful |
| 00000000 | 22223333 | set EASAlarm successful |

The conclusion is：Not related with kill password，access password to set as 00000000.

To make sure the access password is 00000000 before encoding the RFID tag. Otherwise is not able to encode the tag.

Set prefix，make the first 20 HEX are same, and last HEX of EPC will be serial-number. Make sure all EPC different.

After set ready prefix、serial no.、step、number，click“data generate”to get EPC data.

Input an initial serial number to last HEX of EPC.

Step: add number for serial number.

Number: generate how many EPC.

click“Begin writing tag”，start to rewrite EPC for tag one by one. If the tag rewrite successfully, status prompt will be Green, and EPC will be listed in the frame“List of completing data”.

Mark  “Use password to write ” and“EAS Alarm”,To encode the Access pasword and set the EAS alarm.After that to change the new access password

To put one tag with EAS function on the Desktop reader and Read this tag by the demo software.

To read the tag

Switch To

“ISO18000-6C Read” Windows

Stop reading tag

To stop reading tag

To active the EAS status.First to set“No Alarm”

1

2

3

Verify "No Alarm" state, will not find any change in the demo or status of desktop reader.

To test the status of the tag

To select the same tag and set EAS status in“Alarm”

1

2

3

Varity“Alarm”status

The desktop reader will indicate with flashing light and warning voice.

Click here to check the EAS status

The read point is flashing contiuely