VD-67E

RFID Desktop Reader

Directory of Product Information

Content

1	Product Summarize	3

1.1	SHAPE And Interface	3

1.2	Performance Index	3

1.3	Function	4

1.4	PACKING LIST	4

2	Usual Faults	5

3	After-Sale Service	6

3. 1      SERVICE NOTES………………………………………………………………………….

3.1	Support And Service	6

3.2	Service Hotline	8

- Product Summarize

VD-67E Desktop reader is the latest product developed by us, which can read and write tags in accordance with ISO18000-6B/ISO180006C. It can be used to write tags on access control and other data acquisition system.

  - Shape And Interface

VD-67E USB desktop reader as the picture showed:

Dimension: 139 (L)* 86 (W)* 22 (H) mm.

VD-67E has a Micro USB communication interface.

  - Performance Index

- Read & write tag protocol: ISO18000-6B/ ISO18000-6C

- Operating frequency: 860～960MHz(customized)

- Way of Working:by FHSS or fixed frequency launch

- Output power: ≤27dBm (Adjustable)

- Readdistance:read distance>5cm,write distance>3cm( antenna and tag dependent)

- Read rate:32bits/word,6ms in time,average

- Write rate:32bits/word,50ms in time ,average

- Power supply:+5v direct current

- Power:<5w ,in average

- Working temperature: -10℃~+55℃

  - Function

- Read&Write Tag protocol : ISO18000-6B、ISO18000-6C

- Can read EPC tags in different bit.(16,32,48,64,80 or 96)

- Can read EPC Data.

- Can read data from user area.

- Can read data from TID area

- Can visit tag and kill password(32bits)

- Can write different bit length EPC code(16,32,48,64,80 and 96)

- Can write data from user area

- Can revise tag visit and kill code(32bits)

- EPC,TID and user area can write protection

- Password section can write protection

- Can kill tag

  - Packing list

- VD-67E desktop reader     1Pc

- MicroUSB cable          1Pc

- Warranty card           1pc

- Usual Faults

Table 3-1 usual faults and solutions for reader

| Fault Phenomena | Possible Reason | Solution |
| --- | --- | --- |
| can not read card | Beyond reading range of module | make card close to module antenna |
|  | Antenna is broken | Change new antenna |
|  | Parameter of RF Power is too low | Renew parameter |
|  | Tag is broken | Test in other tag |
|  | Reader is broken | Contact technician |
| Can not be connected with reader | No power | Check USB connector |
|  | USB Cable broken or still not get connect with reader | Using others USB Cable or Check it connection with PC |
|  | Reader is broken | Contact technician |
| Reading distance is too near | Parameter value of RF Power is too low | Reset factory parameter |
|  | tag is not in accordance with antenna polarization | Revolve tag in 90degree |
|  | Reader is broken | Contact technician |
|  | Tag is broken | Change a new tag |

- After-Sales Service

- 3.1 Service Notes

- 3.2 Support And Service

Website please be online with internet by computer; enter http://www.vanch.net  to get specifications, purchasing devices and other information about VD-67E.

Customer support  If you have doubts about instruction and can not get solutions from our website,please contact our customers’ service centre.If your device has to be repaired,please contact us for service information,delivery instruction and maintenance cost excludes from after service before it is sent out

Service If device needs to be diagnosed or other service,please contact our customers’ service centre.please don’t send it out before you contact us.If your device is under guarantee,please prepare receipt of purchasing.

Please do as follows before you contact us:

- read introduction

- prepare your machine

  - model

  - serial number

  - your proof of purchase date

- Prepare your machine, customer service representative maybe ask you to operate machine.

- Prepare your questions; detailed problem description can help customer service representative solve your problem at the earliest.

Warranty

Important information this text is a guarantee declaration for hardware products, please read carefully.

One year guarantee for hardware

Shenzhen Vanch Intelligent Technology Co., Ltd offers this hardware product guarantee for no faults in material and manufacture from receipt date of purchasing.

If Shenzhen Vanch Intelligence Technology Co., Ltd receives fault advise as above during Guarantee date.Shenzhen Vanch Intelligent Technology Company. Ltd can decide whether to repair or replace the proved failure product.

If Shenzhen Vanch Intelligent Technology Co., Ltd can not repair or replace the product within reasonable time, customer can ask for refundment.

Scope of warranty

Guarantee above excludes from faults caused as follows:

Misapplication,modification without authorization,open the hardware,operates or stores up this product in an improper environment,damages during transportation,improper maintaince,or caused by not using software ,parts,media,article,expendable from Shenzhen Vanch Intelligent Technology Co.,Ltd,or project not for this product.

This is the only guarantee declaration for products from Shenzhen Vanch Intelligent Technology Co., Ltd, noelse guarantee in writing or oral

Shenzhen Vanch Intelligent Technology will not be responsible for cases as follows: direct, indirect, particular and accidental damage, including profit, whatever it is based on guarantee declaration, agreement, defect, or any other law theory.

Disclaimer clauses:Radio transmission equipment can make interference to the task of any unprotected industry,medical and scientific equipment.There is possible interference to other electronic equipment when this equipment works.If any inquiries in this aspect,please consult from Shenzhen Vanch Intelligent Technology Co.,Ltd for improvement.

  - Service Hotline

Thank you for choosing our product, we will support you with technology about product application, and offer solutions to any difficulties.Contact mode of our company are as follows:

Company: Shenzhen Vanch Intellifent Technology Co., Ltd

Address: 5F, 535, BaGua 6 st., FuTian district, Shenzhen, Guangdong, China

Postal code: 518029

Tel: +86 755 82426775

Fax: +86 755 82403457

Website: www.vanch.net

Email: info@vanch.net