High Performance UHF RFID Fixed Reader

Model：ZK-RFID406

Size：310mmx210mmx41mm

GENERAL DESCRIPTION

ZK-RFID406 is a high performance UHF RFID fixed reader. It is designed upon fully self-intellectual property. Based on proprietory efficient digital signal processing algorithm, it supports fast tag read/write operation with high identification rate. It can be widely applied in many RFID application systems such as logistics, access control, anti-counterfeit and industrial production process control system.

FEATURES

- Self-intellectual property;

- Designed with Impinj R2000 RFID engine;

- Support ISO18000-6C(EPC C1G2) , ISO18000-6B protocol tag;

- 860~868MHz/902~928MHz frequency band(frequency customization optional);

- FHSS or Fix Frequency transmission;

- RF output power up to 30dbm(adjustable);

- 4 TNC antenna port with antenna auto-tuning and failure-detection;

- Support auto-running, answer, trigger and real-time-inventory work mode;

- Tag buffer size up to 800PCS (Max. 496bits EPC length);

- Support EPC and TID inventory;

- Low power dissipation with single +9 DC power supply;

- Support RS232, RS485, RJ45(TCPIP) with WiFi and POE optional;

- High reliability design;

- Support on-the-site firmware upgrading and optional Wince or Linux Operation System.

CHARACTERISTICS

- Absolute Maximum Rating

| ITEM | SYMBOL | VALUE | UNIT |
| --- | --- | --- | --- |
| Power Supply | VCC | 16 | V |
| Operating Temp. | TOPR | -10~+60 | ℃ |
| Storage Temp. | TSTR | -20~+75 | ℃ |

- Electrical and Mechanical Specification

Under TA＝25℃，VCC＝+9V unless specified

| ITEM | SYMBOL | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| Power Supply | VCC | 8 | 9 | 12 | V |
| Current Dissipation | IC |  | 800 | 1500 | mA |
| Frequency | FREQ | 902 | 902~928 | 928 | MHz |
| Size | Size |  | 310×210×41 |  | mm |

MECHANICAL DATA(UNIT mm)：

INTERFACE

1．Power J1

| No. | Symbol | Comment |
| --- | --- | --- |
| 2 | PWR | +9VDC |
| 4 | GND | Ground |
| 1,3 | NC | Reserved |

2．WiFi Antenna J2 (SMA Female)

3.  USB J3 (Slave)

4.  USB J4 (Reserved)

5.  TCPIP RJ45 Socket J5

6.  UART J6 (RS232 DB9 Female)

| No. | Symbol | Comment |
| --- | --- | --- |
| 1 | NC | Reserved |
| 2 | TXD | Data output in RS232 |
| 3 | RXD | Data input in RS232 |
| 4 | NC | Reserved |
| 5 | GND | Ground |
| 6 | NC | Reserved |
| 7 | NC | Reserved |
| 8 | NC | Reserved |
| 9 | NC | Reserved |

7.  GPIO J7 (DB15 Female)

| No. | Symbol | Comment |
| --- | --- | --- |
| 1 | Output1 | General output1 |
| 2 | Output2 | General output2 |
| 3 | Output3 | General output3 |
| 4 | Output4 | General output4 |
| 5 | Input1 | General Intput1 with Internal 47k resistor pulled-down to ground |
| 6 | Input2 | General Intput2 with Internal 47k resistor pulled-down to ground |
| 7 | Input3 | General Intput3 with Internal 47k resistor pulled-down to ground |
| 8 | Input4 | General Intput4 with Internal 47k resistor pulled-down to ground |
| 9 | TGIN | Trigger input with internal 10k resistor pulled-up to +5V |
| 10 | R+ | R+ in RS485 |
| 11 | R- | R- in RS485 |
| 12 | GND | Ground |
| 13 | NC | Normal-Close terminal of internal relay |
| 14 | NO | Normal-Open terminal of internal relay |
| 15 | CM | Common terminal of internal relay |

8．TNC Antenna Port1~Port4 J8~J11

9．LED indicator

| Symbol | Comment |
| --- | --- |
| POWER | Power on indicator |
| RUN | Command-executing indicator |
| TAG | Tag-detected indicator |
| WIRELESS | WiFi active indicator |
| ALARM | Alarm indicator |
| 1~9 | RSSI indicator |

Remark:

1. Specifications are subject to change, please pay attention to our latest version.

2. Shenzhen RoyalRay Science and Technology Co., Ltd. reserves the right to the final interpretation of the above terms.