UHF High Performance Fixed Reader

Model：UHFReader288MP

Size：268/240 x 181 x 28 mm

Weight: 1180g

GENERAL DESCRIPTION

UHF High Performance Fixed Reader UHFReader288MP is designed upon fully self-intellectual property. Based on proprietary efficient digital signal processing algorithm, it supports fast tag read/write operation with high identification rate. RRU2882LITE can be widely applied in many RFID application systems such as logistics, access control, anti-counterfeit and industrial production process control system.

FEATURES

- Self-intellectual property;

- 840~960MHz frequency band (frequency customization optional);

- Based on Impinj R2000 high performance RF engine, excellent multi-tag anti-collision operation, fully support EPC CLASS1 G2 \ ISO18000-6B protocol tags

- FHSS or Fix Frequency transmission, support RSSI,  Maximum inventory speed over 700pcs;

- RF output power up to 33dbm (adjustable);

- Support 4 TNC antenna port with antenna auto-tuning and failure-detection;

- Support answer and real-time-inventory work mode;

- Tag buffer: 1000pcs@96bitsEPC;

- Support EPC and TID anti-collision mode

- Low power dissipation with single +9 DC power supply, POE (Power over Ethernet) is optional;

- Support RS232, USB(Slave), RJ45 (TCP/IP) interface;

- Provide DLL and Demonstration Software Source code to facilitate further development;

- High reliability design, meet the requirements of harsh working environment.

CHARACTERISTICS

- Absolute Maximum Rating

| ITEM | SYMBOL | VALUE | UNIT |
| --- | --- | --- | --- |
| Power Supply | VCC | 16 | V |
| Operating Temp. | TOPR | -20~+55 | ℃ |
| Storage Temp. | TSTR | -20~+85 | ℃ |

- Electrical and Mechanical Specification

Under TA＝25℃，VCC＝+9V unless specified

| ITEM | SYMBOL | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| Power Supply | VCC | 8 | 9 | 12 | V |
| Current Dissipation | IC |  | 0.5 | 1.2 | A |
| Frequency | FREQ | 840 | 860~868 902~928 | 960 | MHz |
| Size | L x W x H |  | 268 x181 x28 |  | mm |

INTERFACE

1．Power (DC JACK)

| No. | Symbol | Comment |
| --- | --- | --- |
| Central | PWR | +9VDC |
| Outer | GND | Ground |

2．USB

3.  UART （RS232 DB9 Female）

| No. | Symbol | Comment |
| --- | --- | --- |
| 1 | NC | Reserved |
| 2 | TXD | Data output in RS232 |
| 3 | RXD | Data input in RS232 |
| 4 | NC | Reserved |
| 5 | GND | Ground |
| 6 | NC | Reserved |
| 7 | NC | Reserved |
| 8 | NC | Reserved |
| 9 | NC | Reserved |

4．GPIO （DB15 Female）

| No. | Symbol | Comment |
| --- | --- | --- |
| 1 | NC | Reserved |
| 2 | NC | Reserved |
| 3 | Input1－ | General OPTO-coupler isolated input - |
| 4 | Input2－ | General OPTO-coupler isolated input - |
| 5 | Output1 | General OPTO-coupler isolated Output1 |
| 6 | Output1 | General OPTO-coupler isolated Output1 |
| 7 | Output2 | General OPTO-coupler isolated Output2 |
| 8 | Output2 | General OPTO-coupler isolated Output2 |
| 9 | Input1＋ | General OPTO-coupler isolated input+ with internal pull-up to 3.3V through a 1k resistor |
| 10 | Input2＋ | General OPTO-coupler isolated input+ with internal pull-up to 3.3V through a 1k resistor |
| 11 | NC | Reserved |
| 12 | GND | Ground |
| 13 | NC | Reserved |
| 14 | NC | Reserved |
| 15 | NC | Reserved |

5．TCPIP network (RJ45)

6．TNC antenna port ANT1~ANT4