com_rfid_uhf_Device.DLL

Dynamic link library manual

V1.0

# Content

Content	2

1	Introduction	3

2	SDK summary	4

3	Programming interface	5

3.1	Connect/disconnect reader	5

3.1.1	OpenComPort	5

3.1.2	CloseSpecComPort	6

3.1.3	OpenNetPort	7

3.1.4	CloseNetPort	8

3.2	18000-6C commands	9

3.2.1	Inventory_G2	9

3.2.2	InventoryMix_G2	12

3.2.3	ReadData_G2	15

3.2.4	WriteData_G2	17

3.2.5	WriteEPC_G2	19

3.2.6	KillTag _G2	20

3.2.7	Lock_G2	21

3.2.8	BlockErase_G2	23

3.3	Reader customised commands	24

3.3.1	GetReaderInformation	24

3.3.2	SetAddress	26

3.3.3	SetInventoryScanTime	27

3.3.4	SetRfPower	28

3.3.5	SetRegion	29

3.3.6	SetBaudRate	31

3.3.7	SetAntennaMultiplexing	32

3.3.8	WriteRfPower	33

3.3.9	ReadRfPower	34

3.3.10	RetryTimes	35

3.3.11	SetReadMode	36

3.3.12	SetCheckAnt	37

3.3.13	GetSeriaNo	38

3.3.14	SetBeepNotification	39

3.3.15	SetDRM	40

3.3.16	GetDRM	41

3.3.17	GetReaderTemperature	42

3.3.18	MeasureReturnLoss	43

Appendix 1: Reader Error code table	44

Appendix 2: Tag ErrorCode table	45

# 1	Introduction

This SDK is designed for java development, supports both 32-bit and 64-bit WINDOWS operation system.

By loading the "com_rfid_uhf_Device.DLL", host application can control the 18000-6C format R2000 readers. Host application can access multiple readers via different communication ports simultaneously.

# 2	SDK summary

This SDK includes the following contents:

| Files | Description |
| --- | --- |
| com_rfid_uhf_Device.dll | Dynamic Link Library (DLL), included all the API interface. |
| UHFREADER289.dll | Dynamic Link Library (DLL), can be called using "com_rfid_uhf_Device.dll" |
| Device.java | Java project source files |

| Programming language | Description |
| --- | --- |
| Java | Copy "com_rfid_uhf_Device.dll" and "UHFREADER289.dll" to java project directory or to C:\WINDOWS\system32. |

# 3	Programming interface

## 3.1	Connect/disconnect reader

3.1.1	OpenComPort

| Definition | public native int OpenComPort(int port, byte[]comAddr, byte baud, int[]PortHandle); |  |  |
| --- | --- | --- | --- |
| Description | Open the serial connection to reader |  |  |
| Parameters | Name | Type | Notes |
|  | port | Int | Serial port number for reader connection. 1 - COM1; 2 - COM2, etc. |
|  | ComAdr | byte[] | Target reader address. In the case when reader address is unknown, the broadcasting address 0xFF can be used to initialise the serial connection. Once the function is called successfully, the actual address of the reader will be returned as the value of this parameter. |
|  | baud | byte | Serial baud rate. 0 - 9600 bps; 1 - 19200 bps; 2 - 38400 bps; 5 - 57600 bps; 6 - 115200 bps. |
|  | PortHandle | Int[] | Reader connected handle will be returned as the value of this parameters. The value of handle will be used in all the future API operation. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |
| Sample code | int Port = 1;//com1 byte[]comAddr=new byte[1]; comAddr[0]=(byte)255; byte baud=5;//57600bps int[] PortHandle= new int[1]; int result = OpenComPort(Port, comAddr, baud, PortHandle); if(result==0) { // Reader is connected successfully …… } |  |  |

3.1.2	CloseSpecComPort

| Definition | public native int CloseSpecComPort(int PortHandle); |  |  |
| --- | --- | --- | --- |
| Description | Close a specific port to disconnect with reader and release associated resource. |  |  |
| Parameters | Name | Type | Notes |
|  | PortHandle | int | Reader connected handle, obtained from the OpenComPort function. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to error code table) . |  |  |
| Sample code | N/A |  |  |

3.1.3	OpenNetPort

| Definition | public native int OpenNetPort(int Port, String IPaddr, byte[]comAddr, int[] PortHandle); |  |  |
| --- | --- | --- | --- |
| Description | Open the TCP connection to reader |  |  |
| Parameters | Name | Type | Notes |
|  | port | int | TCP port number for reader connection. |
|  | IPaddr | String | IP address of the reader. |
|  | ComAdr | byte[] | Target reader address, or simply use broadcasting address 0xff for TCP connection. |
|  | PortHandle | int[] | Reader connected handle will be returned as the value of this parameters. The value of handle will be used in all the future API operation. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |
| Sample code | int Port = 27011; String ipAddr=”192.168.0.250”; byte[]comAddr=new byte[1]; comAddr[0]=(byte)255; int[] PortHandle= new int[1]; int result = OpenNetPort(Port, ipAddr,comAddr, PortHandle); if(result==0) { // Reader is connected successfully …… } |  |  |

3.1.4	CloseNetPort

| Definition | public native int CloseNetPort(int PortHandle); |  |  |
| --- | --- | --- | --- |
| Description | Close the TCP connection with reader and release associated resource |  |  |
| Parameters | Name | Type | Notes |
|  | PortHandle | int | Reader connected handle, obtained from the OpenNetPort function. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |
| Sample code | N/A |  |  |

## 3.2	18000-6C commands

3.2.1	Inventory_G2

| Definition | public native int Inventory_G2(byte[]comAddr, byte QValue, byte Session, byte MaskMem, byte[]MaskAdr, byte MaskLen, byte[]MaskData, byte MaskFlag, byte AdrTID, byte LenTID, byte TIDFlag, byte Target, byte InAnt, byte Scantime, byte FastFlag, byte[]pEPCList, byte[] Ant, int[]Totallen, int[]CardNum, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Inquire RFID tags with corresponding protocol within the effective field. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | QValue | byte | [in] | 1 byte. bit7: Statistic data packet flag; 0 – after inventory，DO NOT deliver statistic data packet of inventory process; 1 – after inventory，deliver statistic data packet of inventory process. bit5: FastID inventory indicator. This function is used to rapidly obtain tag EPC and TID, only supported by some tags from the Impinj Monza series; 0 – disable; 1 – enable. bit4 ~ bit0: the original Q-value of EPC tag inventory. NOTE: 1. The setting of Q-value should follow the rule: 2Q ≈ total amount of tags within the effective field. The range of Q-value is 0 ~ 15, if other value is delivered in this field, reader will return a parameter error status in the response frame. 2. Inventory with statistic data packet is SLOWER than inventory without statistic data packet. |
|  | Session | byte | [in] | 1 byte, the Session-value of EPC tag inventory. 0x00 – apply S0 as Session value; 0x01 – apply S1 as Session value; 0x02 – apply S2 as Session value; 0x03 – apply S3 as Session value. 0xff – apply reader smart configuration (only valid in EPC inventory). Inventory for single tag or small amount of tag, S0 is the recommended setting. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte [] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte [] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | MaskFlag | byte | [in] | 1 byte, mask flag. 0 – disable mask; 1 – enable mask. |
|  | AdrTID | byte | [in] | 1 byte, entry address of TID memory inventory. |
|  | LenTID | byte | [in] | 1 byte, data length for TID inventory operation, the valid range of LenTID is 0 ~ 15. |
|  | TIDFlag | byte | [in] | 1 byte, inventory purpose indicator. 0 – EPC inventory; 1 – TID inventory. |
|  | Target | byte | [in] | 1 byte, the Target value of EPC tag inventory. 0x00 – apply Target A; 0x01 – apply Target B. |
|  | InAnt | byte | [in] | 1 byte, antenna selection for the current inventory. 0x80 – antenna 1; 0x81 – antenna 2; 0x82 – antenna 3; 0x83 – antenna 4； 0x84 – antenna 5; 0x85 – antenna 6; 0x86 – antenna 7; 0x87 – antenna 8. InAnt is 0x80 for single port reader. |
|  | Scantime | byte | [in] | 1 byte, the maximum operation time for inventory. The valid range of Scantime is 0 ~ 255, corresponding to (0 ~ 255)*100ms. For Scantime = 0, operation time is not limited. |
|  | FastFlag | byte | [in] | 1 byte, express inventory indicator 0 – disable express inventory, Target, InAnt and Scantime are NOT essential, can be set to default value 0; 1 – enable express inventory, Target, InAnt and Scantime are need to be defined. |
|  | pEPCList | byte [] | [out] | The inquired tag data, the data block follows the format stated below: EPC/TID length + EPC/TID No. + RSSI. Data of multiple tags is formed by several identical data blocks in sequence. |
|  | Ant | byte [] | [out] | 1 byte indicates which antenna had inquired a tag. 8 antenna ports reader: Every bit in Ant represents one corresponding antenna. For example, 0x04 is 0000 0100 in binary. This indicates Antenna 3 had inquired this specific tag. |
|  | Totallen | int[] | [out] | The total length of the received data stored in pEPCList. |
|  | CardNum | int[] | [out] | The total amount of tag inquired during the current inventory. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | 0x01: inventory completed, tag data will be delivered; 0x02: inventory timeout, operation is aborted, inquired tag data will be delivered. 0xF8: error occurred in antenna check, the target antenna might be disconnected. For description on other error, refer to reader error code table. |  |  |  |
| Sample code | N/A |  |  |  |

3.2.2	InventoryMix_G2

| Definition | public native int Inventory_G2(byte[]comAddr, byte QValue, byte Session, byte MaskMem, byte[]MaskAdr, byte MaskLen, byte[]MaskData, byte MaskFlag, byte ReadMem, byte[] ReadAdr, byte ReadLen, byte[]Psd, byte Target, byte InAnt, byte Scantime, byte FastFlag, byte[]pEPCList, byte[] Ant, int[]Totallen, int[]CardNum, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Inquire RFID tags with corresponding protocol within the effective field, and obtain specific tag data once EPC number is inquired. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | QValue | byte | [in] | 1 byte. bit7: Statistic data packet flag. 0 – after inventory，DO NOT deliver statistic data packet of inventory process; 1 – after inventory，deliver statistic data packet of inventory process. bit6 ~ bit0: the original Q-value of EPC tag inventory. NOTE: 1. The setting of Q-value should follow the rule: 2Q ≈ total amount of tags within the effective field. The range of Q-value is 0 ~ 15, if other value is delivered in this field, reader will return a parameter error status in the response frame. 2. Inventory with statistic data packet is SLOWER than inventory without statistic data packet. |
|  | Session | byte | [in] | 1 byte, the Session-value of inventory. 0x00 – apply S0 as Session value; 0x01 – apply S1 as Session value; 0x02 – apply S2 as Session value; 0x03 – apply S3 as Session value. Inventory for single tag or small amount of tag, S0 is the recommended setting. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte [] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte [] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | MaskFlag | byte | [in] | 1 byte, mask flag. 0 – disable mask; 1 – enable mask. |
|  | ReadMem | byte | [in] | 1 byte, the type of target memory. 0x00 – reserved memory; 0x01 – EPC memory; 0x02 – TID memory; 0x03 – user memory. All other values are reserved, reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | ReadAdr | byte [] | [in] | 2 bytes, the initial word address of target, most- significant byte first. |
|  | ReadLen | byte | [in] | 1 byte, the amount of word to be read in this operation. |
|  | Psd | byte [] | [in] | 4 bytes, the access password. |
|  | Target | byte | [in] | 1 byte, the Target value of tag inventory. 0x00 – apply Target A; 0x01 – apply Target B. |
|  | InAnt | byte | [in] | 1 byte, antenna selection for the current inventory. 0x80 – antenna 1; 0x81 – antenna 2; 0x82 – antenna 3; 0x83 – antenna 4； 0x84 – antenna 5; 0x85 – antenna 6; 0x86 – antenna 7; 0x87 – antenna 8. InAnt is 0x80 for single port reader. |
|  | Scantime | byte | [in] | 1 byte, the maximum operation time for inventory. The valid range of Scantime is 0 ~ 255, corresponding to (0 ~ 255)*100ms. For Scantime = 0, operation time is not limited. |
|  | FastFlag | byte | [in] | 1 byte, express inventory indicator 0 – disable express inventory, Target, InAnt and Scantime are NOT essential, can be set to default value 0. 1 – enable express inventory, Target, InAnt and Scantime are need to be defined. |
|  | pEPCList | byte [] | [out] | The inquired tag data, the data block follows the format stated below: Rolling code + EPC length/data length + EPC No. /data + RSSI. Data of multiple tags is formed by several identical data blocks in sequence. The most-significant bit of rolling code: 0 – this block contains EPC; 1 – this block contains tag memory data. Firstly EPC will be uploaded, followed by tag memory data. If two sets of lower 7 bits of rolling code are consecutive, the corresponding data comes from the same tag. The lower 7 bits of rolling code ranges from 0 to 127. The code will increase for every block uploading and will recount again from 0 after it exceeds 127. |
|  | Ant | byte [] | [out] | 1 byte indicates which antenna had inquired a tag. 8 antenna ports reader: Every bit in Ant represents one corresponding antenna. For example, 0x04 is 0000 0100 in binary. This indicates Antenna 3 had inquired this specific tag. |
|  | Totallen | int[] | [out] | The total length of the received data stored in pEPCList. |
|  | CardNum | int[] | [out] | The total amount of tag inquired during the current inventory. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | 0x01: inventory completed, tag data will be delivered; 0x02: inventory timeout, operation is aborted, inquired tag data will be delivered. 0xF8: error occurred in antenna check, the target antenna might be disconnected. For description on other error, refer to reader error code table. |  |  |  |
| Sample code | N/A |  |  |  |

3.2.3	ReadData_G2

| Definition | public native int ReadData_G2(byte[] comAddr, byte[] EPC ,byte ENum, byte Mem, byte WordPtr, byte Num, byte[] Password, byte MaskMem, byte[] MaskAdr, byte MaskLen, byte[] MaskData, byte[] Data, int[] Errorcode, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Read a part of data or the whole memory area from tag reserved memory, EPC memory, TID memory, user memory. Read operation starts from a specific address. The unit of data is word. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | byte [] | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Mem | byte | [in] | 1 byte, the type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | WordPtr | byte | [in] | 1 byte, the initial word address of target. |
|  | Num | byte | [in] | 1 byte, the amount of words to be read in this operation. |
|  | Password | byte [] | [in] | 4 bytes, the access password. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte [] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte [] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | Data | byte [] | [out] | Inquired tag data, with a length stated in Num. |
|  | ErrorCode | int [] | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.4	WriteData_G2

| Definition | public native int WriteData_G2(byte[]comAddr, byte[]EPC, byte WNum, byte ENum, byte Mem, byte WordPtr, byte[] Writedata, byte[] Password, byte MaskMem, byte[]MaskAdr, byte MaskLen, byte[]MaskData, int[]Errorcode, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Write multiple words to reserved memory, EPC memory, TID memory or user memory in one communication cycle. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | byte [] | [in] | Tag EPC number, with a length stated in Enum. |
|  | Wnum | byte | [in] | 1 byte, the amount of words to be written. For ENum = 255, mask will be enabled. |
|  | Enum | byte | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Mem | byte | [in] | 1 byte, the type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | WordPtr | byte | [in] | 1 byte, the initial word address of target. |
|  | Writedata | byte [] | [in] | Data to be written to tag, with a length stated in WNum. |
|  | Password | byte [] | [in] | 4 bytes, the access password. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte [] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte [] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | int [] | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.5	WriteEPC_G2

| Definition | public native int WriteEPC_G2(byte[]comAddr, byte[] Password, byte[] EPC, byte ENum, int[]Errorcode, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Write EPC number to a tag. During the operation, only one tag is allowed to be place in the antenna effective area. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | Password | byte [] | [in] | 4 bytes, the access password. |
|  | EPC | byte [] | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number. |
|  | ErrorCode | int[] | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.6	KillTag _G2

| Definition | public native int KillTag_G2(byte[]comAddr, byte[] EPC, byte ENum, byte[] Killpwd, byte MaskMem, byte[]MaskAdr, byte MaskLen, byte[]MaskData, int[]Errorcode, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Tag killing command. After the kill operation, the tags will no longer process any command from reader. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | byte [] | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Killpwd | byte [] | [in] | 4 bytes, the kill password. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte [] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte [] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | int [] | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.7	Lock_G2

| Definition | public native int Lock_G2(byte[] comAddr, byte[] EPC, byte ENum, byte select, byte setprotect, byte[] Password, byte MaskMem, byte[]MaskAdr, byte MaskLen, byte[]MaskData, int[] Errorcode, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | The read/write protection status for the following memories: • Reserved memory Readable/ writable without protection, permanently readable/ writable, readable/writeable with password protected or permanently unreadable/ unwritable; • EPC memory /user reserved memory Writable without protection, permanently writable, writeable with password protected or permanently unwritable; EPC memory, user reserved memory and TID memory are permanently readable. Furthermore TID memory is readable only, permanently unwritable. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | byte [] | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | select | byte | [in] | 1 byte. 0x00 – modify kill password read/write protection; 0x01 – modify access password read/write protection; 0x02 – modify EPC memory read/write protection; 0x03 – modify TID memory read/write protection; 0x04 – modify user memory read/write protection. All other values are reserved, reader will not execute the command and will return a parameter error status in the response frame if other value is delivered in this field. |
|  | setprotect | byte | [in] | 1 byte. For Select = 0x00 or 0x01: the kill password or access password protection setting. SetProtect = 0x00 – set to readable/ writable without protection; 0x01 – set to permanently readable/ writable; 0x02 – set to readable/writeable with password protected; 0x03 – set to permanently unreadable/ unwritable. For Select = 0x02, 0x03, and 0x04: the EPC, TID and user memory protection setting. SetProtect = 0x00 – set to writable without protection; 0x01 – set to permanently writable; 0x02 – set to writeable with password protected; 0x03 – set to permanently unwritable. All other values of Select and SetProtect are reserved, reader will not execute the command and will return a parameter error status in the response frame if other value is delivered in this field. |
|  | Password | byte [] | [in] | 4 bytes, the access password. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte [] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte [] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | int [] | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.8	BlockErase_G2

| Definition | public native int BlockErase_G2(byte[]comAddr, byte[] EPC, byte ENum, byte Mem, byte WordPtr, byte Num, byte[] Password, byte MaskMem, byte[]MaskAdr, byte MaskLen, byte[]MaskData, int[]Errorcode, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Erase multiple words from reserved memory, EPC memory, TID memory or user memory. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | byte [] | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Mem | byte | [in] | 1 byte, the type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | WordPtr | byte [] | [in] | 1 byte, the initial word address of target. |
|  | Num | byte | [in] | 1 byte, the amount of words to be erased in this operation. |
|  | Password | byte [] | [in] | 4 bytes, the access password. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte [] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte [] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | int [] | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

## 3.3	Reader customised commands

3.3.1	GetReaderInformation

| Definition | public native int GetReaderInformation(byte[] comAddr, byte[] VersionInfo, byte[] readerType, byte[]dmaxfre, byte[] dminfre, byte[] powerdBm, byte[] ScanTime, byte[] Ant, byte[] BeepEn, byte[] OutputRep, byte[] CheckAnt, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain reader information, including reader address (ComAdr), firmware version (VersionInfo) and other reader information. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte[] | [in/out] | Reader address, obtained from open port commands. |
|  | VersionInfo | byte [] | [out] | 2 bytes, firmware version. The 1st byte: the main version number; The 2nd byte: the subversion number. |
|  | ReaderType | byte[] | [out] | 1 byte.Reader Type code. |
|  | dmaxfre | byte[] | [out] | 1 byte. bit7 ~ bit6: frequency band configuration; bit5 ~ bit0: current maximum frequency point. |
|  | dminfre | byte [] | [out] | 1 byte. bit7 ~ bit6: frequency band configuration; bit5 ~ bit0: current minimum frequency point. |
|  | powerdBm | byte [] | [out] | 1 byte, Output RF power. The range is 0 ~ 30 and the unit is dBm. |
|  | ScanTime | byte [] | [out] | 1 byte, inventory time. Reader will respond to the inventory command delivered from host within this specific inventory time. |
|  | Ant | byte [] | [out] | 1 byte, antenna configuration. |
|  | BeepEn | byte [] | [out] | 1 byte, buzzer status information. |
|  | OutputRep | byte [] | [out] | 1 byte, GPIO status for notification. |
|  | CheckAnt | byte [] | [out] | 1 byte, Antenna check configuration. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

Frequency band configuration table

| MaxFre (bit7) | MaxFre (bit6) | MinFre (bit7) | MinFre (bit6) | FreqBand |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 1 | 0 | Ukraine band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| 1 | 1 | 0 | 0 | US band3 |

3.3.2	SetAddress

| Definition | public native int SetAddress(byte[]comAddr, byte ComAdrData,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Reader will apply the user provided value as its reader address and write this value to EEPROM for preservation purpose. Default setting of reader address is 0x00, the valid value of the setting is 0x00 ~ 0x0FE. Reader address will be reset to 0x00, if the provided address value is 0xFF. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | ComAdrData | byte | [in] | 1 byte, the new address setting. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.3	SetInventoryScanTime

| Definition | public native int SetInventoryScanTime(byte[]comAddr, byte ScanTime,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify maximum operation time for inventory command. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | ScanTime | byte | [in] | 1 byte, operation time for inventory. The acceptable range of Scantime is 0 ~ 255, corresponding to (0 ~ 255)*100ms. For Scantime = 0, operation time is not limited until a whole reading cycle is completed. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.4	SetRfPower

| Definition | public native int SetRfPower(byte[]comAddr, byte PowerDbm,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify reader RF power |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | PowerDbm | byte | [in] | 1 byte, reader RF power. The range is 0 ~ 30 and the unit is dBm. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.5	SetRegion

| Definition | public native int SetRegion(byte[]comAddr, byte dmaxfre,byte dminfre,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Select maximum and minimum frequency. Maximum frequency must be larger or equal the minimum frequency. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte[] | [in/out] | Reader address, obtained from open port commands. |
|  | dmaxfre | byte | [in] | 1 byte. bit7 ~ bit6: frequency band configuration; bit5 ~ bit0: maximum frequency point. |
|  | dminfre | byte | [in] | 1 byte. bit7 ~ bit6: frequency band configuration; bit5 ~ bit0: minimum frequency point. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table) |  |  |  |
| Sample code | N/A |  |  |  |

Frequency band configuration table

| MaxFre (bit7) | MaxFre (bit6) | MinFre (bit7) | MinFre (bit6) | FreqBand |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 1 | 0 | Ukraine band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| 1 | 1 | 0 | 0 | US band3 |

Formulae for different frequency bands:

Chinese band2: Fs = 920.125 + N * 0.25 (MHz) where N∈ [0, 19]. US band: Fs = 902.75 + N * 0.5 (MHz) where N∈ [0, 49]. Korean band: Fs = 917.1 + N * 0.2 (MHz) where N∈ [0, 31]. EU band: Fs = 865.1 + N*0.2(MHz) where N∈ [0, 14]. Ukraine band: Fs = 868.0 + N*0.1(MHz) where N∈ [0, 6]. Chinese band1: Fs = 840.125 + N * 0.25 (MHz) where N∈ [0, 19]. US band3: Fs = 902 + N * 0.5 (MHz) where N∈ [0, 52].

3.3.6	SetBaudRate

| Definition | public native int SetBaudRate(byte[]comAddr, byte baud, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify serial baud rate. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | baud | byte | [in] | 1 byte, serial baud rate. 0 – 9600bps; 1 – 19200bps; 2 – 38400bps; 5 – 57600bps; 6 – 115200bps. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.7	SetAntennaMultiplexing

| Definition | public native int SetAntennaMultiplexing(byte[ ]comAddr, byte SetOnce,byte AntCfg1,byte AntCfg2,, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify antenna configuration of 4 ports reader. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte[] | [in/out] | Reader address, obtained from open port commands. |
|  | SetOnce | byte | [in] | Save information, 0-Save;1-Not save. |
|  | AntCfg1 | byte | [in] | Resverve. |
|  | AntCfg2 | byte | [in] | 1 byte, antenna configuration information. 1 antenna is the MINIMUM requirement for operation. bit0: antenna 1 configuration bit. 0 – disable antenna 1; 1 – enable antenna 1. bit1: antenna 2 configuration bit. 0 – disable antenna 2; 1 – enable antenna 2. bit2: antenna 3 configuration bit. 0 – disable antenna 3; 1 – enable antenna 3. bit3: antenna 4 configuration bit. 0 – disable antenna 4; 1 – enable antenna 4. bit4: antenna 5 configuration bit. 0 – disable antenna 5; 1 – enable antenna 5. bit5: antenna 6 configuration bit. 0 – disable antenna 6; 1 – enable antenna 6. bit6: antenna 7 configuration bit. 0 – disable antenna 7; 1 – enable antenna 7. bit7: antenna 8 configuration bit. 0 – disable antenna 8. 1 – enable antenna 8. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.8	WriteRfPower

| Definition | public native int WriteRfPower(byte[ ]comAddr, byte PowerDbm, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify RF power configuration separately for write operations |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte[] | [in/out] | Reader address, obtained from open port commands. |
|  | PowerDbm | byte | [in] | 1 byte, separate reader RF power for write operation. The range is 0 ~ 30 and the unit is dBm. bit7 = 0: disable; bit7 = 1: enable. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.9	ReadRfPower

| Definition | public native int ReadRfPower(byte[] comAddr, byte[] PowerDbm, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain RF power configuration for write operations |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | PowerDbm | byte [] | [out] | 1 byte, separate reader RF power for write operation. The range is 0 ~ 30 and the unit is dBm. bit7 = 0: disabled; bit7 = 1: enabled. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.10	RetryTimes

| Definition | public native int RetryTimes(byte[] comAddr, byte[] Times, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify or load the maximum operation retry time configuration |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | Times | byte [] | [out] | 1 byte. bit7 = 1: modify maximum write retry time; bit7 = 0: load maximum write retry time The valid range of configuration is 0 ~ 7. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.11	SetReadMode

| Definition | public native int SetReadMode(byte[] comAddr, byte ReadMode, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Select reader working mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte | [in/out] | Reader address, obtained from open port commands. |
|  | ReadMode | byte | [in] | 1 byte. 0x00 – answer mode; 0x01 – real time inventory mode; 0x02 – trigger mode. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.12	SetCheckAnt

| Definition | public native int SetCheckAnt(byte[] comAddr, byte CheckAnt, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Enable antenna check during tag reading/writing operations. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | CheckAnt | byte | [in] | 1 byte, antenna check switch. 0x00 – disable antenna check; 0x01 – enable antenna check. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.13	GetSeriaNo

| Definition | public native int GetSeriaNo(byte[] comAddr, byte[] SeriaNo, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain 4 bytes reader unique serial number. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte [] | [in/out] | Reader address, obtained from open port commands. |
|  | SeriaNo | byte [] | [out] | 4 bytes, the unique serial number of reader. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.14	SetBeepNotification

| Definition | public native int SetBeepNotification(byte[]comAddr, byte BeepEn, int PortHandle);; |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Enable/disable buzzer. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte[] | [in/out] | Reader address, obtained from open port commands. |
|  | BeepEn | byte | [in] | 1 byte. 0x00 – disable buzzer; 0x01 – enable buzzer. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.15	SetDRM

| Definition | public native int SetDRM(byte[] ComAddr, byte DRM, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify DRM status. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte[] | [in/out] | Reader address, obtained from open port commands. |
|  | DRM | byte | [in] | 1 byte, DRM status. 0x00 – disable; 0x01 – enable. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.16	GetDRM

| Definition | public native int GetDRM(byte[] ComAddr, byte[] DRM, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain DRM status. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte[] | [in/out] | Reader address, obtained from open port commands. |
|  | DRM | byte[] | [out] | 1 byte, DRM status. 0x00 – disabled; 0x01 – enabled. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.17	GetReaderTemperature

| Definition | public native int GetReaderTemperature (byte[] ComAddr, byte[] PlusMinus, byte[] Temperature, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain current reader temperature. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte[] | [in/out] | Reader address, obtained from open port commands. |
|  | PlusMinus | byte[] | [out] | 1 byte, temperature range. 0 – below 0 °c; 1 – above 0 °c. |
|  | Temperature | byte[] | [out] | 1 byte, temperature value, the unit is °c. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.18	MeasureReturnLoss

| Definition | public native int MeasureReturnLoss(byte[] ComAddr, byte[] TestFreq, byte Ant, byte[]ReturnLoss, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Measure antenna return loss. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | byte[] | [in/out] | Reader address, obtained from open port commands. |
|  | TestFreq | byte[] | [in] | 4 bytes, target frequency point. |
|  | Ant | byte | [in] | 1 byte, target antenna for this measurement 0 – antenna 1; 1 – antenna 2; 2 – antenna 3; 3 – antenna 4； 4 – antenna 5; 5 – antenna 6; 6 – antenna 7; 7 – antenna 8. |
|  | ReturnLoss | byte[] | [out] | 1 byte, result of the return loss measurement, the unit of ReturnLoss is dB. |
|  | PortHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

# Appendix 1: Reader Error code table

| Error code | Description |
| --- | --- |
| 0x00 | API is called successfully. |
| 0x01 | Tag inventory is completed successfully and reader is able to deliver data response within the predefined inventory time. |
| 0x02 | Inventory timeout. |
| 0x05 | Access password error. |
| 0x09 | Kill password error. |
| 0x0A | All-zero tag killing password is invalid. |
| 0x0B | Command is not support by the tag |
| 0x0C | All-zero tag access password is invalid for such command. |
| 0x0D | Fail to setup read protection for a protection enabled tag. |
| 0x0E | Fail to unlock a protection disabled tag. |
| 0x10 | Some bytes stored in the tag are locked. |
| 0x11 | Lock operation failed. |
| 0x12 | Already locked, lock operation failed. |
| 0x13 | Fail to store the value of some preserved parameters. Configuration will still valid before reader shut down. |
| 0x14 | Modification failed. |
| 0x15 | Response within the predefined inventory time. |
| 0x17 | Further data is waiting to be delivered. |
| 0x18 | Reader memory is full |
| 0x19 | All-zero access password is invalid for such command or command is not supported by the tag. |
| 0xF8 | Error detected in antenna check. |
| 0xF9 | Operation failed. |
| 0xFA | Tag is detected, but fails to complete operation due to poor communication. |
| 0xFB | No tag is detected. |
| 0xFC | Error code returned from tags. |
| 0xFD | Command length error. |
| 0xFE | Illegal command. |
| 0xFF | Parameter error. |
| 0x30 | Communication error. |
| 0x33 | Reader is busy, operation in process. |
| 0x35 | Port is already opened. |
| 0x37 | Invalid handle. |

# Appendix 2: Tag ErrorCode table

| ErrorCode | Description |
| --- | --- |
| 0x00 | Other errors, all other Errorcode non-specified error. |
| 0x03 | Memory overload, location is not found or unsupported PC value. |
| 0x04 | Memory is being temporarily / permanently locked, unable to perform write operation. |
| 0x0B | Unable to perform write operation due to insufficient power supply to tag. |
| 0x0F | Undefined or tag unsupported errors. |