UHF2000.ocx library

# Contents

Contents	2

1	Develop guide	1

1.1	Widget registration	1

1.2	Connect / disconnect reader	1

1.3	Inquired tag data format	1

2	Programming interface	3

2.1	Connect/disconnect reader	3

2.1.1	RFID_ComOpen	3

2.1.2	RFID_OpenTcp	4

2.1.3	RFID_CloseCom	5

2.1.4	RFID_CloseTcp	6

2.2	18000-6C commands	7

2.2.1	RFID_Inventory	7

2.2.2	RFID_Read	8

2.2.3	RFID_Write	9

2.2.4	RFID_WriteEPC	10

2.3	Reader customised commands	11

2.3.1	RFID_GetSeriaNo	11

2.3.2	RFID_SetRegion.	12

2.3.3	RFID_SetRfPower	14

2.3.4	RFID_SetInventoryScanTime	15

2.3.5	RFID_SetBaudRate	16

2.3.6	RFID_SetAntenna	17

2.3.7	RFID_CheckAnt	18

2.3.8	RFID_Beep	19

2.3.9	RFID_WriteRfPower	20

Appendix 1: Reader Error code table	21

# 1	Develop guide

## 1.1	Widget registration

Stet 1: copy library files "UHFREADER289.dll" and "UHF2000.ocx" from ".\SDK\OCX"(as indicated in Figure 1.1) to "C:\WINDOWS\system32" or "C:\WINDOWS\SysWOW64" respectively for 32-bit or 64-bit WINDOWS operation system.

Figure 1.1

Step 2: for 32-bit OS, user can directly double click the "Widget registration (win32).bat" to run this file. For 64-bit OS, user should right click the "Widget registration (win64).bat" file and select "Run as administrator". After the running the Windows Batch File, a window will popup to display the registration status, as shown in Figure 1.2.

Note: some computers may display registration error in the popup window even the registration is completed successfully.

Figure 1.2

## 1.2	Connect / disconnect reader

Reader must be firstly connected before any operation. Reader can be connected via serial connection (by using the "RFID_ComOpen" command) or TCP connection (by using the "RFID_TcpOpen" command).

Once all the operations are completed, user should call the "RFID_ComClose" command or the "RFID_TcpClose" command to shut down serial or TCP connection to reader, hence to release the occupied resources.

## 1.3	Inquired tag data format

Once reader is successfully connected, user can call the "RFID_Inventory" to inqury tag EPC or TID. Every inquired data block follows the format stated below:

actual tag data length (in byte) + EPC/TID No. + RSSI.

For instance, a serial of data blocks is stated as below:  OCE200123412341234444455558D0A1111222233334444555584

The first byte, 0x0c is the data length, i.e. the actual data length of the 1st tag is 12 bytes long. Hence the data inquired from the 1st tag is OCE200123412341234444455558D where the last byte 8D is the RSSI value. Likewise, Data obtain from the 2nd tag is 0A1111222233334444555584, where 0A is the actual data length (10 bytes), and the last byte 84 is the RSSI value.

Ignoring the data length and RSSI values, the actual EPC numbers of these two tags are:

1st tag EPC -- E20012341234123444445555;

2nd tag EPC -- 11112222333344445555.

# 2	Programming interface

## 2.1	Connect/disconnect reader

2.1.1	RFID_ComOpen

| Definition | long RFID_ComOpen(long Port, long Baud); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Open the serial connection to reader |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | Port | long | [in] | Serial port number for reader connection. 1 - COM1; 2 - COM2, etc. |
|  | Baud | long | [in] | Serial baud rate. 0 - 9600 bps; 1 - 19200 bps; 2 - 38400 bps; 5 - 57600 bps; 6 - 115200 bps. |
| Return value (LONG) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code |  |  |  |  |

2.1.2	RFID_OpenTcp

| Definition | long RFID_OpenTcp(LPCSTR ipAddr , long nPort); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Open the TCP connection to reader |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | nPort | LONG | [in] | TCP port number |
|  | ipAddr | LPCSTR | [in] | IP address |
| Return value (LONG) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code |  |  |  |  |

2.1.3	RFID_CloseCom

| Definition | long RFID_ComClose(); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Close a specific port to disconnect with reader and release associated resource. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
| Return value (LONG) | Succeed: 0; Failed: non zero; (for detail error definition, refer to error code table) . |  |  |  |
| Sample code |  |  |  |  |

2.1.4	RFID_CloseTcp

| Definition | long RFID_TcpClose(); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Close the TCP connection with reader and release associated resource |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
| Return value (LONG) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code |  |  |  |  |

## 2.2	18000-6C commands

2.2.1	RFID_Inventory

| Definition | LPCSTR RFID_Inventory(long QValue, long Session, long TidFlag, long Ant, long Target, long scantime); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Inquire RFID tags with corresponding protocol within the effective field. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | QValue | long | [in] | 1 byte, the original Q-value of EPC tag inventory. NOTE: 1. The setting of Q-value should follow the rule: 2Q ≈ total amount of tags within the effective field. The range of Q-value is 0 ~ 15, if other value is delivered in this field, reader will return a parameter error status in the response frame. 2. Inventory with statistic data packet is SLOWER than inventory without statistic data packet. |
|  | Session | long | [in] | 1 byte, the Session-value of EPC tag inventory. 0x00 – apply S0 as Session value; 0x01 – apply S1 as Session value; 0x02 – apply S2 as Session value; 0x03 – apply S3 as Session value. 0xff – apply reader smart configuration (only valid in EPC inventory), values of Ant, Target, scantime will be ignored for this configuration. Inventory for single tag or small amount of tag, S0 is the recommended setting. |
|  | TidFlag | long | [in] | 1 byte, inventory purpose indicator. 0 – EPC inventory; 1 – TID inventory. |
|  | Ant | long | [in] | 1 byte, antenna selection for the current inventory. 0x80 – antenna 1; 0x81 – antenna 2; 0x82 – antenna 3; 0x83 – antenna 4; 0x84 – antenna 5; 0x85 – antenna 6; 0x86 – antenna 7; 0x87 – antenna 8; |
|  | Target | long | [in] | 1 byte, the Target value of EPC tag inventory. 0x00 – apply Target A; 0x01 – apply Target B. |
|  | scantime | long | [in] | 1 byte, the maximum operation time for inventory. The valid range of Scantime is 0 ~ 255, corresponding to (0 ~ 255)*100ms. For Scantime = 0, operation time is not limited. |
| Return value (LPCSTR ) | Null: no tag is detected. Detected tag data will be returned as character string in Hexadecimal, refer to Chapter 1.3 for detail format. |  |  |  |

2.2.2	RFID_Read

| Definition | LPCSTR RFID_Read (LPCSTR EPC,long WordAddr, long ReadNum, long Mem); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Read a part of data or the whole memory area from tag reserved memory, EPC memory, TID memory, user memory. Read operation starts from a specific address. The unit of data is word. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | EPC | LPCSTR | [in] | Tag EPC number. |
|  | WordAddr | LONG | [in] | 1 byte, the initial word address of target. |
|  | ReadNum | LONG | [in] | The amount of words to be read in this operation. |
|  | Mem | LONG | [in] | Type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
| Return value (LPCSTR ) | Succeed: data will be returned as character string in Hexadecimal; Failed: return null. |  |  |  |
| Sample code | N/A |  |  |  |

2.2.3	RFID_Write

| Definition | long RFID_Write (LPCSTR EPC, long WordAddr, long Mem, LPCSTR Wdata); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Write multiple words to reserved memory, EPC memory, TID memory or user memory in one communication cycle. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | EPC | LPCSTR | [in] | Tag EPC number. |
|  | WordAddr | LONG | [in] | The initial word address of target. |
|  | Mem | LONG | [in] | The type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | Wdata | LPCSTR | [in] | Data to be written to tag, character string in Hexadecimal. |
| Return value (LONG) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

2.2.4	RFID_WriteEPC

| Definition | long RFID_WriteEPC(LPCSTR EPC); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Write EPC number to a tag. During the operation, only one tag is allowed to be place in the antenna effective area. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | EPC | LPCSTR | [in] | Tag EPC number, character string in Hexadecimal |
| Return value (LPCSTR) | Succeed: data will be returned as character string in Hexadecimal; Failed: return null. |  |  |  |
| Sample code | N/A |  |  |  |

## 2.3	Reader customised commands

2.3.1	RFID_GetSeriaNo

| Definition | LPCSTR RFID_GetSeriaNo () |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain 4 bytes reader unique serial number. |  |  |  |
| Parameters | Name | Type | Orientation | Not |
| Return value (LONG) | Succeed: serial number will be returned as 4 bytes character string in Hexadecimal; Failed: return null. |  |  |  |
| Sample code | N/A |  |  |  |

2.3.2	RFID_SetRegion.

| Definition | long RFID_SetRegion (long dmaxfre, long dminfre) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Select maximum and minimum frequency. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | dmaxfre | long | [in] | 1 byte. bit7 ~ bit6: frequency band configuration; bit5 ~ bit0: maximum frequency point. |
|  | dminfre | long | [in] | 1 byte. bit7 ~ bit6: frequency band configuration; bit5 ~ bit0: minimum frequency point. |
| Return value (LONG) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table) |  |  |  |
| Sample code | N/A |  |  |  |

Frequency band configuration table

| MaxFre (bit7) | MaxFre (bit6) | MinFre (bit7) | MinFre (bit6) | FreqBand |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 1 | 0 | Ukraine band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| 1 | 1 | 0 | 0 | US band3 |

Formulae for different frequency bands:

Chinese band2: Fs = 920.125 + N * 0.25 (MHz) where N∈ [0, 19]. US band: Fs = 902.75 + N * 0.5 (MHz) where N∈ [0, 49]. Korean band: Fs = 917.1 + N * 0.2 (MHz) where N∈ [0, 31]. EU band: Fs = 865.1 + N*0.2(MHz) where N∈ [0, 14]. Ukraine band: Fs = 868.0 + N*0.1(MHz) where N∈ [0, 6]. Chinese band1: Fs = 840.125 + N * 0.25 (MHz) where N∈ [0, 19]. US band3: Fs = 902 + N * 0.5 (MHz) where N∈ [0, 52].

2.3.3	RFID_SetRfPower

| Definition | long RFID_SetRfPower (long PowerDbm) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify reader RF power |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | PowerDbm | long | [in] | 1 byte, reader RF power. The range is 0 ~ 30 and the unit is dBm. |
| Return value (LONG) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

2.3.4	RFID_SetInventoryScanTime

| Definition | long RFID_SetInventoryScanTime (long ScanTime) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify maximum operation time for inventory command. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ScanTime | long | [in] | 1 byte, operation time for inventory. The acceptable range of Scantime is 0 ~ 255, corresponding to (0 ~ 255)*100ms. For Scantime = 0, operation time is not limited until a whole reading cycle is completed. |
| Return value (LPCSTR) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

2.3.5	RFID_SetBaudRate

| Definition | long RFID_SetBaudRate (long baud) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify serial baud rate. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | baud | long | [in] | 1 byte, serial baud rate. 0 – 9600bps; 1 – 19200bps; 2 – 38400bps; 5 – 57600bps; 6 – 115200bps. |
| Return value (LONG) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

2.3.6	RFID_SetAntenna

| Definition | long RFID_SetAntenna(long SetOnce, long AntCfg1, long AntCfg2) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify antenna configuration of 4 ports reader. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | SetOnce | long | [in] | Save information, 0-Save;1-Not save. |
|  | AntCfg1 | long | [in] | Resverve |
|  | AntCfg2 | long | [in] | 1 byte, antenna configuration information. 1 antenna is the MINIMUM requirement for operation. bit0: antenna 1 configuration bit. 0 – disable antenna 1; 1 – enable antenna 1. bit1: antenna 2 configuration bit. 0 – disable antenna 2; 1 – enable antenna 2. bit2: antenna 3 configuration bit. 0 – disable antenna 3; 1 – enable antenna 3. bit3: antenna 4 configuration bit. 0 – disable antenna 4; 1 – enable antenna 4. bit4: antenna 5 configuration bit. 0 – disable antenna 5; 1 – enable antenna 5. bit5: antenna 6 configuration bit. 0 – disable antenna 6; 1 – enable antenna 6. bit6: antenna 7 configuration bit. 0 – disable antenna 7; 1 – enable antenna 7. bit7: antenna 8 configuration bit. 0 – disable antenna 8. 1 – enable antenna 8. |
| Return value (LONG) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

2.3.7	RFID_CheckAnt

| Definition | long RFID_CheckAnt(long CheckAnt) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Enable antenna check during tag reading/writing operations. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | Antenna | long | [in] | 1 byte, antenna check switch. 0x00 – disable antenna check; 0x01 – enable antenna check. |
| Return value (LONG) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

2.3.8	RFID_Beep

| Definition | long RFID_Beep(long BeepEn) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Enable/disable buzzer. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | BeepEn | long | [in] | 1 byte. 0x00 – disable buzzer; 0x01 – enable buzzer. |
| Return value (LONG) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

2.3.9	RFID_WriteRfPower

| Definition | long RFID_WriteRfPower (long PowerDbm) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify RF power configuration separately for write operations |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | PowerDbm | long | [in] | 1 byte, separate reader RF power for write operation. The range is 0 ~ 30 and the unit is dBm. bit7 = 0: disable; bit7 = 1: enable. |
| Return value (LONG) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

# Appendix 1: Reader Error code table

| Error code | Description |
| --- | --- |
| 0x00 | API is called successfully. |
| 0x01 | Tag inventory is completed successfully and reader is able to deliver data response within the predefined inventory time. |
| 0x02 | Inventory timeout |
| 0x05 | Access password error. |
| 0x09 | Kill password error. |
| 0x0A | All-zero tag killing password is invalid. |
| 0x0B | Command is not support by the tag |
| 0x0C | All-zero tag access password is invalid for such command. |
| 0x0D | Fail to setup read protection for a protection enabled tag. |
| 0x0E | Fail to unlock a protection disabled tag. |
| 0x10 | Some bytes stored in the tag are locked. |
| 0x11 | Lock operation failed. |
| 0x12 | Already locked, lock operation failed. |
| 0x13 | Fail to store the value of some preserved parameters. Configuration will still valid before reader shut down. |
| 0x14 | Modification failed. |
| 0x15 | Response within the predefined inventory time |
| 0x17 | Further data is waiting to be delivered. |
| 0x18 | Reader memory is full |
| 0x19 | All-zero access password is invalid for such command or command is not supported by the tag. |
| 0xF8 | Error detected in antenna check. |
| 0xF9 | Operation failed. |
| 0xFA | Tag is detected, but fails to complete operation due to poor communication. |
| 0xFB | No tag is detected. |
| 0xFC | Error code returned from tags. |
| 0xFD | Command length error. |
| 0xFE | Illegal command. |
| 0xFF | Parameter error. |
| 0x30 | Communication error. |
| 0x33 | Reader is busy, operation in process. |
| 0x35 | Port is already opened. |
| 0x37 | Invalid handle. |